3GPP TSG-RAN3 Meeting #130	R3-258159

Dallas, USA, 17 – 21 Nov 2025

Agenda Item:	11.2.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for Intra-CU conditional LTM

Document for:	Discussion

# 1 Introduction

In RAN3#129bis, below agreements are made for CLTM. In this contribution, we discuss the solution details for the baseline MRO scenarios and define new MRO use cases for CLTM.

### 11.2.2. Intra-CU conditional LTM From RAN3#129bis: Use cases: At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA; FFS whether both L1 and L3 apply to beam related selection optimization. LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong L3 based intra-CU conditional LTM is within the scope of the WID Solution options: Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable. For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as baseline. For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline. For the connection failure cases, the last serving gNB-CU performs initial analysis. Additional information required from UE

# 2 Discussion

## 2.1 CLTM connection failure detection mechanism

Intra-CU CLTM and Intra-CU LTM mainly defer in the configuration of conditional execution criteria at the UE for the candidate LTM cells. Besides this, the connection failure reporting and failure detection occur within the same NG-RAN node and the same MRO information can be reused for Intra-CU CLTM connection failure detection.

Observation 1: Intra-CU CLTM is similar to Intra-CU LTM for connection failure reporting and detection mechanisms.

Proposal 1: Intra-CU LTM MRO for LTM Information IE shall be reused, sent from target DU to CU and from CU to DU via F1AP messages, to support connection failure detection scenarios of BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA.

CLTM failure can happen sometimes if the LTM execution conditions (either L3-based or L1-based or both) are not configured properly or the selected candidate cells are not optimal. It is beneficial to provide information to the RAN to detect the conditions that are triggered in both successful and failure cases.

Proposal 2: MRO for CLTM should support enhancements to optimize CLTM execution (both L3 and L1) conditions configured for initial CLTM and subsequent CLTM.

Proposal 3: MRO for CLTM should support enhancements to optimize the CLTM candidate cells selection for initial CLTM and subsequent CLTM.

Proposal 4: To support optimize the CLTM execution conditions (both L1 and L3), the RAN needs below information from UE either included in RLF Report or SHR:

- CLTM L3 condition(s) (e.g., CondEventA3 or A5) which were met before the RLF or during the successful CLTM

- CLTM L1 condition(s) (e.g., LTM3 or LTM5) which were met before the RLF or during the successful CLTM

- The first satisfied CLTM L3 condition (CondEventA3 or A5)

- The first satisfied CLTM L1 condition (LTM3 or LTM5)

- Time elapsed since the CLTM execution until connection failure

- ID (e.g., CGI or PCI + ARFCN) of the source cell, target cell, failed cell, re-established cell, CLTM recovery cell

## 2.2 CLTM candidate cell TA timer optimization

The gNB configures the ltm-Candidate-TimeAlignmentTimer (referred to as the CLTM TA Timer) via RRC signaling to the UE, which determines how long the UE considers the associated CLTM candidate cells to be uplink time-aligned. Upon receiving the Time Alignment (TA) value for these candidate cells through the LTM Candidate TA Command MAC CE, the UE stores the TA value and starts the CLTM TA Timer. When a CLTM cell switch is triggered for one of the candidate cells, the UE applies the stored TA value to the corresponding pTAG and starts or restarts the timeAlignmentTimer for that pTAG using the remaining duration of the CLTM TA Timer.

Since the CLTM TA Timer governs how long a UE considers candidate cells to be uplink time-aligned, its configuration directly impacts the success of CLTM. MRO enhancements for CLTM should consider optimizing CLTM TA timer to prevent RACH-less CLTM failures and improving overall cell switch performance in both RACH-based and RACH-less CLTM scenarios.

Observation 2: gNB configures ltm-Candidate-TimeAlignmentTimer (referred to as CLTM TA Timer for simplicity) via RRC to the UE, which controls how long the UE considers the CLTM candidate cells associated with this timer to be uplink time aligned

Observation 3: When a CLTM cell switch procedure is triggered for a CLTM candidate cell, the UE 1)applies the stored TA value associated with the CLTM candidate cell for the pTAG, 2) start or restart the timeAlignmentTimer associated with the pTAG with the length of the remaining time till expiry for the ltm-Candidate-TimeAlignmentTimer.

Proposal 5: MRO for CLTM should handle how to optimize CLTM TA timer to reduce RACH-less CLTM failure.

## 2.3 RACH-less CLTM failure scenarios

To support successful execution of RACH-less CLTM, MRO enhancements for detection of the below failure scenarios related to timer expiry should be considered to optimize the CLTM TA timer.

- If the CLTM TA timer expires before the CLTM conditions are met, the RACH-less CLTM cell switch is never initiated and results in missed opportunity of RACH-less CLTM execution.

- If the CLTM TA timer expires while RACH-less CLTM is ongoing (with T304 still running), the UE can “fallback” from RACH-less CLTM to RACH-based CLTM without waiting for T304 expiry. This results in  RACH-less CLTM failure.

- If T304 expires during RACH-less CLTM execution, UE declares RLF and performs RACH-based CLTM recovery (if configured) or performs re-establishment resulting in RACH-less CLTM failure.

- In RACH-less CLTM execution, a successful LTM cell switch occurring very close to the expiry of the CLTM TA timer can result in a minimal TA duration being applied to the pTAG which may result in misalignment of early UL sync.

Proposal 6: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expiry before CLTM conditions are satisfied (RACH-less CLTM was not initiated at all).

Proposal 7: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expires when RACH-less CLTM is ongoing (T304 still running).

Proposal 8: MRO for RACH-less CLTM failure should support detection of T304 expiry when RACH-less CLTM is ongoing (RACH-less CLTM execution failure).

Proposal 9: MRO for RACH-less CLTM failure should support detection of Successful RACH-less CLTM execution, but very close to “CLTM TA timer expiry”, thereby TA timer applied for pTAG is too small.

Additionally, there may be cases where the gNB fails to send a PDCCH order for early UL sync, possibly due to incorrect measurement configurations or implementation issues, even though the UE has reported measurements for the candidate cells. In such scenarios, the UE does not receive a TA value, and the CLTM TA timer is never started. Consequently, when CLTM execution conditions are met, the UE will perform RACH-based CLTM. Supporting detection of this scenario is necessary to minimize RACH-less CLTM execution failures.

Proposal 10: MRO for RACH-less CLTM failure should support detection of RACH-based CLTM execution because early UL sync was not performed towards the candidate cell.

# 3 Summary

Below are the observations and proposals discussed in this document.

CLTM connection failure detection mechanism

Observation 1: Intra-CU CLTM is similar to Intra-CU LTM for connection failure reporting and detection mechanisms.

Proposal 1: Intra-CU LTM MRO for LTM Information IE shall be reused, sent from target DU to CU and from CU to DU via F1AP messages, to support connection failure detection scenarios of BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA

Proposal 2: MRO for CLTM should support enhancements to optimize CLTM execution (both L3 and L1) conditions configured for initial and subsequent CLTM.

Proposal 3: MRO for CLTM should support enhancements to optimize the CLTM candidate cells selection for initial CLTM and subsequent CLTM.

Proposal 4: To support optimize the CLTM execution conditions (both L1 and L3), the RAN needs below information from UE either included in RLF Report or SHR:

- CLTM L3 condition(s) (e.g., CondEventA3 or A5) which were met before the RLF or during the successful CLTM

- CLTM L1 condition(s) (e.g., LTM3 or LTM5) which were met before the RLF or during the successful CLTM

- The first satisfied CLTM L3 condition (CondEventA3 or A5)

- The first satisfied CLTM L1 condition (LTM3 or LTM5)

- Time elapsed since the CLTM execution until connection failure

- ID (e.g., CGI or PCI + ARFCN) of the source cell, target cell, failed cell, re-established cell, CLTM recovery cell

CLTM TA timer scenarios

Observation 3: gNB configures ltm-Candidate-TimeAlignmentTimer (referred to as CLTM TA Timer for simplicity) via RRC to the UE, which controls how long the UE considers the CLTM candidate cells associated with this timer to be uplink time aligned

Observation 4: When a CLTM cell switch procedure is triggered for a CLTM candidate cell, the UE 1)applies the stored TA value associated with the CLTM candidate cell for the pTAG, 2) start or restart the timeAlignmentTimer associated with the pTAG with the length of the remaining time till expiry for the ltm-Candidate-TimeAlignmentTimer.

Proposal 5: MRO for CLTM should handle how to optimize CLTM TA timer to reduce RACH-less CLTM failure

Proposal 6: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expiry before CLTM conditions are satisfied (RACH-less CLTM was not initiated at all).

Proposal 7: MRO for RACH-less CLTM failure should support detection of CLTM TA timer expires when RACH-less CLTM is ongoing (T304 still running).

Proposal 8: MRO for RACH-less CLTM failure should support detection of T304 expiry when RACH-less CLTM is ongoing (RACH-less CLTM execution failure).

Proposal 9: MRO for RACH-less CLTM failure should support detection of Successful RACH-less CLTM execution, but very close to “CLTM TA timer expiry”, thereby TA timer applied for pTAG is too small.

Proposal 10: MRO for RACH-less CLTM failure should support detection of RACH-based CLTM execution because early UL sync was not performed towards the candidate cell.