3GPP TSG-RAN WG3 Meeting #130	R3-258178

Dallas, USA, November 17–21, 2025

Agenda Item:	10.4

Source:	Verizon

Title:	RAN Split Architecture Commercial Deployment Benefits

Document for:	Discussion & Agreement

# 1	Introduction

In the 6G SID [1], the following objectives were approved for 6G RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Additionally, the Chair’s notes from the RAN3#129-bis meeting [2] included the following guidance for the "RAN internal functional split" discussion at the upcoming RAN3#130 meeting:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

To be continued...

The gNB RAN architecture framework was standardized in 3GPP for 5G. At Verizon, we have many years of extensive experience deploying and maintaining both the monolithic or integrated (see Fig 1) and Higher Layer Split (HLS) or CU-DU split (see Fig 2) 5G gNB architectures in our network. We believe our observations in this area will be a valuable contribution to the 6G architecture study.

Fig. 1 Monolithic deployment

Fig. 2 HLS Deployment

While we anticipate diverse experiences and observations across different technical communities (operators, suppliers, etc.), we believe that an open exchange of information will benefit all stakeholders involved in shaping the future of 6G.

There are numerous benefits regarding HLS being discussed in RAN#129-bis: for example, operational agility, deployment flexibility and scalability, rapid deployment and maintenance, support of various deployment scenarios, high availability and resiliency, cost and energy efficiency with pooling gains, multivendor support, cloud friendliness etc.

# 2	HLS Learnings in Commercial Network

This section outlines the differences between monolithic and HLS architectures, specifically regarding their impact on cell site security and reliability.

Security

With monolithic architecture, the hardware is an integrated unit (fixed CU, DU and RU), making security a major concern if the unit is compromised. This necessitates extra measures, such as external enclosures and internal (software) safeguards for user data and control messages. While PDCP can provide ciphering and integrity protection, but the CU is located at the cell site location, so this protection is only available for the air interface. Data must be transported via a backhaul, typically provided by a third party, back to the Core. This backhaul link, even if protected by an SLA, remains a significant risk point.

In contrast, with the HLS architecture, the CU is centrally located typically in a much more secure location such as the switch center and co-located with the Core network. Any physical attempts (on the CU) are minimized due to constant surveillance. Hence, the ciphering and integrity protection benefits provided by the PDCP layer are better realized in this set up. In addition, the data on the standardized F1 interface from the CU at the switch center to the DU at the cell site location is encrypted and integrity protected using PDCP at the CU.

Observation 1: The HLS architecture enables stronger physical security for the centrally located CU and the NG interface between CU and the Core.

Observation 2:  The HLS also enables ciphering and integrity protection for the data between the CU at the switch center and the cell site location.

CU Reliability

With monolithic architecture, a single point of failure is common due to the lack of gNB redundancy. If a site becomes non-functional, and remote restoration attempts fail, the only remaining option is to dispatch technical personnel to the location to either restore or replace the unit. While temporary measures like automated features (e.g. SON) or manual antenna adjustments can minimize coverage holes, there will still be coverage disruption and reduction until the cell site unit is fully restored.

In contrast, the HLS architecture significantly reduces the single point of failure scenario. CUs can be deployed with geo and/or local redundancy (e.g., N + K configuration) to automatically switch over in case of failure. These restoration activities are performed remotely, which is typically faster and eliminates the need to dispatch technical personnel to the sites.

Observation 3: The HLS architecture enables the deployment of CU redundancy at a significantly reduced cost and allows for a faster restoration to maintain coverage and services.

Power Outage at the cell site location

Typically, there is a battery backup deployed at the cell site to account for power failure scenarios. In case of monolithic architecture, when a power outage occurs, the backup battery drains quicker because of the co-located CU processing power consumption (for ciphering and integrity protection).

In contrast, for the HLS deployment, the CU is located in a centralized switch center and does not drain the backup battery power at the cell site location. Hence, the battery at the cell site could last longer in case of a power outage.

Observation 4: The HLS architecture is better for saving back up battery power at the cell site in case of power outage scenario.

# 3	Conclusion

In the previous section, we observed that the 5G HLS architecture is advantageous in terms of security and reliability compared to the monolithic architecture. Since no single solution can cover all deployment scenarios, it is crucial to maintain an open approach to improve existing operating architectures. This openness also facilitates the introduction of new architectural enhancements, which can further advance 6G deployments.

Observation 1: The HLS architecture enables stronger physical security for the centrally located CU and the NG interface between CU and the Core.

Observation 2:  The HLS also enables ciphering and integrity protection for the data between the CU at the switch center and the cell site location.

Observation 3: The HLS architecture enables the deployment of CU redundancy at a significantly reduced cost and allows for a faster restoration to maintain coverage and services.

Observation 4: The HLS architecture is better for saving back up battery power at the cell site in case of power failure scenario.

The goals of 6G RAN architecture should include the following:

- Enable operators to cost-effectively migrate existing 5G infrastructure (monolithic and HLS) to 6G.

- Facilitate the introduction of 6G without negatively impacting the performance or stability of the current 5G network.

With these goals in mind, we made the following proposals:

Proposal 1: Consider both monolithic and Higher Layer Split (HLS) architectures as part of 6G RAN architecture.

Proposal 2: Explore new capabilities for 6G RAN Architectures without disrupting existing 5G services and performance.

# 4	References

[1] RP-252912: New SID: Study on 6G Radio (RAN#109)

[2] Chair Notes RAN3#129-bis