3GPP TSG-RAN WG3 Meeting #130	R3-258188

Dallas, US, 17 - 21 November 2025

Agenda item	: 10.4

Source	: Tejas Networks

Title	: RAN internal functional split and interfaces

Document for	: Discussion & Decision

# Introduction

In this contribution, we provide Tejas Network’s view point of RAN internal functional split and interfaces.

# RAN Logical Architecture

Figure 1 [1] depicts various options considered while studying RAN architecture for New radio (NR).

Figure 1: Various architecture split options considered for NR

For RAN logical architecture for 6GR, we propose that Option 6 which splits between MAC and PHY layers be considered. However, our stand for co-existence of Option 2 (split between PDCP and RLC) with Option 6 is neutral. In what follows, we discuss Option 6 while assuming Option 2 is retained. Thus, PDCP will be hosted in the Central Unit (CU), RLC and MAC layers will be hosted in Distributed Unit (DU) and PHY and RF together will be hosted in a new network function (NF) ``DY-PHY”. In terms of interfaces, we need to introduce a new network interface ``G1” (for want of name) between DU and DU-PHY. An advantage with Option 6 is that PHY layer design and implementation can evolve independent of the hardware platforms considered for MAC and RLC. In addition to the interface between MAC and PHY (scheduling, resource allocation, DCI, etc.) one should also consider messages that help RRC configuration towards the PHY layer.

In case of NTN, Option 6 allows CU and DU to be hosted in an earth station while DU-PHY will be moved into the satellite platform. The user data that gets exchanged over the interface corresponding to Option 6 will then be MAC PDUs which do not place heavy bit rate demand on the transport network implementing the interface.

Figure 2: Option 6 for RAN logical architecture for 6GR.

Proposal 1: The overall RAN architecture should follow Option 6, which introduces split between MAC and PHY and helps TN and NTN integration.

# RAN Internal Functional Split

RAN Functional split between various NFs CU, DU, and DU-PHY can be discussed after architecture option is frozen.

# Conclusion

Proposal 1: The overall RAN architecture should follow Option 6, which introduces split between MAC and PHY and helps TN and NTN integration.

# References

[1] 3GPP TR 38.801, `` Study on New Radio Access Technology; Radio Access Architecture and Interfaces”