3GPP TSG-RAN3 Meeting #130	R3-258194

Dallas, US, Nov 17 - 21, 2025

Agenda Item:	12.2.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Multi-hop UE Trajectory Prediction and Feedback

Document for:	Discussion/Decision

# Background

In the last RAN3#129bis meeting some progress was made and the following agreements, open issues and common understanding was captured.

RAN3#129bis Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order. The following solutions are considered for supporting multi-hop UE trajectory: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. Focus on cell-based granularity for measured and predicted multi-hop UE trajectory. Study multi-hop UE trajectory for L3 HO. The delivery of the measured & predicted cell-based multi-hop UE trajectory should not impact the core network. The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop. Open Issues Transfer the multi-hop UE trajectory prediction Configuration of measurement feedback How to transfer measurement feedback: • Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1) • Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1) • Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1) • Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1). Common understanding: - The cell-based multiple-hop UE trajectory can be updated to remove the past visited cell(s). - When UE moves to unpredicted target cell, the serving gNB can discard the cell-based multiple-hop UE trajectory prediction

In this paper we discuss our views on the open issues on multi-hop UE Trajectory prediction and feedback.

# Discussion

## Multi-hop UE Trajectory Transfer

Multi-hop UE Trajectory Prediction

For single hop, the UE Trajectory prediction is transferred via Handover Request. UE Trajectory Prediction is specific to an UE and cannot be transferred via non-UE associated message. Also UE trajectory prediction is for connected mode mobility. Hence other Handover Request message, there is no other suitable message to carry UE Trajectory prediction.

UE Trajectory Prediction is specific to an UE and cannot be transferred via non-UE associated message. UE trajectory prediction is for connected mode mobility of UE.

- In UE associated messages, Handover Request is the suitable message to transfer UE Trajectory prediction from one gNB to another. Handover Request shall be used to transfer UE Trajectory Prediction over Xn interface.

Multi-hop UE Trajectory Feedback

Data collection procedure is for collecting AI/ML feedback for different use cases in 5G AI/ML framework. Feedback for UE trajectory prediction also should follow the same and use the 5G AI/ML framework Data collection procedure for feedback collection.

- Data Collection procedure is used for feedback collection for all AI/ML use cases in 5G. UE Trajectory prediction feedback shall also use Data Collection procedure to request and receive measured UE Trajectory.

In the last meeting there were also concerns on whether RAN nodes in the neighbourhood over the UE Trajectory Prediction from the previous nodes. This can be based on implementation and need not be mentioned in standards. This concern is not specific to UE Trajectory prediction. It is applicable for any prediction by a RAN node. Any RAN node in the path of the UE Trajectory overwrites the prediction from the previous node, should inform the previous RAN node by terminating the data collection mechanism.

Current gNB replacing the UE Trajectory prediction from previous gNB can be implementation specific and need not be mentioned in standards. Also, this behaviour of gNB can be for any prediction and not specific for UE Trajectory Prediction.

- Any RAN node in the path of the UE Trajectory overwrites the prediction from the source RAN node, should inform the source RAN node by terminating the data collection mechanism.

## Direct UE Trajectory Feedback Collection

The source node which predicts the UE trajectory knows the cells and gNBs where the UE can possibly connect to in near future. Hence the node which predicts the UE trajectory and wants the feedback on the actual UE Trajectory can send Data collection Request to all the possible target nodes where the UE is predicted to connect to. The source node can also send Data Collection Request to all RAN nodes to which it is Xn connected to get feedback on UE Trajectory. UE stays in RRC Connected state for a short period of time and it is unlikely that UE moves beyond the Xn connected neighbouring gNBs. This is also very similar to RRC Inactive Context retrieval from the anchor node.

In the Handover Request along with the Data Collection ID, the UE context reference at the source node (UE ID at the source node) which collects the UE trajectory feedback from other nodes needs to be passed for UE identification at the Feedback collection node. Reference call flow is shown below.

The source node which predicts the UE trajectory knows the cells and gNBs where the UE is predicted to connect. Source gNB is also Xn connected to its neighbouring gNBs

UE stays in RRC Connected state for a short time. Hence it is unlikely that UE moves beyond the Xn connected neighbouring gNBs.

- The source gNB which collects the UE Trajectory Feedback configures all the predicted target gNBs or Xn Connected neighbouring gNBs with Data Collection Request for measured UE Trajectory data collection. The feedback collection is limited to only neighbouring gNBs which are directly connected to the source gNB via Xn interface. This is similar to RRC Inactive UE Context Retrieval over Xn.

- The source gNB (i.e which collects the UE Trajectory feedback) provides the Data collection ID at the source and the UE ID at the source in the Handover Request message to the target. There are only 2 new IEs which are needed in Handover Request for direct UE Trajectory feedback collection.

- When the UE goes to un-predicted target or to a target which is not Xn connected to the source or if the UE goes to RRC_Idle/inactive the feedback collection is terminated in the target, and the collected feedback is reported to the source.

## Comparison between hop-by-hop and direct Feedback

Hop by Hop Feedback

In hop by hop feedback collection every node in the UE Trajectory path configures the subsequent target node for UE Trajectory feedback. The feedback for UE Trajectory prediction is passed from current node to the previous node and the cycle continues until the feedback reaches the UE Trajectory prediction originating source.

Disadvantages:

- All the nodes in the UE Trajectory need to maintain UE context to forward the feedback to the UE Trajectory Prediction originating source node, which implies millions of UE context needs to be maintained until feedback collection period.

- It is unclear on how the Measured UE trajectory collection duration and number of visited cells is percolated from the UE Trajectory Prediction originating source node to the subsequent nodes.

- It is also unclear on how long each node in the UE Trajectory path needs to maintain the UE context.

Advantages:

- If the UE does not follow the predicted path, then the feedback collection can be triggered from the un-predicted node easily as it is hop by hop.

Big disadvantage in hop-by-hop feedback is all nodes in the UE trajectory path need to maintain the UE context for millions of UEs in the RAN node which is a huge overhead.

Direct Feedback

In direct feedback the UE Trajectory feedback is passed from the target node to the UE Trajectory prediction originating source node. The gNBs in the UE Trajectory path need not maintain the UE Context. The UE Trajectory prediction originating source node configures all the predicted targets with the necessary details to send the feedback directly to the source node.

Advantages:

- UE context needs to be retained only in the UE Trajectory Prediction originating source node and the subsequent nodes need not maintain the UE context which is a big advantage.

- Addition of the Data collection ID along with the Predicted node ID and source node UE ID in the Handover Request are the only stage 3 changes needed.

Disadvantages:

- If the UE goes to a gNB which is not Xn connected to the source gNB, then the feedback collection cannot continue dynamically and the feedback collection stops.

UE context need not be stored in all the nodes in the UE trajectory path and only the UE Trajectory Prediction originating source node needs to maintain the UE context.

- Direct feedback collection provides a huge advantage over hop-by-hop feedback collection by not maintaining many UE context(s) in all gNBs in the UE Trajectory path with minimal specification impact.

- RAN3 to agree to pursue direct feedback collection for multi-hop (across gNBs) UE Trajectory prediction and feedback for Rel-20.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	UE Trajectory Prediction is specific to an UE and cannot be transferred via non-UE associated message. UE trajectory prediction is for connected mode mobility of UE.

Observation 2.	Current gNB replacing the UE Trajectory prediction from previous gNB can be implementation specific and need not be mentioned in standards. Also, this behaviour of gNB can be for any prediction and not specific for UE Trajectory Prediction.

Observation 3.	The source node which predicts the UE trajectory knows the cells and gNBs where the UE is predicted to connect. Source gNB is also Xn connected to its neighbouring gNBs

Observation 4.	UE stays in RRC Connected state for a short time. Hence it is unlikely that UE moves beyond the Xn connected neighbouring gNBs.

Observation 5.	Big disadvantage in hop-by-hop feedback is all nodes in the UE trajectory path need to maintain the UE context for millions of UEs in the RAN node which is a huge overhead.

Observation 6.	UE context need not be stored in all the nodes in the UE trajectory path and only the UE Trajectory Prediction originating source node needs to maintain the UE context.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	In UE associated messages, Handover Request is the suitable message to transfer UE Trajectory prediction from one gNB to another. Handover Request shall be used to transfer UE Trajectory Prediction over Xn interface.

Proposal 2:	Data Collection procedure is used for feedback collection for all AI/ML use cases in 5G. UE Trajectory prediction feedback shall also use Data Collection procedure to request and receive measured UE Trajectory.

Proposal 3:	Any RAN node in the path of the UE Trajectory overwrites the prediction from the source RAN node, should inform the source RAN node by terminating the data collection mechanism.

Proposal 4:	The source gNB which collects the UE Trajectory Feedback configures all the predicted target gNBs or Xn Connected neighbouring gNBs with Data Collection Request for measured UE Trajectory data collection. The feedback collection is limited to only neighbouring gNBs which are directly connected to the source gNB via Xn interface. This is similar to RRC Inactive UE Context Retrieval over Xn.

Proposal 5:	The source gNB (i.e which collects the UE Trajectory feedback) provides the Data collection ID at the source and the UE ID at the source in the Handover Request message to the target. There are only 2 new IEs which are needed in Handover Request for direct UE Trajectory feedback collection.

Proposal 6:	When the UE goes to un-predicted target or to a target which is not Xn connected to the source or if the UE goes to RRC_Idle/inactive the feedback collection is terminated in the target, and the collected feedback is reported to the source.

Proposal 7:	Direct feedback collection provides a huge advantage over hop-by-hop feedback collection by not maintaining many UE context(s) in all gNBs in the UE Trajectory path with minimal specification impact.

Proposal 8:	RAN3 to agree to pursue direct feedback collection for multi-hop (across gNBs) UE Trajectory prediction and feedback for Rel-20.