3GPP TSG-RAN WG3  Meeting #130	R3-258235

Dallas 17th -21st 2025

Source: 	Vodafone, Telecom Italia, Deutsche Telekom

Title: 		NG-C Requirements (Legacy Services)

Document for: decision

Agenda Item: 10.3.1

Introduction

RAN Plenary tasks RAN3 to make a study on RAN-CN interface: P2P vs SBI.

During the last RAN3#129bis meeting, some progress was reached on basic requirements for Ng-Interface.

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

This paper aims to provide additional basic requirements focusing on 6G NG-C-Interface for legacy services. As a starting point, the current protocol stack should be taken as reference:

Proposal/Requirement 1: It is proposed to take the current SCTP based protocol stack as a reference.

It is proposed to discuss and agree on the following requirements:

Setup of new 6GNR

The setup of the new base station is an important step for operator and therefore, it should be evaluated how the process of the base station setup can be simplified and automated, and which features would be needed to be standardized to realize it.

Proposal/Requirement 2: The setup of NG-Interface should be faster and with a need for less manual intervention.

Proposal/Requirement 3: NG-Interface shall provide means to ease of rolling out updates and managing software versions.

Performance & Efficiency

Latency: How quickly NG-AP messages are delivered between termination points. It is proposed to study the delay introduced by a particular protocol option and describe the reasons for this delay.

Connection Setup Time: It is proposed to analyze the influence of the Latency of every proposed option for idle to connected state transition

Proposal/ Requirement 4: NG-Interface shall be designed in a way that does not introduce additional delays in signaling and user plane transport and decrease the delay for the initial connection setup

Overhead: Some of the protocols might introduce considerable additional protocol overheads, hand check procedures, etc. This overhead might require additional bandwidth, processing capabilities in the NB and CN and contribute to delay, which shall be avoided.

Proposal/ Requirement 5: NG-Interface shall be designed in a way that does not introduce additional overhead once exchanging NG-AP messages (e.g. consider additional protocol headers overhead and procedures (e.g. Hand Check procedures)

Scalability:

Multiplexing Efficiency: Efficient multiplexing of streams and avoidance of head-of-line blocking is an important metric for an efficient and reliable design allowing to increase transmission efficiency.

Proposal/ Requirement 6: NG-Interface shall support more efficient multiplexing of streams and provide means to avoid head-of-line blocking

Session Management: How efficient the protocol stack handles establishment, maintenance, and termination of signaling sessions

Load Balancing: Advantages of each option related to Load Balancing should be studied to find the most efficient option from load balancing point of view

Resource Usage: All possible options should be analyzed in terms of CPU and memory footprint.

Proposal/ Requirement 7: NG-Interface shall support means for more scalable design, allowing more efficient session management, load balancing and resource utilization

Integration with Existing Systems: How easily it fits into current 5G core setups. The introduction of a totally new CN is costly and delays the introduction of new RAT and therefore, this is especially important as many operators are just about to invest in the 5G SA and any crucial re-design will question this investment.

Proposal/ Requirement 8: NG-Interface shall be designed with minimum impact to the current RAN and 5G core to minimize costs on operator side once introducing a new RAT, while solving real and known problems.

# Annex

## pCR to TR 38.760

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-     the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

-    the 6G RAN-CN Interface for control plane shall take legacy SCTP based design as reference point

-    the setup of 6G RAN-CN Interface for control plane should be faster and with a need for less manual interventions

-   the 6G RAN-CN Interface for control plane shall provide means to ease of rolling out updates and managing software versions

-   the 6G RAN-CN Interface for control plane shall be designed in a way that does not introduce additional delay once exchanging NG-AP messages and decrease call setup time

- NG-Interface shall be designed in a way that does not introduce additional overhead once exchanging NG-AP messages

-  the 6G RAN-CN Interface for control plane shall support more efficient multiplexing of streams compared to the legacy design and provide means to avoid head-of-line blocking problems

- the 6G RAN-CN Interface for control plane shall support means for more scalable design, allowing more efficient session management, load balancing and resource utilization

- the 6G RAN-CN Interface for control plane shall shall be designed with minimum impact to the current RAN and 5G core to minimize costs on operator side once introducing a new RAT, while solving real and known problems.