3GPP TSG-RAN WG3 Meeting #130						R3-258243

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		13.2

Title:		(TP to BL pCR of TR38.765) The discussion on ISAC network architecture

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the ISAC SID [1], the following objects should be studied in RAN3 that:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

And the following progress is captured in the last meeting:

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 Continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

This contribution will continuously discuss the network architecture regarding ISAC.

# 2. Discussion

## 2.1 Protocol stack for gNB and Sensing Function

Progress summary from other WG

In the last meeting, RAN3 agreed on the logical architecture for ISAC: the connection between the gNB and SF via the Nx interface can be either direct or indirect. The selection of a direct or indirect connection depends on SA2’s progress, but SA2 has not yet concluded this issue.

The ISAC measurement were discussed in RAN1 [2], which decides the volume of sensing measurement reporting, and we summarized here:

| Level | Definition | Data size |
| --- | --- | --- |
| Level 1 | - Level 1: raw data, which is the subcarrier-domain samples of the channel response from each antenna port, or anything equivalent | Msubcarrier*Ncpi*Mport*Nport*Nquantization e.g., 3276*(80*4)*(4*8*2)*32 = 2.1Gbits |
| Level 2 | - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle | Mcp*Ncpi*Mport*Nport*Nquantization, e.g., 288*(80*4)*(4*8*2)*32 = 188.7Mbits |
| Level 3 | - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects | - |
| Level 4 | - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity | Npoint*5* Nquantization, e.g., 100*5*32 = 16Kbits |
| Level 5 | - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported | - |
| Level 6 | - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported | (Npoint=1)*5* Nquantization 15*5*32 = 2400bits |

The six different levels of ISAC measurement were discussed in RAN1, and the data sizes of measurement results vary significantly between different levels. However, the selection of these six levels remained highly controversial in the last RAN1 meeting.

Observation 1: Six different levels of ISAC measurement were discussed in RAN1, and the data sizes of their measurement results vary significantly between different levels.

Protocol stack for the sensing reporting

Regarding the sensing reporting protocol, there are four solutions mentioned in the last meeting that:

- Option 1: NG-C based protocol stack.

- Option 2: Additional protocol layer over NG-AP.

- Option 3: New interface.

- Option 4: GTP-U based protocol stack.

For Option 1, the NG-AP is reused and enhanced to carry the sensing measurement reporting, which is the same solution as the Ambient IoT. For Option 2, the NG-AP is reused, and an additional protocol layer for sensing reporting is defined over the NG-AP, and the positioning has the similar solution (NRPPa layer). For Option 3 (the new interface solution) the NG-AP is replaced by a new interface to support sensing measurement reporting. For Option 4, the GTP-U is reused and enhanced to carry the sensing measurement reporting.

From RAN1 progress, we know that the data sizes of measurement results differ greatly between different levels. This directly impacts the sensing reporting protocol stack in RAN3. Below is an analysis of the protocol stack for different levels of sensing measurement reporting:

| Level | NG-C based protocol stack | Additional protocol layer over NG-AP | New interface | GTP-U based protocol stack |
| --- | --- | --- | --- | --- |
| Level 1: 2.1Gbits | Not applicable The AIoT-like solution is the enhancement on the NG-C, but the NG-C interface cannot handle such a massive volume of measurement data. | Not applicable The new ISAC protocol layer carries over NG-C, same issue as the NG-C. | It depends on the new protocol design. If it is designed based on the SCTP/IP, it may not appliable because the SCTP/IP do not have such big throughput. If it is designed based on the GTP-U, it may be applicable for such a data size. | Applicable |
| Level 2: 188.7Mbits |  |  |  |  |
| Level 4: 16Kbits | Applicable The data volume may be acceptable for the NG-C. | Applicable The data volume may be acceptable for the NG-C. | Applicable The data volume may be acceptable for the new protocol stack over the SCTP/IP. | Applicable |
| Level 6: 2400bits |  |  |  |  |

In addition, an indirect connection would not be feasible if Levels 1 and 2 are selected by RAN1, as the AMF would also be unable to transmit such large volumes of sensing data.

Therefore, we suggest that RAN3 can discuss the protocol stack for sensing control information transmission, and decide on the sensing measurement reporting protocol stack after RAN1 reaches a stable conclusion on sensing measurements, and relevant measurement quantization evaluation.

Observation 2: The protocol stack for sensing reporting is directly impacted by sensing measurements, and relevant measurement quantization evaluation, which is dependent on RAN1.

Proposal 1: RAN3 can discuss the protocol stack for sensing control information transmission. And decide the sensing measurement reporting protocol stack after RAN1 has a stable conclusion on the sensing measurements, and relevant measurement quantization evaluation.

Protocol stack for sensing control signaling

There is the protocol stack for the control signaling transmission that:

- Option 1: NG-AP based protocol stack.

- Option 2: Additional protocol layer over NG-AP.

- Option 3: New interface.

Under the first option, the NG-AP is reused and enhanced to carry sensing control signaling—a solution consistent with the one used for Ambient IoT. The Ambient IoT architecture, frozen in Rel-19, is highly similar as ISAC architecture that: both involve the gNB connecting to the relevant CN node either directly or indirectly via the AMF. Therefore, based on the protocol stack of AIoT, we can derive a comparable architecture for ISAC:

| Direct connection | Indirect connection |
| --- | --- |

For Option 2, the NG-AP is reused, and an additional protocol layer is defined over it, which is the solution adopted in positioning. By referencing the NRPPa protocol stack, the sensing protocol stack should be structured as:

| Direct connection | Indirect connection |
| --- | --- |

For Option 3, the NG-AP is replaced by a new interface to support sensing control signaling transmission. And it is only considered in direct connection cases.

Direct connection

Note that the protocol stacks outlined above are specifically for sensing control signaling transmission.

Proposal 2: The following protocol stack options should be discussed in RAN3:

- Option 1: NG-AP based protocol stack.

- Option 2: Additional protocol layer over NG-AP.

- Option 3: New interface.

Proposal 3: RAN3 is suggested discussing and capturing the aforementioned protocol stacks for sensing control signaling transmission, and agree on the corresponding TPs in the Annex.

# 3. Conclusion

Observation 1: Six different levels of ISAC measurement were discussed in RAN1, and the data sizes of their measurement results vary significantly between different levels.

Observation 2: The protocol stack for sensing reporting is directly impacted by sensing measurements, and relevant measurement quantizationevaluation, which is dependent on RAN1.

Proposal 1: RAN3 can discuss the protocol stack for sensing control information transmission. And decide the sensing measurement reporting protocol stack after RAN1 has a stable conclusion on the sensing measurements, and relevant measurement quantization evaluation.

Proposal 2: The following protocol stack options should be discussed in RAN3:

- Option 1: NG-AP based protocol stack.

- Option 2: Additional protocol layer over NG-AP.

- Option 3: New interface.

Proposal 3: RAN3 is suggested discussing and capturing the aforementioned protocol stacks for sensing control signaling transmission, and agree on the corresponding TPs in the Annex.

# 4. References

[1] RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR

[2] R1-2507427, Summary #5 on evaluations for NR ISAC

# 5.	Annex (TP for TR 38.765)

Start of Change

## 7.x Protocol stack

There are three protocol stacks for sensing control signalling transmission:

- NG-AP based protocol stack, the NG-AP is reused to transfer the sensing control signaling.

- Additional protocol layer over NG-AP, the existing NG-AP is reused, and a new layer is defined above it for sensing control signaling transmission.

- New interface, a new interface replaces the NG-AP to support sensing control signalling transmission.

Editor’s Note: The protocol stack for sensing data reporting is to be further studied (FFS).

### 7.x.1 NG-AP based protocol stack

Figure 7.x.1-1 illustrates the direct connection protocol stack for sensing control signalling via the NG-AP:

Figure: 7.x.1-1: Direct connection protocol stack for sensing control signalling via NG-AP

The sensing control information is transmitted explicitly over NG-AP.

Figure 7.x.1-2 illustrates the indirect connection protocol stack for sensing control signalling via NGAP:

Figure: 7.x.1-2: Indirect connection protocol stack for sensing control signalling via NGAP

The sensing control information is transmitted between gNB and SF and transparent to the AMF.

### 7.x.2 2.	Additional protocol layer over NGAP

Figure 7.x.2-1 illustrates the direct connection protocol stack for additional sensing layer over NG-AP for control signalling transmission:

Figure: 7.x.1-1: Direct connection protocol stack for sensing control signalling via new layer over NG-AP

The sensing control information is transmitted explicitly over the Sensing layer and transparently over NG-AP.

Figure 7.x.2-2 illustrates the indirect connection protocol stack for additional sensing layer over NG-AP for control signalling transmission:

Figure: 7.x.1-2: Indirect connection protocol stack for sensing control signalling via new layer over NG-AP

The sensing control information is transmitted explicitly over the Sensing layer and transparently over AMF and NG-AP.

### 7.x.3 3.	New interface

Figure 7.x.3-1 shows the protocol stack for sensing control signalling via new interface:

Figure: 7.x.3-1: Direct connection protocol stack for sensing control signalling via new interface

he sensing control information is transmitted explicitly over the sensing interface

End Change