3GPP TSG-RAN WG3 Meeting #130	R3-258245

Dallas, USA, 17 – 21 Nov. 2025

Agenda item:		14.2

Title:		Discussion on Topology 2 of Ambient IoT

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The RAN3 objectives for A-IoT Topology 2 enhancement mentioned in WID [1] that

RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

And the following progress was made in the last meeting that

General Architecture: For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied. NGAP procedures specified for Topology 1 are baseline for Topology 2. NGAP signaling procedures (Inventory and Command): The AIOT procedures (Topology 2, RRC-based solution) in TR38.769 can be as baseline. UE Reader Authorization: UE Reader Authorization is performed at 5GC The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB. FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

This contribution will continue to discuss the related issues.

# 2. Discussion

## 2.1 NGAP signaling procedures

General Architecture: For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied. NGAP procedures specified for Topology 1 are baseline for Topology 2. NGAP signaling procedures (Inventory and Command): The AIOT procedures (Topology 2, RRC-based solution) in TR38.769 can be as baseline.

According to the Topology 2 RRC-based solution procedures in TR 38.769, the following TP should be included in the Stage 2 description in TS 38.300:

### 16.23.6 Inventory Procedure #### 16.23.6.1 Topology 1 Figure 16.23.6-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure for Topology 1. Figure 16.23.6-1: Inventory procedure for topology 1 <<<<<<<<<<<< Omit unrelated part >>>>>>>>>>>> #### 16.23.6.2 Topology 2 Figure 16.23.6-2 depicts the basic communication between the UE, gNB and the A-IoT CN node for the Inventory procedure for Topology 2. Figure 16.23.6-2: Inventory procedure for topology 2 1. The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB, and then forwards it to the authorized UE Reader. 2. The gNB allocates A-IoT radio resources for the UE Reader to perform A-IoT operation. Editor Note: FFS for the details, depending on RAN2. 3/4. The RRC Inventory Request/Response between UE and gNB. Editor Note: Message name and details depending on RAN2. 5. Upon receiving the RRC Inventory Response from the UE, the gNB sends an Inventory Response message to the A-IoT CN. 6. The Inventory procedure over the A-IoT radio interface is performed. Editor Note: Details depending on RAN1/2. 7/9. Upon receiving the response(s) from the A-IoT device(s), the UE sends the RRC Inventory Report to the gNB. Editor Note: The content of RRC Inventory Report depending on RAN2. 8/10. After receiving the inventory result reported by the UE, the gNB sends an Inventory Report message towards the A-IoT CN. Editor Note: The details of the Inventory Request/Response and Inventory Report message over NG-C is FFS.

### 16.23.7 Command Procedure ### 16.23.7.1 Topology 1 Figure 16.23.7-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure for Topology 1. Figure 16.23.7-1: Command procedure for Topology 1 <<<<<<<<<<<< Omit unrelated part >>>>>>>>>>>> ### 16.23.7.1 Topology 2 Figure 16.23.7-2 depicts the basic communication between the UE, gNB and the A-IoT CN node for the Command procedure for Topology 2. Figure 16.23.7-2: Command procedure for Topology 2 The Command procedure addresses only one A-IoT device. 1. Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices. 2. The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device via the UE Reader. 3. The gNB allocates A-IoT radio resources to the UE for A-IoT operation. 4. The gNB sends RRC Command Request message to the UE. 5. The UE performs the Command procedure over the A-IoT radio interface. 6. The UE replies the RRC Command Response message to the gNB when it receives the A-IoT NAS PDU from A-IoT device. Editor Note: Details of Steps 4/5/6 depend on RAN1/2. 7. After receiving RRC Command Response from the UE, the gNB sends Command Response message towards the A-IoT CN. Editor Note: FFS the details of Command Request/Response message over NGAP.

Proposal 1: RAN3 is suggested to discuss and agree the TP above.

2.2 UE Reader authorization

Progress about the UE Reader authorization in the last RAN3 meeting:

UE Reader Authorization is performed at 5GC The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB. FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

FFS from the source/old gNB to the target/new gNB

Whether the source gNB sends the UE Reader authorization status was discussed and marked as FFS in the last meeting.

First of all, for the inter-gNB handover case, the target gNB should know the UE Reader authorization status since the target gNB should prepare the AIoT resources for the UE Reader according to the UE Reader authorization status. From the last meeting discussion, the target gNB can know the UE Reader authorization status in the following methods:

- Option 1: The source gNB transmits the UE Reader authorization status to the target gNB.

- Option 2: The AMF can notify the target gNB about the UE Reader authorization status.

For Option 1, the UE Reader authorization status (authorized, non-authorized) should be stored as part of the UE context in the source gNB, and it will be transmitted to the target gNB in the HANDOVER REQUEST message during the handover.

For Option 2, when the target gNB sends the PATH SWITCH REQUEST message to the AMF, and the AMF recognizes that the UE Reader has handover to the target gNB and sends the UE reader authorization status to the target gNB via the PATH SWITCH REQUEST ACKNOWLEDGE message.

From technical perspective, both options are workable for the network, but we prefer Option 1 since it is a simpler and straightforward solution.

Proposal 2: The source gNB transmits the UE Reader authorization status, as part of the UE context, to the target gNB in the HANDOVER REQUEST message during the handover.

FFS from the gNB-CU to the gNB-DU

Regarding whether the UE Reader authorization status is transmitted from gNB-CU to gNB-DU, we do not see that it is necessary. If the gNB-CU knows the UE Reader authorization status is authorized, the CU will decide to allocate the resources for the UE Reader and send the resource allocation decision to the gNB-DU. If the UE Reader authorization status is not-authorized, no specific actions are required for gNB-CU and gNB-DU regarding the UE reader. So, the gNB-DU does not need to be aware of the UE Reader authorization status, and there is no benefit to transferring the UE Reader authorization status from the gNB-CU to the gNB-DU.

Proposal 3: There is no benefit to transferring the UE Reader authorization status from the gNB-CU to the gNB-DU.

Overall, at least the following messages should include the UE Reader authorization status:

- NG-AP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST.

- Xn-AP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE.

Proposal 4: At least the following messages should include the UE Reader authorization status:

- NG-AP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST.

- Xn-AP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE.

2.3 UE reader selection

Reader selection

SA2 has some conclusions about the UE reader selection in TR 23.700-30 that:

AIOTF Discovery and Selection for UE reader ID: Editor's note: How to discover and select AIOTF when AF provides UE reader ID is FFS. UE reader selection part: Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader. - For AF providing UE reader ID case: - If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN. Editor's note: How to select UE reader(s) based on the AF provided area information is FFS. UE reader ID allocation: Editor's note: Whether and how to allocate the UE reader ID is FFS.

Similar to Topology 1, the AIoT CN may send the UE Reader ID(s) or intended service area to the AIoT-enabled gNB to make the UE reader selection.

There are the following cases for the UE reader selection:

- Case 1: UE Reader ID(s) is provided by the AIoT CN.

- Case 2: Intended service area (AIoT Area ID and/or Cell ID and/or gNB ID) is provided by the AIoT CN.

- Case 3: None of the above information is provided by the AIoT CN.

For Case 1, as the principle captured in TR 23.700-30, if the AIoT CN provides UE reader ID(s), the AIoT-enabled RAN shall use the indicated UE(s) as the UE reader for the AIoT service.

For Case 2, the AIoT RAN may select authorized UE(s) within the intended service area to act as UE readers. However, whether the RAN should select all authorized UEs or a subset of them based on implementation requires further discussion.

For Case 3, RAN3 can follow the principle agreed in Topology 1 that the AIoT-enabled gNB to selects all the authorized UEs in the intended service area.

Proposal 5: RAN3 should discuss the following cases for UE reader selection:

- Case 1: UE Reader ID(s) is provided by the AIoT CN.

- Case 2: Intended service area (AIoT Area ID and/or Cell ID and/or gNB ID) is provided by the AIoT CN.

- Case 3: None of the above information is provided by the AIoT CN.

Proposal 6: If the AIoT CN sends UE reader ID(s) to the AIoT-enabled gNB, the gNB should consider the indicated UE(s) as the UE readers to provide the AIoT service.

Proposal 7: If the intended service area is provided, the AIoT RAN may select authorized UE(s) within that area as UE reader(s) to perform the AIoT operation. (Note: Whether the RAN should select all authorized UEs or a subset based requires further discussion.)

Proposal 8: If neither the intended service area nor UE Reader ID(s) are provided, the A-IoT RAN may select all the authorized UEs it serves.

UE reader capability Reporting

In Rel-19, we have an agreement that “AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list”, so that the AIoTF can select the Reader(s) in Topology 1. Similarly, in Topology 2, in order to enable the AIoT CN to select the UE reader, the AIoT CN should be aware of the AIoT UE information. However, the OAM configuration cannot inform the AIoTF about the UE reader AIoT UE information. Therefore, the UE should report the AIoT UE information to the AIoT CN when the UE accesses the network, so that the AIoT CN can obtain the AIoT UE information, and then perform UE Reader authorization and UE Reader selection. And the AIoT UE information may include the AIoT capability and the UE Reader location.

Proposal 9: The UE should report the AIoT related information (AIoT capability and the UE Reader location) to the AIoT CN via signaling when the UE accesses the network, so that the AIoTF can be aware of the AIoT information and then perform UE Reader authorization and UE Reader selection.

Paging the UE Reader that not in RRC_CONNECTED state

For Topology 2 Reader selection, in Case 1 where the AIoT CN indicates the UE Reader list in the Inventory Request message, the precondition is that the UE Reader is in the RRC_CONNECTED state. This allows the AIoT CN to know the serving gNB of the UE Reader, so the gNB can send the Inventory Request message to the UE Reader.

However, the UE Reader cannot always remain in the RRC_CONNECTED state before the Inventory service is configured. If the UE Reader is in the idle state, the AIoT CN cannot know its serving gNB. Therefore, if the AIoT CN intends to select a UE Reader that is not in the RRC_CONNECTED state, the CN needs to page the UE Reader to the RRC_CONNECTED state (with UE Reader authorization also performed in this step), then send the Inventory Request message to the serving gNB, which is then forwarded to the UE Reader.

Observation 1: Before configuring the AIoT service for a UE Reader, the UE Reader cannot always remain in the RRC_CONNECTED state. As a result, the AIoT CN cannot know the serving gNB of the UE Reader, and the gNB cannot send the Inventory Request message to the UE Reader.

Proposal 10: If the AIoT CN intends to select a UE Reader that is not in the RRC_CONNECTED state, the CN needs to page the UE Reader to the RRC_CONNECTED state, then send the Inventory Request message to the serving gNB, which is then forwarded to the UE Reader.

## 2.4 The definition of UE reader ID

In Topology 1, as a gNB serves one or multiple Readers and the gNB ID cannot uniquely identify a RAN Reader, the RAN Reader ID is defined as “Global gNB ID + Reader Index”. The index is assigned by the AIoT RAN, and the AIoTF becomes aware of the RAN Reader ID via OAM.

For Topology 2, RAN3 needs to discuss whether the UE Reader ID should follow the same format as Topology 1 (“Global gNB ID + Reader Index”, where the Global gNB ID is that of the UE's serving gNB) or simply reuse the legacy UE ID. The legacy UE ID refers to the RAN UE NGAP ID over the NG interface and the C-RNTI over the Uu interface.

From our perspective, reusing the legacy UE ID as the UE Reader ID is sufficient for Topology 2, because the legacy UE ID can uniquely identify the UE Reader. Moreover, considering the mobility of UE Readers, this approach avoids the need to update the UE Reader ID during inter-gNB handovers, which would be necessary if a “Global gNB ID + Reader Index” format is used.

Proposal 11: The legacy UE ID (i.e., the RAN UE NGAP ID over the NG interface and the C-RNTI on the Uu interface) can uniquely identify a UE Reader.

## 2.5 UE Reader mobility

The UE Reader mobility is another aspect that needs to be discussed in Topology 2 as mentioned in WID [1] that:

RAN2 - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. RAN3 - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

From the WID [1], we can deduce that the UE reader should be able to perform the configured AIoT operation after the UE reader handover to another cell or gNB.

However, the A-IoT services should be highly location-dependent. For instance, a service might be configured to a specific factory area or storage room. In such cases, if a UE Reader handover out of the AIoT service area, the UE Reader is expected to stop the AIoT operation.

Proposal 12: If a UE Reader handover out of the intended service area, the UE Reader is expected to stop the AIoT operation.

If the target gNB is in the intended service area, the UE reader should be able to continue the AIoT operation after handover to the target gNB successfully.

Proposal 13: If a UE Reader handover to a target gNB that is in the intended service area, the UE Reader is expected to continue the AIoT operation after handover to the target gNB successfully.

The AIoT RAN will allocate resources for UE readers to perform AIoT operation, and, as indicated in the WID, the AIoT resource is only valid within the serving cell. So, when the UE handover to another cell or gNB, it will trigger the new serving cell or gNB to allocate the AIoT radio resource for the UE reader. Since the inter-gNB resource coordination is not specified, some assistance information for the AIoT resource allocation should be discussed in case of inter-gNB UE handover.

Proposal 14: Some assistance information for the AIoT resource allocation over Xn-AP should be discussed in case of inter-gNB handover.

# 3. Conclusion

2.1 NGAP signaling procedures

Proposal 1: RAN3 is suggested to discuss and agree the TP above.

2.2 UE Reader authorization

Proposal 2: The source gNB transmits the UE Reader authorization status, as part of the UE context, to the target gNB in the HANDOVER REQUEST message during the handover.

Proposal 3: There is no benefit to transferring the UE Reader authorization status from the gNB-CU to the gNB-DU.

Proposal 4: At least the following messages should include the UE Reader authorization status:

- NG-AP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST.

- Xn-AP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE.

2.3 UE reader selection

Proposal 5: RAN3 should discuss the following cases for UE reader selection:

- Case 1: UE Reader ID(s) is provided by the AIoT CN.

- Case 2: Intended service area (AIoT Area ID and/or Cell ID and/or gNB ID) is provided by the AIoT CN.

- Case 3: None of the above information is provided by the AIoT CN.

Proposal 6: If the AIoT CN sends UE reader ID(s) to the AIoT-enabled gNB, the gNB should consider the indicated UE(s) as the UE readers to provide the AIoT service.

Proposal 7: If the intended service area is provided, the AIoT RAN may select authorized UE(s) within that area as UE reader(s) to perform the AIoT operation. (Note: Whether the RAN should select all authorized UEs or a subset based requires further discussion.)

Proposal 8: If neither the intended service area nor UE Reader ID(s) are provided, the A-IoT RAN may select all the authorized UEs it serves.

Proposal 9: The UE should report the AIoT related information (AIoT capability and the UE Reader location) to the AIoT CN via signaling when the UE accesses the network, so that the AIoTF can be aware of the AIoT information and then perform UE Reader authorization and UE Reader selection.

Observation 1: Before configuring the AIoT service for a UE Reader, the UE Reader cannot always remain in the RRC_CONNECTED state. As a result, the AIoT CN cannot know the serving gNB of the UE Reader, and the gNB cannot send the Inventory Request message to the UE Reader.

Proposal 10: If the AIoT CN intends to select a UE Reader that is not in the RRC_CONNECTED state, the CN needs to page the UE Reader to the RRC_CONNECTED state, then send the Inventory Request message to the serving gNB, which is then forwarded to the UE Reader.

2.4 The definition of UE reader ID

Proposal 11: The legacy UE ID (i.e., the RAN UE NGAP ID over the NG interface and the C-RNTI on the Uu interface) can uniquely identify a UE Reader.

2.5 UE Reader mobility

Proposal 12: If a UE Reader handover out of the intended service area, the UE Reader is expected to stop the AIoT operation.

Proposal 13: If a UE Reader handover to a target gNB that is in the intended service area, the UE Reader is expected to continue the AIoT operation after handover to the target gNB successfully.

Proposal 14: Some assistance information for the AIoT resource allocation over Xn-AP should be discussed in case of inter-gNB handover.

# References

[1] RP-252894, Solutions for Ambient IoT (Internet of Things) in NR