3GPP TSG-RAN WG3 Meeting #130	R3-258246

Dallas, USA, 17 – 21 November 2025

Source:				NEC

Agenda item:	11.2.2 Intra-CU conditional LTM

Title:	MRO for Intra-CU conditional LTM

Document for:				Discussion and decision

- Introduction

During last RAN3 meeting, initial discussion on MRO for intra-CU CLTM was made, and the following progress were made:

1. Use cases At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA; FFS whether both L1 and L3 apply to beam related selection optimization. LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong L3 based intra-CU conditional LTM is witin the scope of the WID 2. Solution options Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable. For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line. For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line. For the connection failure cases, the last serving gNB-CU performs initial analysis.

In this paper, we would like to provide our initial thinking on the MRO enhancement for intra-CU conditional LTM.

- Discussion

In CHO, the UE stores and reports to the network information of CHO candidate cells and the execution condition configuration of the candidate cells. However, in Rel-19 MRO for intra-CU LTM, whether the UE needs to log candidate cell list was discussed, and it was agreed that network-based solution is used for LTM candidate cell list, because the cell list is going to be stored in the UE context in the network side for a long time due to the support of subsequent LTM, and therefore there is no need for the UE to store such information. Similarly, for CLTM, there is no need for the UE to log the cell list and execution condition associated with the LTM candidate cell.

Proposal 1. The UE does not log the execution condition associated with the LTM candidate cell, i.e., network-based solution is used.

For Rel-19 MRO for intra-CU LTM, for failures due to inappropriate cell switch triggering, CU initiates analysis and forwards the failure information to the DU responsible for the failure. For failure of CLTM, although it the UE triggered to LTM, but the execution condition is set by the source DU. Therefore, we think the same F1 signalling and procedure can be used for intra-CU LTM, such that the source DU can adjust the execution condition based on the failure information from the CU.

Proposal 2. The F1 signalling and procedure for intra-CU LTM failure (i.e., too early LTM, too late LTM and LTM to wrong cell) for can be reused for intra-CU CLTM.

In Rel-19 MRO for intra-CU LTM, the target DU can identify the BFR shortly after a successful LTM cell switch, and send the recovery beam information to the source DU via the CU. For intra-CU CLTM, BFR shortly after a successful LTM cell switch can also happen, the difference is that the beam selected during the CLTM is determined by the UE based on the fulfilled execution condition set by the source DU. Since for CLTM, the source DU has no information of the beam selected by the UE during the RACH-less CLTM, it is better to provide also the beam selected during the RACH-less CLTM to the source DU.

Proposal 3. For BFR shortly after successful intra-CU CLTM, the target DU also provides the information of selected beam during the RACH-less CLTM to the source DU via the CU.

In Rel-19 MRO for intra-CU LTM, the target DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell, and send the re-established or recovery beam information and/or the TA value used at successful RACH-based access to the source DU via the CU. However, for CLTM, since there is no DU-CU CELL SWITCH NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION message, the target DU is not aware of the initiation of the CLTM cell switch procedure, and therefore has no information of the beam selected by the UE, the TA or TAT information when the LTM cell switch is triggered, and even has no idea of whether the CLTM cell switch procedure is a normal one or the one triggered by LTM recovery. It would be easier for the UE to collect the information to optimize the concerned scenario.

Proposal 4. For the MRO of CLTM failure due to wrong beam and outdated TA, UE-based solution should be used.

In Rel-19 MRO of intra-CU LTM, the ping-pong issue of LTM cell switch was discussed and solved by sending from CU to DU the UE history information with a ping-pong indicator using CELL SWITCH NOTIFICATION message. For intra-CU CLTM, the ping-pong cell switch issue also exists. However, there is no CU-DU CELL SWITCH NOTIFICATION message transmitted from the CU to the candidate DU, instead UE CONTEXT MODIFICATION REQUEST message can be transmitted from CU to the candidate DU during/after completion of the CLTM procedure. Therefore, it is proposed to use this message to transfer Last Visited LTM Cells list from the CU to the DU.

Proposal 5. For ping-pong issue of CLTM, the CU sends Last Visited LTM Cells list to the DU by UE CONTEXT MODIFCATION REQUEST message.

For RACH-less CLTM, early UL synchronization can be triggered by the source DU before the initiation of the CLTM cell switch, and different from non-conditional LTM, the UE performs TA maintenance of the candidate cell by running the TAT associated with the candidate cell. However, the timing of the early UL synchronization may be “too early”. For example, when the CLTM cell switch execution is initiated, the TAT associated with the candidate cell is already expired, or the remaining time of the TAT associated with the candidate cell is below a threshold, which results a high possibility of fallback from RACH-less CLTM to RACH-based CLTM. Or the early UL synchronization may be “too late”. For example, when the CLTM cell switch execution is initiated, the early UL synchronization is still not triggered by the source DU. To make the early UL synchronization procedure more efficient for the CLTM procedure, we propose to support MRO for “too early” and “too late” early UL synchronization for the CLTM.

Proposal 6. Support MRO for “too early” and “too late” early UL synchronization for CLTM.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1. The UE does not log the execution condition associated with the LTM candidate cell, i.e., network-based solution is used.

Proposal 2. The F1 signalling and procedure for intra-CU LTM failure (i.e., too early LTM, too late LTM and LTM to wrong cell) for can be reused for intra-CU CLTM.

Proposal 3. For BFR shortly after successful intra-CU CLTM, the target DU also provides the information of selected beam during the RACH-less CLTM to the source DU via the CU.

Proposal 4. For the MRO of CLTM failure due to wrong beam and outdated TA, UE-based solution should be used.

Proposal 5. For ping-pong issue of CLTM, the CU sends Last Visited LTM Cells list to the DU by UE CONTEXT MODIFCATION REQUEST message.

Proposal 6. Support MRO for “too early” and “too late” early UL synchronization for CLTM.