3GPP TSG-RAN WG3 Meeting #130	R3-258250

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		10.3.2

Source:		NEC

Title:			(pCR to TR38.760-3) 6G RAN-CN interface protocol consideration

Document for:		Discussion and decision

# Introduction

In the study scope of the new SI on 6G Radio [1], the following items have RAN3 involved:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide our thoughts on the RAN-CN interface options, P2P vs SBI.

# Discussion

Service-Based Interface (SBI) is introduced as a fundamental architectural component of the 5GC, which replaces traditional dedicated point-to-point (P2P) interfaces and serves as a logical service bus for Network Functions (NFs) using standardized APIs. This enables 5GC transfer to a cloud-native, microservices-based design which makes the network more flexible, scalable, and efficient. Figure 1 below depicts the 5G System Architecture with service-based interfaces in the Control Plane.

Figure 1: 5G System Architecture with SBI

Regarding the RAN-CN interface design for 6G, there are three options:

- Option 1: Traditional P2P interface (NG-like)

- Option 2: Fully SBI (API based)

- Option 3: Partial P2P interface and partial SBI (between new NFs)

In 6G, besides the services already supported in 5G, new services, including AI, sensing, immersive communication, NTN, are emergent, which will become new revenue streams for operators. As outlined in the 6G TR38.914, “The 6G RAN architecture shall support enhanced service awareness in RAN.” For RAN domain, in order to facilitate the quick deployment of these new services, SBI adopted in RAN will be a better choice, by which one RAN NF can directly consume other RAN NFs or CN NFs.

For example, for AI service, Nran interface which is an API based open interface can be used to support the exchange of signalling and data among RAN NFs or between RAN NFs and CN NFs. Nran interface can consist of multiple services, e.g. Nran_AI/ML/Data reporting or Nran_AI/ML/Model transfer, where a RAN NF acts as producer and other RAN/CN NFs as consumers. This will largely simplify the procedure and ease data accessibility and reusability.

However, considering the high standard effort for a groundbreaking design and the requirement of smooth interoperability with 5G, we suggest to adopt a two-step strategy: first to support Option 3, which can provide 6G new services in a more flexible and scalable way, and meanwhile keep traditional communication functions unchanged as legacy; and then to move toward Option 2 in a later phase, which realized a true cloud implementation of RAN.

Proposal 1: Study partial P2P interface (for traditional communication functions) and partial SBI (for new NFs) scheme for 6G RAN-CN interface.

TP to TR38.760-3 which captures the above analysis is in Annex.

Proposal 2: Agree the TP to TR38.760-3 for RAN-CN interface options in Annex.

# Proposals

In this contribution, we provide our views on the 6G RAN-CN interface options and made the following observations and proposals.

Proposal 1: Study partial P2P interface (for traditional communication functions) and partial SBI (for new NFs) scheme for 6G RAN-CN interface.

Proposal 2: Agree the TP to TR38.760-3 for RAN-CN interface options in Annex.

# Reference

[1] RP-252912, New SI: Study on 6G Radio

# Annex	TP for TR38.760-3

<Start of change>

### 6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

Service-Based Interface (SBI) is introduced as a fundamental architectural component of the 5GC, which replaces traditional dedicated point-to-point (P2P) interfaces and serves as a logical service bus for Network Functions (NFs) using standardized APIs. This enables 5GC transfer to a cloud-native, microservices-based design which makes the network more flexible, scalable, and efficient.

Regarding the RAN-CN interface for 6G, there are three options:

- Option 1: Traditional P2P interface (NG-like)

- Option 2: Fully SBI (API based)

- Option 3: Partial P2P interface and partial SBI (between new NFs)

In 6G, new services, including AI, sensing, immersive communication, NTN, are emergent, which will become new revenue streams for operators. In order to facilitate the quick deployment of these new services, SBI adopted in RAN will be a better choice, by which one RAN NF can directly consume other RAN NFs or CN NFs. This can largely simplify the procedure and ease data accessibility and reusability. However, a groundbreaking design on RAN-CN interface needs high standard effort and may cause issue on smooth interoperability with 5G.

<End of change>