3GPP TSG-RAN WG3 Meeting #130	R3-258256

Dallas, USA, 17th – 21st November 2025

Source:				NEC

Agenda item: 	11.2.1 Inter-CU LTM

Title: 	MRO for inter-CU LTM

Document for:				Discussion and decision

- Introduction

For Rel-20 Data collection for SON, the solution or the following objective is to be discussed and decided:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

In last meeting, the following agreements has been made:

1. Use cases At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM, i.e.: BFR shortly after successful LTM cell switch, failure due to wrong beam and outdated TA; LTM connection failure: too late and too early LTM cell switch, LTM cell switch to wrong cell; Near failure case: SHR successful HO Successful case: ping/pong Potential new cases will not be precluded. 2. Solution options Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable. For inter-CU LTM, the definition of connection failure cases in 38.300 are taken as base line. For inter-CU LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line. For the connection failure cases: - After receiving a failure indication, the last serving gNB/gNB-CU performs initial analysis. L3 based inter-CU LTM is within the scope of the WID

In this paper, we would like to provide our thinking on the MRO enhancement for inter-CU LTM.

- Discussion

In handover procedure, when the connection failure happens, the node to which the UE recovers or re-establishes connection transmits the failure information including RLF report to the gNB last serving the UE, and the last serving gNB should perform the root cause analysis based on the information received as TS 38.300. For inter-CU LTM, one difference is that the initiating gNB which decides to configure LTM for the UE including preparing the candidate cell list may be not the source gNB during a subsequent LTM cell switch. Therefore, considering this case, the forwarding of  LTM failure information to the initiating gNB of the LTM cell switch procedure should be supported.

Proposal 1. 	Considering subsequent inter-CU LTM cell switch, the forwarding of LTM failure information (e.g., RLF report) should be supported.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify the BFR shortly after a successful LTM cell switch, and send the recovery beam information to the source gNB-DU via the gNB-CU. For inter-CU LTM, the same case exists, and the same mechanism can be used.

Proposal 2. 	Support MRO enhancement over Xn interface for BFR happened in the UE shortly after a successful inter-CU LTM cell switch.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell, and send the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU. For inter-CU LTM, the same case exists, and the same mechanism can be used.

Proposal 3. 	Support MRO enhancement over Xn interface for the case that UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after inter-CU LTM cell switch failure.

In Rel-19 MRO of intra-CU LTM, the ping-pong issue of LTM cell switch was discussed and solved by sending from CU to DU the UE history information with a ping-pong indicator using CELL SWITCH NOTIFICATION message. For Rel-20 MRO of inter-CU LTM, the ping-pong issue of subsequent inter-CU LTM should also be discussed, because the UHI in handover request message from the source gNB to the candidate gNB is useless for the ping-pong happened during subsequent inter-CU LTM, solution should be considered and specified. RAN3 should discuss on using which message to sends the UHI information or ping-pong information, and when to send it between the gNBs.

Proposal 4. 	Support MRO enhancement over Xn interface for optimization of ping-pong issue during inter-CU LTM cell switch.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1. 	Considering subsequent inter-CU LTM cell switch, the forwarding of LTM failure information (e.g., RLF report) should be supported.

Proposal 2. 	Support MRO enhancement over Xn interface for BFR happened in the UE shortly after a successful inter-CU LTM cell switch.

Proposal 3. 	Support MRO enhancement over Xn interface for the case that UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after inter-CU LTM cell switch failure.

Proposal 4. 	Support MRO enhancement over Xn interface for optimization of ping-pong issue during inter-CU LTM cell switch.