3GPP TSG-RAN WG3 Meeting #130	R3-258288

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	10.4

Source: 	OPPO

Title:  	Discussion on 6G RAN internal interfaces

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Besides, an interim milestone is set for RAN3 as shown below.

4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026.

Based on the guidance of Chairman in RAN3#129bis meeting, this contribution would like to provide our considerations on RAN internal interfaces, mainly covering pain points and benefits (and potentially derived requirements) of 5G HLS.

- Discussion

During the early stage of 5G research, different potential options for CU-DU split were studied and discussed in RAN3, as illustrated in the figure below.

In 5G, non-ideal transport conditions between CU and DU need to be considered. The main trade-off for CU-DU split is between centralization gains and hardware/transport costs. Considering the deployment flexibility, higher layer split option is easier to deployed.

Besides, the support of Dual-Connectivity architectures is of course an important factor on the selection of CU-DU split option, which require a centralized PDCP layer. Therefore, the options which does not align with DC architectures is definitely excluded, such as Option 1. And Option 2 is the only choice that aligns with all different DC architectures.

Considering the deployment flexibility and the alignment of DC, higher layer split option (i.e., Option 2 for PDCP/RLC split) is adopted as 5G HLS solution.

- Option 2 for PDCP/RLC split is adopted as 5G HLS solution in 5G RAN, considering 
1/ the trade-off between centralization gains and hardware/transport costs, and 
2/ the alignment with Dual-Connectivity architecture.

As captured in TR 38.801, the table below shows the characteristics of different CU-DU split options, where Option 2 is highlighted.

|  | Opt. 1 | Opt. 2 | Opt. 3-2 | Opt. 3-1 | Opt. 5 | Opt. 6 | Opt. 7-3 (only for DL) | Opt. 7-2 | Opt. 7-1 |  | Opt. 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline available | No | Yes (LTE DC) | No |  |  |  |  |  |  |  | Yes (CPRI) |  |
| Traffic aggregation | No | Yes |  |  |  |  |  |  |  |  |  |  |
| ARQ location | DU |  |  | CU May be more robust under non-ideal transport conditions |  |  |  |  |  |  |  |  |
| Resource pooling in CU | Lowest | in between (higher on the right) |  |  |  |  |  |  |  | Highest |  |  |
|  | RRC only | RRC + L2 (partial) |  |  |  | RRC + L2 | RRC + L2 + PHY (partial) |  |  | RRC + L2 + PHY |  |  |
| Transport NW latency requirement | Loose |  |  |  | NOTE 7 | Tight |  |  |  |  |  |  |
| Transport NW Peak BW requirement | N/A | Lowest | in between (higher on the right) |  |  |  |  |  |  | Highest |  |  |
|  | No UP req. | baseband bits |  |  |  |  |  | Quantized IQ (f) |  | Quant. IQ (t) |  |  |
|  | - | Scales with MIMO layers |  |  |  |  |  |  | Scales with antenna ports |  |  |  |
| Multi-cell/freq. coordination | multiple schedulers (independent per DU) |  |  |  | centralized scheduler (can be common per CU) |  |  |  |  |  |  |  |
| UL Adv. Rx | NOTE 7 |  |  |  |  |  | NA | NOTE 7 | Yes |  |  |  |
| Remarks | NOTE 4 |  |  |  | NOTE 5/6 | NOTE 5 | NOTE 5 | NOTE 5 |  |  |  |  |

- In TR 38.801 for 5G study, the characteristics of 5G HLS, i.e., Option 2 for PDCP/RLC split, include that 
1/ Baseline available: Yes (LTE DC),
2/ Traffic aggregation: Yes,
3/ ARQ location: DU,
4/ Resource pooling in CU: In between, RRC + L2 (partial),
5/ Transport NW latency requirement: Loose,
6/ Transport NW Peak BW requirement: Lowest, baseband bits, Scales with MIMO layers, and
7/ Multi-cell/freq. coordination: multiple schedulers (independent per DU).

The pros and cons of each option have also been comprehensively analyzed in RAN3. As captured in TR 38.801, the benefits, justification and cons of Option 2 (PDCP/RLC split) are shown as below.

#### 11.1.2.2 Option 2 (PDCP/RLC split) Option 2 may be a base on an X2-like design due to similarity on U-plane but some functionality may be different e.g. C-plane since some new procedures may be needed. Option 2-1 Split U-plane only (3C like split) Description: In this split option, RRC, PDCP are in the central unit. RLC, MAC, physical layer and RF are in the distributed unit. Benefits and Justification: - This option will allow traffic aggregation from NR and E-UTRA transmission points to be centralized. Additionally, it can facilitate the management of traffic load between NR and E-UTRA transmission points. - Fundamentals for achieving a PDCP-RLC split have already been standardized for LTE Dual Connectivity, alternative 3C. Therefore this split option should be the most straightforward option to standardize and the incremental effort required to standardize it should be relatively small. NOTE 1: U-plane aspect was justified in the study phase. C-plane aspect was not addressed in the study phase. - The alignment between LTE-NR tight interworking and functional split may be beneficial at least in user-plane, considering migration. NOTE 2: An enhancement for the fast-centralized retransmission of lost RLC PDUs in this option was identified, but the solution details were not discussed in this study. Option 2-2: In this split option, RRC, PDCP are in the central unit. RLC, MAC, physical layer and RF are in the distributed unit. In addition, this option can be achieved by separating the RRC and PDCP for the CP stack and the PDCP for the UP stack into different central entities. Benefits and Justification: - This option will allow traffic aggregation from NR and E-UTRA transmission points to be centralized. Additionally, it can facilitate the management of traffic load between NR and E-UTRA transmission points. - This option enables centralization of the PDCP layer, which may be predominantly affected by UP process and may scale with UP traffic load. - This option allows a separate U-plane while having a centralised RRC/RRM. Cons: - Coordination of security configurations between different PDCP instances for Option 2-2 needs to be ensured.

- In TR 38.801 for 5G study, for 5G HLS, i.e., Option 2 (PDCP/RLC split), the benefits and justification include
1/ allowing traffic aggregation / traffic load management between NR and E-UTRA,
2/ LTE DC alternative 3C as baseline,
3/ alignment with LTE-NR DC, which is beneficial at least in user-plane, considering migration,
4/ enabling centralization of PDCP layer,
5/ allowing separate U-plane while having a centralised RRC/RRM.

- In TR 38.801 for 5G study, for 5G HLS, i.e., Option 2 (PDCP/RLC split), the cons include coordination of security configurations between different PDCP instances.

Furthermore, we observe that, in 5G, although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

- For 5G HLS, although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

From user plane data transmission perspective, according to the current 5G user plane protocols stacks, the tight cross-layer coordination is still needed between the PDCP layer and the RLC layer.

For example, in 5G Rel-18/19 XR, in order to make the XR traffic transmission properly, as illustrated above, the PDCP layer needs to provide different indications (i.e. SDU discard indication, remaining-time-based RLC retransmission indication and remaining-time-based RLC polling indication) to RLC entity so as to allow the differentiated RLC discard/retransmission/polling behaviours (i.e. SDU discard, remaining-time-based RLC retransmission, RLC-only retransmission, remaining-time-based RLC polling and RLC-only polling).

In case of 5G CU-DU split architecture, PDCP layer is in CU and RLC layer is in DU, thus the discard indication from the CU PDCP to the DU RLC causes extra latency and signalling overheads over F1 interface.

- From user plane data transmission perspective, tight PDCP-RLC interaction between CU and DU is still needed in case of 5G HLS, which may cause extra latency and signalling overheads over F1 interface.

From control plane perspective, taking MAC CE and RRC signalling as an example, when discussing solutions between MAC CE and RRC signalling, we usually consider the latency of MAC CE is much lower than RRC signalling, since non-ideal transport conditions between CU and DU leads to additional latency for RRC layer in CU. Therefore, Layer-1/2 mobility, i.e., LTM, is introduced in 5G on top of Layer-3 mobility. In LTM, DU can directly decide to execute LTM based on L1 measurement reporting of UE, without interaction between DU and CU.

- From control plane perspective, 5G HLS leads to additional latency for RRC layer in CU.

In the last, a standardized 5G HLS solution is for inter-vender CU and DU. In 5G, we spend much standardization effort in RAN3, while there are quite limited deployments for 5G HLS. We understand only in case that there is real need on inter-vender CU and DU, a standardized CU-DU split solution is necessary. Otherwise, it can leave to NW implementation, and much standard effort would be avoided.

- For 5G HLS, RAN3 spent much standardization effort in RAN3, while there are quite limited deployments.

Therefore, regarding pain points and benefits (and potentially derived requirements) of 5G HLS, we propose to take all observations above into account.

- Regarding pain points and benefits (and potentially derived requirements) of 5G HLS, RAN3 take all observations above into account.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Option 2 for PDCP/RLC split is adopted as 5G HLS solution in 5G RAN, considering 
1/ the trade-off between centralization gains and hardware/transport costs, and 
2/ the alignment with Dual-Connectivity architecture.

- In TR 38.801 for 5G study, the characteristics of 5G HLS, i.e., Option 2 for PDCP/RLC split, include that 
1/ Baseline available: Yes (LTE DC),
2/ Traffic aggregation: Yes,
3/ ARQ location: DU,
4/ Resource pooling in CU: In between, RRC + L2 (partial),
5/ Transport NW latency requirement: Loose,
6/ Transport NW Peak BW requirement: Lowest, baseband bits, Scales with MIMO layers, and
7/ Multi-cell/freq. coordination: multiple schedulers (independent per DU).

- In TR 38.801 for 5G study, for 5G HLS, i.e., Option 2 (PDCP/RLC split), the benefits and justification include
1/ allowing traffic aggregation / traffic load management between NR and E-UTRA,
2/ LTE DC alternative 3C as baseline,
3/ alignment with LTE-NR DC, which is beneficial at least in user-plane, considering migration,
4/ enabling centralization of PDCP layer,
5/ allowing separate U-plane while having a centralised RRC/RRM.

- In TR 38.801 for 5G study, for 5G HLS, i.e., Option 2 (PDCP/RLC split), the cons include coordination of security configurations between different PDCP instances.

- For 5G HLS, although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

- From user plane data transmission perspective, tight PDCP-RLC interaction between CU and DU is still needed in case of 5G HLS, which may cause extra latency and signalling overheads over F1 interface.

- From control plane perspective, 5G HLS leads to additional latency for RRC layer in CU.

- For 5G HLS, RAN3 spent much standardization effort in RAN3, while there are quite limited deployments.

- Regarding pain points and benefits (and potentially derived requirements) of 5G HLS, RAN3 take all observations above into account.

4. Reference

[1] RP-252912, New SID: Study on 6G Radio, NTT DOCOMO