3GPP TSG-RAN WG3 Meeting #130	R3-258290

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	13.3

Source: 	OPPO

Title:  	Discussion on RAN-CN procedures and signalling for sensing

Document for:	Discussion and Decision

- Introduction

In Rel-20 5G-A ISAC SID [1], the objectives led by RAN3 are captured as follows.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution would like to provide our considerations on these objectives, mainly covering RAN-CN procedures and signalling for sensing.

- Discussion

UAV use cases

According to the Rel-20 5G-A ISAC SID [1], the scopes of Rel-20 5G-A ISAC only include gNB-based mono-static sensing for the use case of UAV sensing target.

In practical scenario, UAV regulation is essential, such as identifying unauthorized UAVs, where the sensing tasks include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

- The use cases of UAV sensing target include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

Based on TR 22.837 and TS 22.137, the requirements of detection and/or tracking of UAV are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

For example,

1/ For the use case of UAV detection, we only expect to identify the presence of targets, and optionally determine the number of existing targets in multi-target scenario. In this case, event-based or periodical reporting of sensing measurement results from gNB to Sensing Function is sufficient, and it seems no significant issue from each WGs.

2/ However, for the use case of UAV tracking, on top of UAV detection, we further expect to keep tracking the UAV flying continuously, and identify and distinguish a certain UAV target uniquely in multi-target scenario. In this case, even though the identification of non-3GPP target could be left to application layer via target feature exposure, the support of sensing service continuity might be required further, where all these issues need to coordinate with SA2. Besides, the feasibility of UAV tracking also depends on the evaluation of RAN1.

- The requirements of UAV detection and UAV tracking are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

Therefore, we understand that the UAV use case in SID should be clarified clearer.

- For the design of RAN-CN procedures and signalling, RAN3 assume that the UAV use cases at least include UAV detection. The support of UAV tracking is up to further progress of RAN1/SA2.

How to provide sensing capability to SF

In SA2 Rel-20 ISAC TR 23.700-14 [2], some solutions assume that every sensing entity registered its sensing capability at the SF, so that SF can perform procedures such as sensing entity selection accordingly, where the supported sensing capability may include its location or supported sensing service area and supported sensing mode and its role (as sensing transmitter, sensing receiver, or both).

In our understanding, the need of sensing capability reporting is under the assumption that the sensing entity could also be a UE. If the sensing entity is a UE, the procedure and signalling of sensing capability reporting between UE and SF is of course required considering different sensing capability of different UE. However, in case that the sensing entity only refers to gNB in this release, the need of sensing capability reporting is doubtful.

For gNB, there are two options to provide sensing capability to SF, including

- Option-1: Sensing capability reporting between gNB and SF, i.e., specify dedicated procedure and signalling to allow dynamic sensing capability reporting from gNB to SF.

- Option-2: OAM-based pre-configuration of sensing capability to SF, i.e., up to operator to configure sensing capability of gNB to SF statically.

Considering that for gNB, the sensing capability would not change dynamically, we understand Option-2 is sufficient, which does not require additional spec effort to specify the procedure and signalling.

- Considering that the sensing entity only refers to gNB in this release, the need to specify sensing capability reporting is doubtful, considering that for gNB the sensing capability would not change dynamically.

- RAN3 rely on OAM-based pre-configuration manner to configure sensing capability of gNB to SF.

Sensing request and report

In RAN3#129-bis meeting, the following TP for basic message flow of sensing reporting was agreed.

8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

Here we further discuss on some details of sensing request and report.

For sensing request message, based on SA2 Rel-20 ISAC TR 23.700-14 [2], we understand at least the sensing area information is needed as the parameters in sensing request message, to allow SF trigger sensing service on a certain area to gNB. The sensing area information could be in different granularities, such as cell level (i.e., cell list), tracking area level (i.e., TAC list), or even GAD shapes defined in TS 23.032.

- In the sensing request message, at least the sensing area information (e.g., the TAC list, cell list, or GAD shapes defined in TS 23.032) is included.

Regarding sensing reporting, as mentioned above, considering different UAV use cases such UAV detection and UAV tracking, it would lead to different reporting types of sensing measurement data from gNB to SF, such as

1/ one-shot reporting: gNB reports sensing measurement data to SF only once, based on sensing request.

2/ periodical reporting: gNB reports sensing measurement data to SF periodically, based on parameters (such as time interval, starting time, and/or number of reporting) provided in sensing request message.

3/ event-triggered reporting: gNB reports sensing measurement data to SF, only in case that reporting event is triggered. In this kind of reporting type, additional effort needed to define events. For example, considering that UAVs rarely appear in the sky, the reporting event for UAV detection could be defined based on the significant change of sensing measurement data, in order to avoid unnecessary reporting. The reporting event also need to be configured in sensing request message.

4/ semi-persistent reporting: gNB reports sensing measurement data once or periodically based on sensing measurement (de)activation. In this kind of reporting type, it introduces more flexibility for network. For example, it allows to provide sensing request to gNB firstly and activate sensing measurement and reporting later when gNB is ready. However, additional procedure/message to (de)activate sensing measurement and reporting is needed to design.

- In order to fit different UAV use cases such as UAV detection and UAV tracking, different reporting types of sensing measurement data could be considered, such as one-shot, periodical, event-triggered, and/or semi-persistent reporting.

In our understanding, one-shot reporting and periodical reporting as two basic reporting types, could be considered to support at least. For event-triggered or semi-persistent reporting, more inputs from RAN1/SA2 and more efforts from RAN3 perspective are both needed, so we can leave them as FFS.

- For sensing reporting procedure, RAN3 at least support one-shot and periodical reporting. FFS on event-triggered and semi-persistent reporting. Further check with RAN1/SA2 is needed.

Besides, in case of multiple sensing reporting types supported, sensing request message needs to capture the indication of sensing reporting type and/or the related configuration parameters, to distinguish among different reporting types and provide configuration parameters specific to different reporting types.

- If multiple sensing reporting types are supported, the indication of sensing reporting type and/or the related configuration parameters also need to be included in the sensing request message.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

UAV use cases

- The use cases of UAV sensing target include 1/ to detect the UAV existence (e.g., for intrusion detection), and 2/ to track the UAV flying (e.g., for collision detection and warning).

- The requirements of UAV detection and UAV tracking are quite different from each other, which may lead to different design on the network architecture and its procedures and signalling.

- For the design of RAN-CN procedures and signalling, RAN3 assume that the UAV use cases at least include UAV detection. The support of UAV tracking is up to further progress of RAN1/SA2.

How to provide sensing capability to SF

- Considering that the sensing entity only refers to gNB in this release, the need to specify sensing capability reporting is doubtful, considering that for gNB the sensing capability would not change dynamically.

- RAN3 rely on OAM-based pre-configuration manner to configure sensing capability of gNB to SF.

Sensing request and report

- In the sensing request message, at least the sensing area information (e.g., the TAC list, cell list, or GAD shapes defined in TS 23.032) is included.

- In order to fit different UAV use cases such as UAV detection and UAV tracking, different reporting types of sensing measurement data could be considered, such as one-shot, periodical, event-triggered, and/or semi-persistent reporting.

- For sensing reporting procedure, RAN3 at least support one-shot and periodical reporting. FFS on event-triggered and semi-persistent reporting. Further check with RAN1/SA2 is needed.

- If multiple sensing reporting types are supported, the indication of sensing reporting type and/or the related configuration parameters also need to be included in the sensing request message.

4. Reference

[1] RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[2] 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”, V1.0.0 (2025-09)