3GPP TSG-RAN WG3 Meeting #130	R3-258335

Dallas, USA, 17th – 21st November 2025

Agenda Item:	14.2

Source: 				LG Electronics Inc.

Title:			Discussion on support of Topology 2

Document for:		Discussion and Decision

1.	Introduction

In previous RAN3 meeting, it was agreed that the UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB. However, it is still FFS on whether this UE Reader authorization is forwarded from the source/old gNB to the target/new gNB, or from the gNB-CU to the gNB-DU. Also, the UE Reader ID allocation and UE Reader selection need to be further discussed in RAN3. In this contribution, we will discuss the RAN3 impacts on mechanisms to support the Topology 2, and provide our view on it.

2.	Discussion

In RAN3#129bis meeting, it was agreed that the NGAP procedures specified for Topology 1 in Rel-19 are used as a baseline for Topology 2 in Rel-20. Also, the AIOT procedures for Topology 2 captured in TR 38.769 [1] can be considered as a baseline. However, it is still missing on the protocol stack for Topology 2.

In TR 38.769 [1], the protocol stack for the RRC-based solution of Topology 2 is already captured as follows:

Figure 1: RRC based solution of Topology 2 in TR 38.769 [1]

We think that this protocol stack can be used as a baseline in Rel-20. In this figure, XXAP should be NGAP. In addition, considering the A-IoT NAS between A-IoT Device and the AIOTF, and provision of A-IoT specific information over NG interface as in Rel-19 Topology 1, the protocol stack for Topology 2 can be updated as follows:

Figure 2: RRC based solution of Topology 2

Therefore, the protocol stack in Figure 2 can be used as a baseline for the protocol stack of Topology 2.

Proposal 1: It is proposed to use the protocol stack in Figure 2 as a baseline for the protocol stack of Topology 2.

Regarding the UE Reader authorization, it is still FFS on whether this authorization information is delivered from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU. According to WID [2], the case where the UE Reader is served by the split RAN architecture should be considered. From the operator point of view, it is also possible to upgrade the existing split gNB to support the A-IoT service operation. Therefore, the CU-DU split case needs to be considered in this release.

In case of split gNB architecture, our view is that the gNB-DU should be in charge of the A-IoT radio resource allocation. This is because the gNB-DU is generally responsible for performing the resource allocation based on the request of the gNB-CU. To this end, as in V2X, the gNB-DU needs to know whether the UE is authorized as the Reader before allocating the A-IoT radio resource for that UE Reader. Therefore, upon the reception of the UE Reader authorization indication from the 5GC, the gNB-CU needs to forward it to the gNB-DU via F1AP signaling.

Proposal 2: The CU-DU split case should be considered for Topology 2.

Proposal 3: In CU-DU split case, the gNB-DU performs the A-IoT radio resource allocation for the UE Reader.

Proposal 4: In CU-DU split case, the Reader authorization indication is also provided to the gNB-DU via F1AP signaling.

Another open issue for the UE Reader authorization is whether this information is delivered from the source/old gNB to the target/new gNB via the Xn interface. In Topology 2, the UE Reader may be moved to other gNB. This mobility scenario is also considered in WID [2] as follows:

RP-252894 [2]: … RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility …

Therefore, it is required to consider how to handle the UE Reader authorization in case of the UE Reader mobility scenario. Our understanding is that the target/new gNB should have the knowledge on whether the UE is authorized to act as a Reader in order to properly allocate the A-IoT radio resources to the that UE.

Observation 1: During the UE mobility, the Reader authorization indication needs to be provided to the target/new gNB.

In NG-based HO, the AMF can provide the UE Reader authorization to the target/new gNB by using the NGAP HANDOVER REQUEST message. In Xn-based HO or Xn Context Retrieval, the AMF can also use the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message to provide the (updated) UE Reader authorization to the target/new gNB. Therefore, it is true that during the UE mobility, the target/new gNB receives the UE Reader authorization via the NGAP signaling. However, we think that in Xn-based HO or Xn Context Retrieval, the source/old gNB needs to provide this information to the target/new gNB by using the XnAP signaling (i.e., HANDOVER REQUEST message) as well. This is because the target/new gNB needs to know the UE Reader authorization during the handover preparation phase. For example, the target/new gNB may decide to reject the handover for that UE due to e.g., the current resource load of the A-IoT radio in the target/new gNB, and then send the XnAP HANDOVER PREPARATION FAILURE message to the source/old gNB. It is also possible that since the target/new gNB does not support the A-IoT operation, it may reject the handover of the UE upon the reception of the UE Reader authorization in the XnAP HANDOVER REQUEST message. Then, when the target/new gNB has the split RAN architecture, the gNB-CU of the target/new gNB should provide this authorization information to its gNB-DU as described in Proposal 4.

Proposal 5: The UE Reader authorization is provided to the target/new gNB by using the following signalings:

- XnAP HANDOVER REQUEST message,

- NGAP HANDOVER REQUEST message,

- NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

When the UE Reader authorization status changes (either “authorized”, or “not authorized”), the AMF informs the gNB (i.e., the NG-RAN serving the UE Reader) of the new authorization accordingly. If the UE Reader authorization status changes from “authorized” to “not authorized”, the gNB should stop allocating the A-IoT radio resources to the UE Reader because it cannot operate as the Reader (e.g. due to de-authorization). Once the UE Reader authorization status changes from “not authorized” to “authorized”, the gNB can (re-)initiate the resource allocation for the UE Reader.

Proposal 6: The updated UE Reader authorization status should be provided to the gNB.

Proposal 7: If the UE Reader authorization status changes from “authorized” to “not authorized”, the gNB should stop allocating the A-IoT radio resources to the UE Reader.

Proposal 8: Once the UE Reader authorization status changes from “not authorized” to “authorized”, the gNB can (re-)initiate the resource allocation for the UE Reader.

In Rel-19 Topology 1, the AIOTF is aware of the Reader related information (e.g., Reader ID, supported A-IoT Areas, Reader’s location) by means of OAM configuration. This is because the RAN Reader is fixed. However, in Rel-20 Topology 2, the UE Reader mobility is considered as mentioned above. Our understanding is that the A-IoT Area ID and the Reader ID of the UE Reader may need to be updated as it moves. Therefore, it seems to be difficult to preconfigure the Reader related information in AIOTF.

Observation 2: In Rel-20 Topology 2, it is difficult to preconfigure the Reader related information in AIOTF.

To alleviate this problem, the following potential options can be considered as a solution:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE. The UE Reader ID should be globally unique to identify the UE Reader in the network. In this option, the supported A-IoT Areas of the NG-RAN are consisted of the A-IoT Areas for the UE Reader(s) belonging to the NG-RAN. Therefore, the NG-RAN may frequently update its supported A-IoT Areas whenever the list of the UE Reader belonging to the NG-RAN is changed. During the Registration procedure, the UE reports its A-IoT Area ID(s) and the Reader ID to the AMF. Then, the AMF can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports.

- Option 2: During the Registration procedure, the NG-RAN determines/allocates the A-IoT Area ID and the Reader ID of the UE Reader, and then provides this information to the UE and the AMF, respectively. The AMF can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports. Since the UE Reader ID is allocated by the NG-RAN, the Reader ID collision may be occurred in AMF and/or AIOTF. Therefore, in order to avoid this problem, the AMF and/or AIOTF may also take into account the NG-RAN node ID. In addition, during the HO, the target NG-RAN should reallocate the UE Reader ID to avoid the Reader ID collision.

- Option 3: During the Registration procedure, the AMF determines/allocates the A-IoT Area ID and the Reader ID of the UE Reader, and then provides this information to the UE and the AMF, respectively. Also, it can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports. In this option, the Reader ID collision may be occurred in AIOTF as well. In addition, in case where the AMF is changed during the NG-based HO, new AMF should reallocate the UE Reader ID to avoid the Reader ID collision.

- Option 4: The AIOTF can determine/allocate the A-IoT Area ID and the Reader ID of the UE Reader. To this end, the AMF should request the allocation for the A-IoT Area ID and the Reader ID of the UE Reader to the AIOTF during the Registration procedure. In this option, the UE Reader ID is globally unique in AIOTF. Because the service area of the AIOTF is generally wide, the UE Reader can be uniquely identified in wide coverage. However, frequent signaling exchange between AMF and AIOTF may be occurred to manage the Reader related information (e.g., the A-IoT Area ID and UE Reader ID) due to e.g., authorization status change or UE RRC state change.

Based on these options, RAN3 needs to further discuss how to allocate/manage the Reader related information in Topology 2. Note that as shown in [3][4], SA2 also has on-going discussion on the UE Reader ID allocation. For now, there is no SA2 conclusion on this issue. Anyhow, this issue can be further discussed from the RAN3 point of view.

Proposal 9: It is proposed to discuss how to allocate/manage the Reader related information in Topology 2 based on the following options:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE.

- Option 2: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the NG-RAN.

- Option 3: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AMF.

- Option 4: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AIOTF.

As mentioned earlier, the UE in RRC_CONNECTED state can act as the UE Reader. Therefore, since the AMF already knows the UE location on a serving NG-RAN node granularity and the serving cell ID can be additionally provided to the AMF by using the User Location Information, the AMF can determine the UE Reader’s location on a per cell level granularity. If needed, the AMF is also able to deduce the UE Reader’s location on a finer granularity with the help of the LMF. Therefore, the AIOTF can receive the UE Reader’s location from the AMF directly, or retrieve this information from the UDM (or ADM) when the AMF updates it in the UDM (or ADM). Upon the reception of the UE Reader ID in the INVENTORY REPORT message as in Rel-19 Topology 1, the AIOTF can determine the A-IoT Device location. As a result, the A-IoT Device localization based on Reader ID as specified in Rel-19 can be considered.

Proposal 10: The A-IoT Device localization based on Reader ID can be considered as a baseline for Topology 2.

As shown in [3][4], there are two types of UE Reader in Topology 2: fixed UE Reader and mobile UE Reader. In case of the fixed UE Reader, there is no significant difference between the fixed UE Reader and RAN Reader in terms of the Reader selection by the AIOTF. As mentioned in Proposal 9, we think that the AIOTF can retrieve/acquire/receive the Reader related information (e.g., Reader ID, supported A-IoT Areas, Reader’s location) for the UE Reader from the AMF or UDM (or ADM), and then construct the list of the available UE Reader(s) within the service area of the AIOTF. Hence, by using this information, the AIOTF can perform the RAN node selection and UE Reader selection as in Rel-19. In Topology 2, however, the AF may indicate the specific target UE ID(s) (e.g., GPSI) to the AIOTF via NEF. In this case, the AIOTF just selects the target UE Reader(s) based on the UE ID(s) from the AF. In other words, the AIOTF determines the UE Reader ID(s) mapped to the received UE ID(s) from the AF, and then forwards it to the gNB. As a result, the gNB just follows the AIOTF decision, and then allocates to the UE Reader(s) the A-IoT radio resources for the inventory/command operation. At some time, the UE may be not authorized to operate as the Reader anymore, due to e.g., UE RRC state change or subscription update related to UE Reader authorization. In this case, the AIOTF is able to remove this UE Reader from the list of the available UE Reader(s) belonging to the AIOTF upon the reception of the event notification from the UDM or the AMF serving the UE Reader. This means that the AIOTF has the up-to-date list of the available UE Reader within its service area. Therefore, the AIOTF can determine the Requested Reader List based on the up-to-date list of the available UE Reader and the AF request. As a result, in the fixed UE reader case of Topology 2, there is no issue to reuse the Reader selection mechanism defined in Rel-19 Topology 1.

Regarding the mobile UE Reader, we also think that the Reader selection mechanism in Rel-19 Topology 1 can be used as a baseline for the mobile UE Reader type. Only a concern is that compared to the fixed UE Reader, the additional signalings are needed to make sure that the list of the available UE Reader in the AIOTF is up-to-date. That is, whenever authorization status change, UE mobility, or UE RRC state change is occurred, the AMF (or UDM) may frequently exchange the signaling to update the available UE Reader list in AIOTF with the AIOTF.

Proposal 11: The Reader selection mechanism in Rel-19 Topology 1 can be used as a baseline for Topology 2.

Proposal 12: Upon the reception of the specific UE Reader ID(s) from the AF via the NEF, the AIOTF just provides this UE Reader ID(s) to the gNB. Then, the gNB just follows the AIOTF instructions.

3.	Conclusion

In this contribution, we focused on the RAN3 impacts on mechanisms to support the Topology 2, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to use the protocol stack in Figure 2 as a baseline for the protocol stack of Topology 2.

Proposal 2: The CU-DU split case should be considered for Topology 2.

Proposal 3: In CU-DU split case, the gNB-DU performs the A-IoT radio resource allocation for the UE Reader.

Proposal 4: In CU-DU split case, the Reader authorization indication is also provided to the gNB-DU via F1AP signaling.

Proposal 5: The UE Reader authorization is provided to the target/new gNB by using the following signalings:

- XnAP HANDOVER REQUEST message,

- NGAP HANDOVER REQUEST message,

- NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

Proposal 6: The updated UE Reader authorization status should be provided to the gNB.

Proposal 7: If the UE Reader authorization status changes from “authorized” to “not authorized”, the gNB should stop allocating the A-IoT radio resources to the UE Reader.

Proposal 8: Once the UE Reader authorization status changes from “not authorized” to “authorized”, the gNB can (re-)initiate the resource allocation for the UE Reader.

Proposal 9: It is proposed to discuss how to allocate/manage the Reader related information in Topology 2 based on the following options:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE.

- Option 2: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the NG-RAN.

- Option 3: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AMF.

- Option 4: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AIOTF.

Proposal 10: The A-IoT Device localization based on Reader ID can be considered as a baseline for Topology 2.

Proposal 11: The Reader selection mechanism in Rel-19 Topology 1 can be used as a baseline for Topology 2.

Proposal 12: Upon the reception of the specific UE Reader ID(s) from the AF via the NEF, the AIOTF just provides this UE Reader ID(s) to the gNB. Then, the gNB just follows the AIOTF instructions.

4.	Reference

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, v19.0.0.

- RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, September, 2025.

- TR 23.700-30, “Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”, v0.2.0.

- S2-25xxxxx, “SA2#171 R20 Ambient IoT pre-meeting call”, Huawei, OPPO (rapporteurs), November, 2025.