3GPP TSG-RAN WG3 Meeting #130	R3-258340

Dallas, US, November 17-21, 2025

Agenda item:	10.3.2

Source:	Tejas Networks

Title:	Discussion on RAN-CN interface

Document for:	Discussion

# 1	Introduction

In this document, we discuss the impact of introducing SBI in RAN-CN interface.

# 2	Discussion

### 2.1 SBI vs P2P Interfaces: Flexibility and Modularity

Traditional point-to-point (P2P) interfaces have long been the backbone of mobile network communication, offering predictable and stable connectivity between network functions. However, as 6G introduces a new generation of data-centric services like AI/ML, ISAC, etc., the limitations of P2P become increasingly evident. These services require dynamic orchestration, real-time responsiveness, and the ability to adapt to changing network conditions. P2P interfaces, being rigid and tightly coupled, struggle to support such flexibility. They often rely on static configurations and centralized routing, which can become bottlenecks in high-demand, distributed environments.

In contrast, the Service-Based Interface (SBI) model introduces a paradigm shift by enabling loosely coupled, service-oriented communication. SBI allows RAN functions to discover and interact with Core Network Functions (NFs) dynamically, based on service needs and current conditions. This is particularly beneficial for AI/ML workloads, where inference engines may need to bind to different data sources or analytics modules depending on context. Similarly, ISAC services benefit from the ability to scale sensing, processing, and decision-making components independently. SBI supports this modularity through standardized APIs, service registration, and discovery mechanisms.

Moreover, SBI aligns with cloud-native principles, making it easier to deploy RAN functions as containerized microservices on commercial cloud platforms. This enables elastic scaling, fault isolation, and rapid service evolution. For example, a data analytics module can be updated or scaled without affecting the communication stack or other dependent services. This decoupling of components fosters innovation and agility, which are essential for the diverse and evolving service landscape of 6G. Overall, SBI provides the architectural flexibility needed to support modular, scalable, and intelligent network behavior for something P2P interfaces cannot efficiently deliver.

Observation 1: SBI offers architectural flexibility and modularity essential for supporting dynamic, scalable, and intelligent services in 6G, which traditional P2P interfaces cannot efficiently accommodate.

### 2.2 Operational and Architectural Benefits of SBI

Adopting SBI for the RAN-CN interface brings significant operational and architectural advantages that align with the long-term goals of 6G. One of the key benefits is enhanced agility where SBI enables network functions to interact in a service-driven manner, allowing for faster deployment and adaptation of new features. This agility is crucial for supporting AI/ML and ISAC services, which often require rapid updates and context-aware behavior. SBI also supports distributed intelligence by allowing RAN nodes to directly access relevant Core NFs, reducing reliance on centralized entities like AMF.

This decentralization leads to reduced signaling bottlenecks and improved efficiency in data exchange. For instance, instead of routing all requests through AMF, RAN functions can directly query NWDAF for analytics or UDM for user context, streamlining operations. SBI also enhances interoperability across vendors by adhering to open standards and APIs. This openness supports multi-vendor deployments and avoids vendor lock-in, fostering a competitive and innovative ecosystem. Additionally, SBI allows for independent evolution of control and user plane functionalities, enabling operators to upgrade or modify specific components without disrupting the entire system.

As 6G networks scale to support massive connectivity and diverse services, architectural flexibility becomes paramount. SBI provides the foundation for managing complex workloads, dynamic service chains, and heterogeneous deployments. However, this flexibility must be balanced by efficient service registration, discovery, and lifecycle management which are essential to prevent signaling overhead and maintain performance. With proper orchestration and coordination, SBI can transform the RAN-CN interface into a future-proof, intelligent, and adaptable framework that meets the demands of next-generation networks.

Observation 2: SBI offers the architectural flexibility needed for 6G scalability and service diversity, but its success depends on efficient service management to avoid signaling overhead and ensure performance.

2.3 SBI Integration Challenges

While SBI offers flexibility, it introduces security concerns due to broader network exposure, requiring coordination with SA3 to manage access control. Integrating all RAN nodes into the SBI chain of the Core Network is challenging due to the large number of NFs and potential transport network load issues. Additionally, lessons from 5GC show that SBI may increase procedural complexity and signaling overhead, potentially impacting operational efficiency

Proposal 1: RAN3 to study the feasibility and implications of introducing a Service-Based Interface (SBI) for the RAN-CN interface in 6G, considering architectural flexibility, service enablement, and operational efficiency.

# 3	Conclusions

Observation 1: SBI offers architectural flexibility and modularity essential for supporting dynamic, scalable, and intelligent services in 6G, which traditional P2P interfaces cannot efficiently accommodate.

Observation 2: SBI offers the architectural flexibility needed for 6G scalability and service diversity, but its success depends on efficient service management to avoid signaling overhead and ensure performance.

Proposal 1: RAN3 to study the feasibility and implications of introducing a Service-Based Interface (SBI) for the RAN-CN interface in 6G, considering architectural flexibility, service enablement, and operational efficiency.

# 4	References

[1] RP-251881, New SID: Study on 6G Radio, RAN#108

[2] RAN3#129bis chair note