3GPP TSG-RAN WG3 Meeting #130	R3-258362

Dallas, US, 17 – 21 November 2025

Agenda item:	10.3.1

Source: 	Lenovo

Title: 	General Principles and Requirements of 6G RAN-CN Functional Split and Interface

Document for:	pCR

1. Introduction

The general principles and requirements of 6G RAN-CN functional split and interface were discussed at the RAN3#129bis meeting with the following agreements [1].

The general principles for the specification of the 6G RAN-CN interface are as follows: - the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN; - the 6G RAN-CN interface supports control plane and user plane separation; - the 6G RAN-CN interface supports future enhancements; - the 6G RAN-CN interface supports all possible RAN deployment scenarios; - the 6G RAN-CN interface supports RAN sharing between multiple operators; - the 6G RAN-CN interface supports the operation of network slicing; - the 6G RAN-CN interface supports enhanced service awareness in RAN; - the 6G RAN-CN control plane interface supports reliable signalling transmission; - the 6G RAN-CN interface is designed with a clear functional split between RAN and CN. RAN-CN control plane interface supports following functions: - UE context management : The functionality to manage UE context between the RAN and CN; - Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress; - PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

In this contribution, we discuss the remaining issues of the general principles and requirements of 6G RAN-CN functional split and interface.

2. Discussion

As an initial step to define the 6G RAN-CN functional split and interface, it would be useful to take the 5G RAN-CN interface principles and NG interface functions [2] as a reference at first stage.

In 5G, the general principles of the NG interface are as follows:

-	the NG interface shall be open;

-	the NG interface shall support the exchange of signalling information between the New RAN and NGC;

-	from a logical standpoint, the NG is a point-to-point interface between a New RAN node and an NGC node. A point-to-point logical interface shall be feasible even in the absence of a physical direct connection between the New RAN and NGC;

-	the NG interface shall support control plane and user plane separation;

-	the NG interface shall separate Radio Network Layer and Transport Network Layer;

-	the NG interface shall be future proof to fulfil different new requirements and support of new services and new functions;

-	the NG interface shall be decoupled with the possible New RAN deployment variants.

-	the NG Application Protocol shall support modular procedures design and use a syntax allowing optimized encoding /decoding efficiency.

From our point of view, most of the above principles can be reused for the 6G RAN-CN interface, as they enable the successful deployment of the 5GS. Some of the principles were already captured in the TR, while others were not. Therefore, we propose to capture the following general principles into the TR:

-	the 6G RAN-CN interface shall be open;

-	the 6G RAN-CN interface shall separate Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface application protocol shall support modular procedures design and use a syntax allowing optimized encoding /decoding efficiency.

In the meanwhile, the syntax of the interface protocol is relevant to the interface option discussion, e.g., P2P vs. SBI. In legacy P2P protocol, e.g., NG-AP, uses ASN.1 syntax since it is very light and processing friendly. In comparison, the 5GC SBI uses OpenAPI based YAML as it’s more extensible, programmable and cloud-native. At the end, the selection of the syntax will have to balance the processing efficiency and the characteristics of the service/function.

Proposal 1: Capture the following general principles of the 6G RAN-CN interface into the 6G radio RAN3 TR.

-	the interface shall be open;

-	the interface shall separate Radio Network Layer and Transport Network Layer;

-	the interface application protocol shall support modular procedures design and use a syntax balancing the encoding/decoding efficiency and the service/function characteristics.

In 5G, the NG-C interface supports following functions:

-	Interface management: The functionality to manage the NG-C interface;

-	UE context management	: The functionality to manage the UE context between the New RAN and CN;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the New RAN and CN;

-	Transport of NAS messages: procedures to transfer NAS messages between the CN and UE;

-	Paging: The functionality to enable the CN to generate Paging messages sent to the New RAN and to allow the New RAN to page the UE in RRC_IDLE state;

-	PDU Session Management: The functionality to establish, manage and remove PDU sessions and respective New RAN resources that are made of data flows carrying UP traffic.

-	Configuration Transfer: the functionality to transfer the New RAN configuration information (e.g. transport layer addresses for establishment of  Xn interface) between two New RAN nodes via the NGC.

For the function split between 6GR node and 6G CN, the RAN-CN functional split in 5G showed its flexibility and multi-vendor support. In addition, motivated by the relatively recent uptake by many operators of 5GC, it is reasonable to assume that the 6G CN is an evolution of the 5GC to reduce the investments. In line with the assumption, the interfaces related to the 6G RAN can be the evolution of the interfaces related to the 5G RAN, and there is no strong requirement to change the functionality assignment between RAN and CN.

Some of the functions were already captured in the TR, while others were not. Therefore, they should be captured into the TR.

Besides the abovementioned functions, 6G will support a range of new services—such as AI-driven services and integrated sensing. To accommodate these emerging services, the 6G RAN-CN interface should be designed to enable the new type of services.

Proposal 2: Capture the following functions supported by the 6G RAN-CN interface into the 6G radio RAN3 TR.

-	Interface management;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the 6GR and 6G CN;

-	Paging: The functionality to enable the CN to generate Paging messages sent to the 6GR and to allow the 6GR to page the UE in RRC_IDLE state;

-	Configuration Transfer: the functionality to transfer the 6GR configuration information between two 6G BSs via the 6G CN.

-	New service messages: procedures to transfer the messages related to new service between the 6GR and 6G CN, e.g., data collection, AI, sensing.

Even though the functional split between the 6GR and 6G CN are similar with 5G, several things may be changed and quite different from 5G.

An important thing is that the 6G network are expected to include new service types beyond traditional communication, such as sensing and AI. As a result, more demands on network capability exposure will be required, while the complexity for configuration and internal function interaction should be reduced. A potential approach is to adopt a service-based interface (SBI) between the RAN and CN for the new functions supporting the new service types, which can avoid too many P2P interface definitions and unnecessary transparent forwarding among network entities.

Proposal 3: RAN3 to discuss the Service Based Interface between the RAN and CN for the new functions supporting AI and sensing services.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: Capture the following general principles of the 6G RAN-CN interface into the 6G radio RAN3 TR.

-	the interface shall be open;

-	the interface shall separate Radio Network Layer and Transport Network Layer;

-	the interface application protocol shall support modular procedures design and use a syntax balancing the encoding/decoding efficiency and the service/function characteristics.

Proposal 2: Capture the following functions supported by the 6G RAN-CN interface into the 6G radio RAN3 TR.

-	Interface management;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the 6GR and 6G CN;

-	Paging: The functionality to enable the CN to generate Paging messages sent to the 6GR and to allow the 6GR to page the UE in RRC_IDLE state;

-	Configuration Transfer: the functionality to transfer the 6GR configuration information between two 6G BSs via the 6G CN.

-	New service messages: procedures to transfer the messages related to new service between the 6GR and 6G CN, e.g., data collection, AI, sensing.

Proposal 3: RAN3 to discuss the Service Based Interface between the RAN and CN for the new functions supporting AI and sensing services.

4. Reference

- TR 38.760-3 v0.1.0: Study on 6G radio RAN3 aspects.

- TR 38.801: Study on new radio access technology: radio access architecture and interfaces.

# 5. Text Proposal

pCR for TR 38.760-3.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN;

-	the 6G RAN-CN interface shall be open;

-	the 6G RAN-CN interface shall separate Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface application protocol shall support modular procedures design and use a syntax balancing the encoding/decoding efficiency and the service/function characteristics.

6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress;

-	Interface Management;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN;

-	Paging: The functionality to enable the CN to generate Paging messages sent to the RAN and to allow the RAN to page the UE in RRC_IDLE state;

-	Configuration Transfer: The functionality to transfer the 6GR configuration information between two 6G BSs via the CN;

-	New service messages Transfer: The functionality to transfer the messages related to new service between the RAN and CN, e.g., data collection, AI, sensing.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>