3GPP TSG-RAN WG3 #130	R3-258365

Dallas, USA, 17th – 21st Nov 2025

Agenda Item:	10.2.1

Source:	Lenovo

Title: 		Consideration on 6G data collection framework

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the considerations for 6G data collection framework from RAN3 perspective.

# Discussion

6G SID - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

RAN2#131bis Chair Notes: For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

SA2#171 S2-2509794 [WT#5 Data framework] WT scope and KI on 6G Data framework 3、whether we focus on data produced and consumed by CN first in this WT? or no limitation on data source and data consumer？ Majority view: no limitation, UE, 6G RAN, 6G CN and AF can all be data producers or data consumers. One company: focus on data produced and consumed by CN only; One company: to make progress, can focus on CN data first. To make progress, pen holder propose: Way forward 3: no limitation on data source and data consumer.

For 6G data transfer, there is a common interest to design a more unified framework compared to 5G, that can be used to transfer multiple types of data (e.g., AIML, sensing, SON/MDT, QoE). RAN2 and SA2 had initial discussion in their last meeting, and with the aim of a “unified data collection framework” there seems to be also a common interest to consider all possible scenarios, e.g., data transfer among UE, RAN, CN, and OAM.

RAN2 and SA2 have interest to consider all possible scenarios (e.g., data transfer among UE, RAN, CN, OAM) for data collection framework discussion.

The first question is what are the scenarios that should be considered in RAN3 data collection framework study. In our understanding, the following scenarios can be considered at a start point:

| Data Source (where data is originally generated) | Data Consumer | Possible Intermediate Hop | Example |
| --- | --- | --- | --- |
| RAN | UE | None/CN/OAM | Support of on-device AIML training/refining/monitoring for a UE-side model Depending on how data is delivered, it can be delivered directly via air interface or via UP from CN. |
|  | RAN | None/CN/OAM | Support of AIML training for a RAN-side model |
|  | CN | None/OAM | Network performance analysis in CN |
|  | OAM | None/CN | Network performance monitoring Including the case of MDT/Tracing, wherein the report is originally generated by RAN node |
| UE | RAN | None/RAN/CN/OAM | Support of AIML training/finetuning/monitoring for a RAN-side model Depending on how UE side data is collected, it can be collected directly via air interface or via UP to CN first. Including the case of SON/MDT/QoE, wherein the report is originally generated by UE. |
| RAN |  | None/CN/OAM | Support of AIML training/finetuning/monitoring for a RAN-side model |
| CN |  | None/OAM | Support of AIML training/finetuning/monitoring for a RAN-side model |
| OAM |  | None/CN | Support of AIML training/finetuning/monitoring for a RAN-side model |

For the data collection framework study, RAN3 initially considers the following scenarios:

  - RAN is data source (where data is originally generated), and data consumer can be UE/RAN/CN/OAM with/without intermediate hop(s).

  - RAN is data consumer, and data source can be UE/RAN/CN/OAM with/without intermediate hop(s).

To have a data collection framework that is efficient and secured, we believe the following general requirement can be considered.

- When RAN is data source, the repetitive data provision should be avoided, e.g., providing the same data to different consumers.

- When RAN is data consumer, the repetitive data request should be avoided, e.g., requesting the same data from different data sources.

- The data collected is secured, and confidentiality for that data is ensured.

- The design is future-proof and extendable.

- User data privacy, anonymity and user consent is respected.

The following requirements are considered for data collection framework study:

  - When RAN is data source, the repetitive data provision should be avoided, e.g., providing the same data to different consumers.

  - When RAN is data consumer, the repetitive data request should be avoided, e.g., requesting the same data from different data sources.

  - The data collected is secured, and confidentiality for that data is ensured.

  - The design is future-proof and extendable.

  - User data privacy, anonymity and user consent is respected.

Another aspect that would be good to clarify is that, we believe the data collection framework study does not include those measurement/data that needs to be transferred in real-time, e.g., the UE measurement result transfer during handover, or input data for real-time AIML inference (e.g., radio status), or input data for real-time sensing.

Comparatively, the following use cases can be considered initially for data collection framework study: AIML training, sensing, QoE, SON, MDT.

RAN3 clarifies that the collection of real-time data is not within the scope of data collection framework study.

RAN3 considers the following use cases in data collection framework study: AIML training, sensing, QoE, SON, MDT.

# Conclusion

Based on the discussion above, we observe:

Observation 1	RAN2 and SA2 have interest to consider all possible scenarios (e.g., data transfer among UE, RAN, CN, OAM) for data collection framework discussion.

Based on the discussion above, we propose:

Proposal 1	For the data collection framework study, RAN3 initially considers the following scenarios:

a.	RAN is data source (where data is originally generated), and data consumer can be UE/RAN/CN/OAM with/without intermediate hop(s).

b.	RAN is data consumer, and data source can be UE/RAN/CN/OAM with/without intermediate hop(s).

Proposal 2	The following requirements are considered for data collection framework study:

a.	When RAN is data source, the repetitive data provision should be avoided, e.g., providing the same data to different consumers.

b.	When RAN is data consumer, the repetitive data request should be avoided, e.g., requesting the same data from different data sources.

c.	The data collected is secured, and confidentiality for that data is ensured.

d.	The design is future-proof and extendable.

e.	User data privacy, anonymity and user consent is respected.

Proposal 3	RAN3 clarifies that the collection of real-time data is not within the scope of data collection framework study.

Proposal 4	RAN3 considers the following use cases in data collection framework study: AIML training, sensing, QoE, SON, MDT.

# TP to TR38.760

***********************************  Change Starts ********************************

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

The general principles for the data collection architecture are as follows:

-	When RAN is data source, the repetitive data provision should be avoided, e.g., providing the same data to different consumers.

-	When RAN is data consumer, the repetitive data request should be avoided, e.g., requesting the same data from different data sources.

-	The data collected is secured, and confidentiality for that data is ensured.

-	The design is future-proof and extendable.

-	User data privacy, anonymity and user consent is respected.

***********************************  Change Ends ********************************