3GPP TSG-RAN WG3 Meeting #130	R3-258391

Dallas, U.S.A., 17-21 November 2025

Title:	(pCR to TR 38.745) AI/ML based mobility – Multi-hop UE trajectory

Source:	Ericsson

Agenda item:	12.2.1

Document Type:	Discussion and agreement

# Introduction

At the RAN3#129-bis meeting, the following agreements and “to-be-continued” were captured in the meeting minutes in relation to the multi-hop UE trajectory across gNBs scenario for the AI/ML-based mobility use case:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

Configuration of measurement feedback:

- Data collection initiate procedure

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

In this contribution, we discuss the consequences of the above agreements and analyze the possible transfer options for measured multi-hop UE trajectory.

# Discussion on support for multi-hop UE trajectory

## Scope of the multi-hop UE trajectory scenario

During the RAN3#129-bis meeting, the following agreement was captured:

Study multi-hop UE trajectory for L3 HO.

As we discussed in our previous contribution [1] in relation to legacy L3 handover scenarios, as a UE moves in connected mode across multiple gNBs, each handover is configured independently by the serving gNB at each hop. As a result, the first gNB has no practical reason to collect measured multi-hop UE trajectory and train AI/ML models that provide predictions across multiple gNBs, since it cannot influence the UE’s mobility beyond the initial handover. The “single hop” nature of legacy L3 handover preparation thus limits the usefulness of multi-hop UE trajectory collection at the first gNB.

During the offline discussion at the previous meeting, a few companies argued that the benefit of multi-hop UE trajectory lies in “subsequent mobility decisions” [2]. In the context of legacy L3 handover scenarios, this should be interpreted as signaling the predicted multi-hop UE trajectory to future gNBs to assist them in making the “subsequent mobility decisions” since, as described above, it is the future gNBs that configure and execute the “subsequent mobility decisions”. However, it is unclear how accurate such prediction can be several hops ahead in the UE’s path, and therefore, how practically useful this scenario would be.

Observation 1:  A UE trajectory prediction across multiple hops is most accurate for the first hop and becomes progressively less reliable the further into the future it extends.

Observation 2:  In legacy L3 handover scenarios, the predicted multi-hop UE trajectory is unlikely to be reliable for “subsequent mobility decisions” beyond the first hop.

On the other hand, the usefulness of multi-hop UE trajectory for “subsequent mobility decisions” is evident when considered in the context of inter-gNB (subsequent) LTM. In these mobility scenarios, the source gNB is responsible for configuring multiple candidate cells, potentially across multiple nodes, by means of the Handover Preparation procedure, which creates a clear need for multi-hop trajectory information at the source gNB. By collecting measured multi-hop UE trajectory, the source gNB can train (or re-train) AI/ML models that predict UE mobility patterns across multiple nodes. Since the LTM configuration is determined by the first gNB across multiple hops, having access to multi-hop UE trajectory (both measured and predicted) at that node is clearly beneficial.

It is important to note that in inter-gNB (subsequent) LTM scenarios, the source gNB is not required to predict a single, accurate UE path to be signaled during the Handover Preparation procedure, like in legacy L3 handover scenarios as previously discussed. Instead, the source gNB can predict multiple possible UE paths, which is entirely acceptable in this context. This flexibility is enabled by the ability of the source gNB to configure multiple LTM candidate cells, thereby accommodating uncertainty in the UE’s future trajectory.

Observation 3:  In inter-gNB (subsequent) LTM scenarios, the predicted multi-hop UE trajectory is used to select candidate cells for mobility, making it a more realistic application of trajectory predictions that may lose accuracy several hops in the future.

For these reasons, and because the work on the AI/ML-based mobility use case should take a forward-looking perspective, we believe that inter-gNB (subsequent) LTM scenarios should be included within the scope of multi-hop UE trajectory. RAN3 should aim for a simple, low-impact solution that fits within the current specifications and is applicable to both legacy L3 handover and inter-gNB LTM scenarios.

Proposal 1:  RAN3 to consider inter-gNB (subsequent) LTM scenarios within the scope of multi-hop UE trajectory.

## Procedures for the multi-hop UE trajectory scenario

During the RAN3#129-bis meeting, the following agreement was captured in the meeting minutes:

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

While this agreement is a step forward, we believe it should be extended to cover the entire trajectory, not just the first hop.

The Data Collection Reporting Initiation and Data Collection Reporting procedures over the Xn interface were standardized in Rel-18 to enable efficient data collection between gNBs. These procedures were further enhanced in Rel-19 and, after two releases of development and refinement, are now well-defined, robust, and applicable to a wide range of use cases. Restricting their use to only the first hop of measured multi-hop UE trajectory introduces unnecessary complexity and would require the creation of new procedures for subsequent hops, an approach we believe is neither efficient nor justified.

Instead, RAN3 should aim to reuse the existing, proven procedures for data collection across the full trajectory, ensuring consistency, minimizing specification impact, and leveraging the strengths of the current framework.

Proposal 2:  RAN3 to agree to use the Data Collection Reporting Initiation and Data Collection Reporting procedures in support of measured multi-hop UE trajectory across all hops.

## Transfer of measured multi-hop UE trajectory

### Different approaches

During the RAN3#129-bis meeting, the following options were noted as candidates for the transfer of measured multi-hop UE trajectory:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

•	Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Option 4 was excluded already during the same meeting due to the following agreement:

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

Furthermore, Option 3 should also be excluded based on our discussion in the preceding section and summarized in Proposal 2, since it is not possible for Node 2 to report the measured UE trajectory forward to Node 3 using the Data Collection framework.

This leaves Option 1 (direct reporting to the first gNB) and Option 2 (hop-by-hop reporting) as the remaining candidates for further analysis. While the listed options only describe the reporting direction, it should be assumed that the configuration follows a similar structure, but in the opposite direction.

Proposal 3:  RAN3 to agree that, in Option 1, the configuration is done in parallel by the initial source gNB (i.e., Node1 to Node2, Node1 to Node3), while in Option 2, the configuration is done hop by hop (i.e., Node1 to Node2, Node2 to Node3).

In the following, we discuss these two options.

### Limitations of the hop-by-hop approach

In the hop-by-hop approach, the trajectory prediction is piggybacked forward as the UE moves, while the measured trajectory is propagated backward along the UE’s path as depicted in Figure 1. However, this approach introduces significant complexity. For measured UE trajectory reporting, it requires that all gNBs along the path have compatible Data Collection contexts configured. For example, the collection duration in step 5 in Figure 1 depends on the configuration in step 1 and the time of stay in step 4.

More problematic is the fact that intermediate nodes must store locally measured UE trajectory until exit conditions are triggered in the final node, at which point the information cascades backward; in Figure 1, gNB2 needs to store the UE trajectory from step 4 until it receives gNB3’s trajectory in step 9. The storing of information includes also UE context storing, which to a certain extent needs to be in place to be able to associate the information reported from gNB3 with the UE for which the measured trajectory monitoring was triggered.

We note that the Collection Time Duration for UE Trajectory IE, which is part of the UE Trajectory Collection Configuration IE, allows to configure a collection time of up to 4096 seconds. This means that intermediate nodes may need to store information for potentially more than one hour!

Observation 4:  The “hop-by-hop” approach for multi-hop UE trajectory forces intermediate gNBs to store information (e.g., UE context, measured UE trajectory) for a potentially long time after the UE has left.

One of the main drawbacks of the “hop-by-hop” approach, though, is that intermediate gNBs most likely need to set up Data Collection contexts with multiple neighboring gNBs, even if they do not intend to collect data themselves. This is a commonly overlooked issue that is not clearly reflected in simplified flowcharts like the one in Figure 1. Such diagrams often make the incorrect assumption that gNB2 only needs to set up a Data Collection context with gNB3, whereas in practice, gNB2 may need to configure contexts with several neighbors, making the procedure significantly more complex.

Figure 1: Hop-by-hop approach flow diagram from the RAN3#129-bis meeting SOD [2]

The reason why gNB2 may need to configure contexts with several neighbors is because the true path of the UE is not perfectly known in advance. A gNB cannot wait until handover is imminent to initiate the Data Collection Reporting Initiation procedure (as in step 5) with the relevant target gNB. Instead, it must preemptively start the procedure with several neighbors. This leads to a cascading effect: if one gNB requests measured multi-hop UE trajectory, its neighbors must forward the request to their own neighbors, and so on. As a result, even though the “hop-by-hop” approach can technically be implemented using the Rel-18 Data Collection framework, it is not a technically sound solution due to its inherent complexity and inefficiency.

Observation 5:  The “hop-by-hop” approach for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework because it requires intermediate gNBs to preemptively set up contexts with multiple neighbors, leading to unnecessary complexity and cascading configuration overhead.

In summary, we argue that it is not a technically sound solution to require intermediate gNBs to store information they have no interest in, nor to compel them to establish Data Collection contexts with other gNBs solely to relay data they themselves do not need. The “hop-by-hop” approach, which appears simpler in flowcharts like the one in Figure 1, introduces unnecessary complexity and resource overhead for nodes that are not actively interested in this data. A more efficient and scalable solution is to delegate this responsibility to the first gNB, which is the node interested in the data. This first gNB should directly request and collect the measured trajectory information from all relevant future gNBs, as we explain next.

Proposal 4:  RAN3 to agree that the “hop-by-hop” approach for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework and should not be considered.

### Direct reporting to the first gNB

Given the above arguments, we consider Option 1 (direct reporting to the first gNB) to be the most technically sound approach for both the configuration and reporting of measured multi-hop UE trajectory. While this approach requires a few additions to the specifications, these changes are minimal in scope and impact. Moreover, the cost of these additions is significantly lower than the complexity and inefficiencies introduced by the hop-by-hop approach.

The key aspects of this approach are the following:

- The gNB interested in collecting multi-hop measured UE trajectory needs to request this information to relevant gNBs with which it has an active Xn connection.

- The relevant Data Collection IDs are sent in the HANDOVER REQUEST message as well as a UE ID that the gNBs need to use when reporting the collected data.

- The Data Collection IDs and UE ID are forwarded by every intermediate gNB in subsequent HANDOVER REQUEST messages to other gNBs. The remaining collection time and remaining number of cells is also included in these messages.

  - If the UE arrives to a gNB that has not been requested the collection of multi-hop measured UE trajectory, the gNB will not recognize any of the Data Collection IDs and this information will not be forwarded anymore.

- Each gNB that serves the UE reports the locally collected measured UE trajectory to the requesting gNB including the received UE ID.

Figure 2 below shows a flow diagram for collection of measured multi-hop UE trajectory with this approach.

Figure 2: Flow diagram for collection of measured multi-hop UE trajectory

The main steps of the procedure are as follows:

- gNB1 sends a DATA COLLECTION REQUEST message to gNB2 with a request for measured multi-hop UE trajectory, and gNB2 responds with a DATA COLLECTION RESPONSE message accepting the request. The values for NG-RAN node1 Measurement ID IE and NG-RAN node2 Measurement ID IE obtained in this step are called “Data Collection ID 1” for simplicity.

- gNB1 sends a DATA COLLECTION REQUEST message to gNB3 with a request for measured multi-hop UE trajectory, and gNB3 responds with a DATA COLLECTION RESPONSE message accepting the request. The values for NG-RAN node1 Measurement ID IE and NG-RAN node2 Measurement ID IE obtained in this step are called “Data Collection ID 2” for simplicity.

- gNB1 decides to hand over a UE to gNB2, and thus it initiates a HANDOVER REQUEST message where it includes the relevant Data Collection IDs (i.e., “Data Collection ID 1” and “Data Collection ID 2”) to indicate the request of measured multi-hop UE trajectory for this UE. gNB1 also includes a UE ID that should be used when reporting the collected measured multi-hop UE trajectory.
Although not shown in the flow diagram, gNB2 responds with a HANDOVER REQUEST ACKNOWLEDGE message.

- The UE is successfully handed over to gNB2, which starts collecting the measured UE trajectory.

- gNB2 decides to hand over the UE to gNB3, and thus it initiates a HANDOVER REQUEST message where it forwards the remaining Data Collection IDs signaled by gNB1 (i.e., “Data Collection ID 2”) to indicate the request of measured multi-hop UE trajectory for this UE. gNB2 also includes the UE ID, signaled by gNB1, that should be used when reporting the collected measured multi-hop UE trajectory.
Although not shown in the flow diagram, gNB3 responds with a HANDOVER REQUEST ACKNOWLEDGE message.

- The UE is successfully handed over to gNB3, which starts collecting the measured UE trajectory.

- Since the UE is no longer being served by gNB2, gNB2 sends a DATA COLLECTION UPDATE message to gNB1 with the measured UE trajectory in cells of gNB2. The UE Assistant Identifier IE linked to the measured UE trajectory is the UE ID signaled in step 3.

- An exit condition for collection of measured UE trajectory is reached, e.g., the UE goes to idle or the collection time duration is exceeded. gNB3 thus terminates the measured multi-hop UE trajectory collection.

- gNB3 sends a DATA COLLECTION UPDATE message to gNB1 with the measured UE trajectory for cells of gNB3. The UE Assistant Identifier IE linked to the measured UE trajectory is the UE ID signaled in step 5.

As it is seen in the described steps, the legacy Data Collection framework allows the collection of measured multi-hop UE trajectory almost without any change, only the HANDOVER REQUEST message needs to be enhanced. We believe that these enhancements have a very limited impact while enabling a robust solution for the collection of measured multi-hop UE trajectory. Therefore, RAN3 should consider them as a preferred solution.

Proposal 5:  RAN3 to agree to the collection of measured multi-hop UE trajectory using Option 1 (direct reporting to the first gNB).

## Predicted multi-hop UE trajectory

As we mentioned in Section 2.1, a few companies believe that the benefit of multi-hop UE trajectory lies in “subsequent mobility decisions” [2] which, in the context of legacy L3 handover scenarios, can only be interpreted as signaling the predicted multi-hop UE trajectory to future gNBs. However, we argued that this benefit may be difficult to realize in practice, as trajectory predictions tend to lose accuracy the further into the future they extend, thereby limiting their usefulness for mobility decisions beyond the first hop (see Observation 1 and Observation 2).

Nonetheless, the HANDOVER REQUEST message already allows a source gNB to signal the predicted UE trajectory in cells of the target gNB. Extending this behavior to include cells of multiple gNBs in the Cell Based UE Trajectory Prediction IE is trivial. Each gNB receiving a predicted multi-hop UE trajectory in an incoming HANDOVER REQUEST message may, at its discretion, forward the relevant portion of this information to subsequent gNBs during future mobility events. Importantly, the decision to forward such predictions should remain optional and left to the serving gNB, as it may be under heavy load or may determine that the prediction is no longer valid or useful.

Proposal 6:  RAN3 to agree to the forwarding of predicted multi-hop UE trajectory by intermediate gNBs if it is an optional feature.

Proposal 7:  RAN3 to agree to the pCR to TR 38.745 in Appendix A.

# Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1:  A UE trajectory prediction across multiple hops is most accurate for the first hop and becomes progressively less reliable the further into the future it extends.

Observation 2:  In legacy L3 handover scenarios, the predicted multi-hop UE trajectory is unlikely to be reliable for “subsequent mobility decisions” beyond the first hop.

Observation 3:  In inter-gNB (subsequent) LTM scenarios, the predicted multi-hop UE trajectory is used to select candidate cells for mobility, making it a more realistic application of trajectory predictions that may lose accuracy several hops in the future.

Proposal 1:  RAN3 to consider inter-gNB (subsequent) LTM scenarios within the scope of multi-hop UE trajectory.

Proposal 2:  RAN3 to agree to use the Data Collection Reporting Initiation and Data Collection Reporting procedures in support of measured multi-hop UE trajectory across all hops.

Proposal 3:  RAN3 to agree that, in Option 1, the configuration is done in parallel by the initial source gNB (i.e., Node1 to Node2, Node1 to Node3), while in Option 2, the configuration is done hop by hop (i.e., Node1 to Node2, Node2 to Node3).

Observation 4:  The “hop-by-hop” approach for multi-hop UE trajectory forces intermediate gNBs to store information (e.g., UE context, measured UE trajectory) for a potentially long time after the UE has left.

Observation 5:  The “hop-by-hop” approach for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework because it requires intermediate gNBs to preemptively set up contexts with multiple neighbors, leading to unnecessary complexity and cascading configuration overhead.

Proposal 4:  RAN3 to agree that the “hop-by-hop” approach for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework and should not be considered.

Proposal 5:  RAN3 to agree to the collection of measured multi-hop UE trajectory using Option 1 (direct reporting to the first gNB).

Proposal 6:  RAN3 to agree to the forwarding of predicted multi-hop UE trajectory by intermediate gNBs if it is an optional feature.

Proposal 7:  RAN3 to agree to the pCR to TR 38.745 in Appendix A.

# References

- R3-257072, (TP to TP to TR 38.745) Support for multi-hop UE trajectory, Ericsson

- R3-257236, Summary of Offline Discussion for CB: # 21_AIMLMultiHop, ZTE Corporation (moderator)

# Appendix A – pCR to TR 38.745 – Multi-hop UE trajectory

# 4	Use cases and Solutions

<<<<<<<<<<<<<<<<<<<< First Changes >>>>>>>>>>>>>>>>>>>>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction and cell-based measured UE trajectory are limited to the first-hop target NG-RAN node.

Predicted multi-hop UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order. A gNB receiving a predicted multi-hop UE trajectory in relation to an incoming UE may forward the relevant portion of this information to subsequent gNBs during future mobility events.

Measured multi-hop UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE was connected and these cells are listed in chronological order.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input to predicted multi-hop UE trajectory

The following data is required as input data for predicted multi-hop UE trajectory.

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

-	UE Mobility History Information.

From the neighbouring RAN nodes:

-	UE History Information from neighbour.

-	UE handovers or LTM cell switches in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism.

-	Current/predicted resource status.

From the local node:

-	Current/predicted resource status.

-	Current/predicted UE traffic.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>