3GPP TSG-RAN WG3#130	R3-258395

Dallas, U.S.A., 17-21 November 2025

Title: 	(pCR to TR 38.745) AI/ML based mobility – Handover Enhancements

Source: 	Ericsson

Agenda item:	12.2.3

Document Type:	Discussion and Decision

# Introduction

The objective of the Study on AI/ML for NG-RAN Phase 3 (RP-252867) is to explore new scenarios for the AI/ML-based mobility use case and to identify enhancements to support AI/ML functionality, and is specified as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

  - handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

During the RAN3#129-bis meeting, very little progress was made for the SI objective related to handover enhancements, and only the following FFS was captured:

- Study inter-CU LTM in R20 SI?

However, the “handover enhancements” is one of the scenario stipulated by the SID, therefore RAN3 should discuss it, and provide conclusions as to whether some normative work is needed.

# Discussion

## Handover enhancements for AI/ML based mobility

### Present status

In Rel-18, the AI/ML for NG-RAN WI introduced signalling procedures to configure and report data pertaining to different use cases, and further extensions were agreed in Rel-19. In particular, the Data Collection Reporting Initiation and Data Collection Reporting procedures introduced in Rel-18 allow the configuration and the reporting of AI/ML-related information between neighbouring nodes.

To obtain UE related measurements (UE performance or UE’s trajectory) after a UE is handed over from a source gNB to a target gNB, the source gNB can start a Data Collection Reporting Initiation procedure towards the target gNB; the combination of the Measurement IDs assigned by the source gNB and the target gNB, respectively, determine the Data Collection ID of the procedure. When the source gNB initiates the handover, it includes the Data Collection ID in the HANDOVER REQUEST message. When the target gNB receives the Data Collection ID in the HANDOVER REQUEST message, it understands that it needs to collect and report to the source gNB the previously requested measurements, for the incoming UE.

In the following, we describe further enhancements to improve these procedures to support additional scenarios.

### Data collection and reporting enhancements

The request for collection (and the corresponding reporting) of UE related measurements has been specified without any consideration for LTM. Although it can be expected that the current signalling design can be reused to a large extent, LTM introduces some differences compared to “legacy” L3 handover (e.g., new procedures to notify the cell switch execution, support for subsequent LTM) that were not considered when defining data collection and reporting for legacy handover. We think RAN3 should look at these gaps, and study potential updates in the specification to take LTM into account.

For example, in a subsequent LTM scenario, the current means to configure and report UE measurements after mobility may not be enough to collect UE performance. As an example, consider a UE that is configured with LTM candidate cells belonging to three gNBs: gNB1 (configuring gNB), gNB2, and gNB3. First, there is an LTM cell switch executed from gNB1 to gNB2, and then a subsequent LTM cell switch is done from gNB2 to gNB3; in this case, gNB2 cannot use the HANDOVER REQUEST message, i.e., the message currently used to request measurements from the target in L3 handover, to let gNB3 understand that gNB2 is interested in receiving UE performance after the subsequent LTM cell switch to gNB3.

Observation 1: The current signaling for configuring and reporting UE feedback after cannot support subsequent LTM.

Another aspect we think is worth looking at concerns the conditions for terminating the collection of requested UE feedback when LTM is used. As we know, in the context of UE performance collection, TS 38.423 currently specifies the following:

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node 2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node 2 shall terminate the collection when at least one of the following conditions is fulfilled:

- the time since UE was successfully handed over to NG-RAN node 2 is equal to the value of the Collection Time Duration for UE Performance IE;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

As of now, the exit conditions for UE performance collection do not distinguish if an LTM or layer 3 handover is used. Therefore, if a UE is configured with intra- and inter-gNB LTM candidate cells and moves between different LTM candidate cells, the current definition of the exit conditions imposes that the collection of UE performance terminates when one (the first) LTM cell switch is performed.

For LTM the current definition is limiting, as there are situations in which collecting UE performance in one node for more than one mobility event can be used for LTM optimization purposes. Consider a UE configured with LTM candidate cells belonging to two gNBs: Cell A in gNB1 (configuring gNB), Cell B and Cell C in gNB2. The gNB2 also has another cell, Cell D, which is not an LTM candidate cell. First, there is an LTM cell switch executed from gNB1 to gNB2 (Cell A to Cell B), and then a L3 handover from Cell B to Cell D (i.e., the UE remains in gNB2). The gNB1 does not receive UE performance for the UE when it is served by Cell D because the exit conditions stipulates that the collection is terminated when the first handover is done. However, it would be good for the gNB1 to know such performance, because then it may choose for example to add Cell C as an LTM candidate, which that can be visited by the UE due to subsequent LTM. In this way, the advantage of LTM e.g., derived from the early acquisition of the TA value for Cell C can be exploited and the mobility interruption time reduced. Also, the gNB1 may decide to not consider Cell D as LTM candidate cell, if that is never or rarely used. We also note that in the proposed example, even if the UE had first moved from Cell A to Cell B, and then to Cell C (i.e., the UE remained within the set of LTM candidate cells), the current signalling does not allow the gNB2 (if requested by gNB1) to understand that it should provide UE performance when in Cell C, because the HANDOVER REQUEST message is not used to initiate the subsequent LTM.

Without going into details, we think that a similar reasoning as above also applies to the case of UE trajectory collection.

Observation 2: The current exit conditions defined for terminating the collection of UE related feedback are not optimal for LTM.

Therefore, we think it is beneficial (e.g., to optimize intra- or inter-gNB LTM candidate cell selection) for RAN3 to study enhancements in the reporting of UE feedback in case of LTM.

Proposal 1: As part of handover enhancements for AI/ML based mobility, study enhancements for configuring and reporting UE feedback in case of LTM.

### Enhancements to UE performance collection

As previously described, in case of handover, the exit conditions under which a target gNB (e.g. gNB2) terminates the collection of UE performance and reports it back to the source gNB (e.g., gNB1) are described in clause 8.4.13.2 in TS 38.423. In the case of measured UE performance, for single connectivity, the exit conditions are expiration of a timer, the UE moving to RRC_IDLE or RRC_INACTIVE state, or the UE moving to another cell.

When receiving the UE feedback, gNB1 is not aware of the particular exit condition that was met in gNB2, nor the amount of time during which gNB2 collected the data. So, if we assume that gNB1 is interested in collecting UE performance during a 5-second interval, the UE can move to RRC_INACTIVE, or to RRC_IDLE, or can be handed over to another cell at any time between the moment the collection starts and until expiration of the timer. The usefulness of the measured UE performance is not the same if the exit condition manifests itself after a relatively short time (e.g. 200ms) or after a relatively long time (e.g., 4800ms). The amount of time during which the UE performance was collected will make the reported UE Performance feedback more or less relevant. However, the gNB1 is unaware of the real collection time duration and it cannot judge how relevant the reported UE Performance metrics can be.

Observation 3: The relevance of the collected UE performance depends on the amount of time during which the UE performance was collected.

Therefore, we propose that, in the scope of AI/ML-based mobility concerning handover enhancements, RAN3 studies enhancements for reporting the duration for which UE performance was collected.

Proposal 2: As part of handover enhancements for AI/ML based mobility, study enhancements for reporting the duration for which UE performance was collected.

## AI/ML assistance for inter-CU LTM

As part of the study item on AI/ML for NG-RAN Phase 3, at the last RAN3 meeting, the primarily discussed Agenda Items (AI) were 12.2.1 (multi-hop trajectory) and 12.2.2 (intra-CU LTM), whereas for AI 12.2.3 (handover enhancements) only the above FFS was captured. Companies brought proposals on several topics to this AI, where the most sought-after enhancement was in the area of inter-gNB LTM.

However, discussions in RAN3 gravitated towards discussing AI/ML-assisted intra-gNB LTM first, which is also in scope of the current study, as some companies thought that this is a logical first step in introducing AI/ML assistance in LTM. RAN3 agreed to study the following (in AI 12.2.2):

AI/ML assisted candidate cell selection for LTM Handover Preparation

While we are in favour of studying AI/ML assistance in intra-gNB LTM, we believe, it is more time-efficient to leverage the commonalities between intra- and inter-gNB LTM, where, at a high-level, agreements impacting the F1 interface for intra-gNB LTM may be translatable to impacts on the Xn interface for inter-gNB LTM. Note that the above statement is purely in the context of AI/ML assistance information that may be signalled between the gNB-CU and the gNB-DU in case of intra-CU LTM, which may translate to similar messages or signalling extensions on the Xn interface between two gNBs in the case of inter-CU LTM.

Proposal 3: RAN3 to discuss AI/ML assisted candidate cell selection for intra-gNB LTM and inter-gNB LTM in parallel in the current SI and the following WI as part of Release 20.

During the RAN3#129-bis meeting, for the intra-CU LTM scenario, RAN3 agreed to the re-use of the already established framework for the location of AI/ML model training and model inference functions.

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

In addition to the above, the following FFS was also captured:

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

Our paper to AI 12.2.2 in R3-258393 discusses the above agreements/FFS in the context of intra-gNB LTM. In the present paper however, we make the case for the AI/ML model training/inference function location in the context of inter-gNB LTM.

First, it is important to note that the scope of the current study item, as defined in the SID is the following:

Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture …

The reference to existing NG-RAN interfaces and architecture points towards the agreed principles for AI/ML for NG-RAN as part of Release 18, where the principles of location of AI/ML models split and non-split architectures have been specified. In the above, there is no support for locating a model in the gNB-DU.

Furthermore, for AI/ML-based mobility, load balancing, and energy saving use-cases defined in Release-18, and the subsequent Enhancements for AI/ML for NG-RAN in Release-19 have been finalized with AI/ML model inference being located only at the gNB-CU. Any introduction of AI/ML inference at the gNB-DU will only introduce a further need for coordination between the two functions which have not been discussed in earlier releases and makes the solution more complex. We therefore propose:

Proposal 4: The gNB-CU can infer the candidate LTM cells for inter-CU LTM.

Proposal 5: RAN3 to agree to the pCR to TR 38.745 in Appendix A.

In the following figure and text description, we describe at a high-level AI/ML assisted candidate cell selection for inter-gNB LTM.

Figure 1: High-level overview of inter-gNB-CU LTM

Step 0: As a means to exchange measurements and predictions, e.g., UE trajectory, UE performance, node-level energy cost, measured and predicted cell-level metrics, the Data Collection Reporting and Resource Status Reporting procedures may be established between the different gNBs.

Step 1: A UE with no pre-existing LTM configuration connected to gNB1 may be configured to measure and report RRM measurements.

Step 2: gNB1, currently serving the UE, may decide to configure inter-gNB LTM for the UE depending on the received measurements reports from the UE, other known information about the UE, e.g., measured/predicted trajectory, and neighbour nodes’ status, e.g., load, energy.

Step 3: The serving gNB initiates LTM preparation towards the candidate gNBs. The candidate gNBs for LTM may be determined either purely based on the available measurements and reports at the serving gNB, or based on using AI/ML on the available measurements and reports to predict one or several candidate cells for LTM.

Step 4: The serving gNB consolidates the LTM configuration for the UE across the different candidate LTM nodes and signals the LTM configuration to the UE. This is followed by the UE performing early UL/DL synchronization towards the different candidates.

Step 5: The serving gNB may determine to trigger LTM execution to any of the configured LTM candidates depending on the received measurement reports from the UE. In the above example, the RAN determines to trigger an LTM handover to gNB4, which was determined as a candidate based on AI/ML assistance/predictions.

# Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: The current signaling for configuring and reporting UE feedback after cannot support subsequent LTM.

Observation 2: The current exit conditions defined for terminating the collection of UE related feedback are not optimal for LTM.

Proposal 1: As part of handover enhancements for AI/ML based mobility, study enhancements for configuring and reporting UE feedback in case of LTM.

Observation 3: The relevance of the collected UE performance depends on the amount of time during which the UE performance was collected.

Proposal 2: As part of handover enhancements for AI/ML based mobility, study enhancements for reporting the duration for which UE performance was collected.

Proposal 3: RAN3 to discuss AI/ML assisted candidate cell selection for intra-gNB LTM and inter-gNB LTM in parallel in the current SI and the following WI as part of Release 20.

Proposal 4: The gNB-CU can infer the candidate LTM cells for inter-CU LTM.

Proposal 5: RAN3 to agree to the pCR to TR 38.745 in Appendix A.

# Appendix A – pCR to TR 38.745 – Handover enhancement

4.3	Other handover enhancements

Editor’s Note: Identify other handover enhancements via AI/ML, e.g., inter-CU LTM

### 4.3.1	Use case description

### 4.3.1.1 Data Collection and reporting enhancements

Procedures for collection and reporting of data for AI/ML use-cases are defined in 8.4.13 and 8.4.13 in TS 38.423. These procedures, i.e., the Data Collection Reporting Initiation and Data Collection Reporting procedures, were specified without taking LTM into account. These procedures can be enhanced taking LTM into account as well as more general improvements applicable for both legacy and new use-cases.

Studying data collection and reporting enhancements aims at identifying:

- LTM-specific enhancements for data collection reporting initiation and data collection reporting

- Enhancements to the data collection procedures applicable for both legacy and new use-cases.

### 4.3.1.2 AI/ML assisted inter-CU LTM

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2]. Inter-CU LTM is specified in TS 38.401[3].

AI/ML can be used to optimise inter-CU LTM procedures, e.g., to enhance network and UE performance, optimise resource allocation, and reduce mobility failures. AI/ML assisted inter-CU LTM is studied by assuming inference at the gNB-CU only.

AI/ML assisted inter-CU LTM aims at the following objectives:

- Identifying the optimal set of LTM candidate cells,

- Reduction in LTM execution failures,

- Increased use of RACH-less LTM,

- Improved resource utilization at the source and LTM candidate cell

### 4.3.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

### 4.3.2.1	Location of AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting AI/ML assisted inter-CU LTM:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

### 4.3.2.2	Input to AI/ML assisted inter-CU LTM

Editor’s Note: To be updated

### 4.3.2.3	Output data of AI/ML assisted inter-CU LTM

Editor’s Note: To be updated

<<<<< End of changes >>>>>>