3GPP TSG-RAN WG3 #130																	R3-258439

Dallas, U.S.A., 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0492 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 corrections for AI/ML based CCO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, InterDigital, Fibercop |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A first change relates to when new CCO issue is predicted. In this case, a gNB-DU may indicate a future coverage modification to its controlling gNB-CU in two scenarios: 1) in response to received Predicted CCO Assistance Information, if the gNB-DU serves the corresponding predicted affected cells and beams; 2) in response to received Neighbour Future Coverage Modification if the gNB-DU does not serve the corresponding predicted affected cells and beams. The current specification contains two separate paragraphs describing the information received by the gNB-DU for scenario 1) and 2), and the text can be interpreted as describing a response behaviour for scenario 2, but not for scenario 1. A second change relates to when a predicted CCO issue is cancelled. In this case, the DU receives the indication that a predicted CCO issue is cancelled from CU, and then, if a previously generated future coverage modification has not been applied yet, the DU may respond to the CU and notify that the future coverage modification is cancelled. In the current specification, the text can be interpreted as if a DU may notify the CU that a previously generated future coverage state is not applied, without having been previously informed by the CU that the corresponding predicted CCO issue is cancelled. This behaviour is not aligned with the Rel-19 solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the case of a new predicted CCO issue, unify the two paragraphs describing the information a gNB-DU may receive, to clarify that the specification text describing the gNB-DU response applies to both scenario 1) and scenario 2). For the case of a predicted CCO issue being cancelled, clarify that the gNB-DU notifies that a future coverage modification is cancelled only if it has previously received a notification from CU that the corresponding predicted CCO issue is cancelled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The action of a gNB-DU receiving Predicted CCO Assistance Information when the gNB-DU serves the predicted affected cells and beam is not clear. A gNB-DU may notify the gNB-CU that a previously generated future coverage state is not applied, without having previously been informed by the CU that the corresponding predicted CCO issue is cancelled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>

### 7.11.3	Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

-	Energy Cost (EC),

-	UE performance feedback.

The collection of EC is configured through the Resource Status Reporting Initiation procedure while the reporting is performed through the Resource Status Reporting procedure. Packet delay measurement collection for UE Performance from gNB-DU is triggered through the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages and the packet delay measurements collected by the gNB-DU are reported from gNB-DU to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame, specified in TS 38.425 [33].

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of UE performance feedback from a gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the reporting is performed through the Data Collection Reporting procedure.

Signaling by a split gNB of the RLC Average UE UL/DL Throughput over the Xn interface is not supported in this version of the specifications.

At any given point in time, for a list of predicted affected cells and beams, there is only one predicted CCO issue generated by a gNB-CU.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled. In response, t

he gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>