3GPP TSG-RAN WG3 Meeting #130	R3-258446

Dallas, US, 17 – 21 November 2025

Agenda Item:	10.2.2

Source:	CATT

Title:	Discussion on 6G RAN function and Logical Architecture

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the Overall RAN architecture aspects, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze a potential 6G RAN Overall Architecture direction from RAN3 perspective, and subsequently, based on this direction, we further provide some specific proposals for architecture and interface design.

2  Discussion

2.1  Overall Architecture

The 6G RAN consists of the following logical nodes:

-	6G RAN nodes providing the 6G U-plane, C-plane and D-plane protocol terminations towards the UE;

The logical nodes in 6G RAN are interconnected with each other by means of the Xn+ interface.

For legacy service, the logical nodes in 6G RAN are connected to the 6GC NFs by means of the NG+ interface. The NG+interface supports a many-to-many relation between 6GC NFs and the logical nodes in 6G RAN.

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN within the Control Plane enables the 6GC NFs to access their services by means of a service-based interface N6gr.

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN are directly connected to the corresponding NFs in 6GC by means of the Ndp interface of D-plane.

The 6G RAN architecture is illustrated in following figure.

Figure 1: 6G RAN overall architecture

Proposal 1:  Capture the aforementioned architectural diagrams and relevant descriptions in the RAN3 TR.

2.1.1 Design Principle for Legacy Services

In 5G, as specified in 38.300,NG-RAN nodes connected with each other via Xn interface.They also connect to the 5GC through the NG interface, more specifically, to the AMF by means of the NG-C interface and to the UPF  by means of the NG-U interface.

For 6G, we anticipate that legacy services will not impose significant new architectural requirements beyond those of 5G. A related open issue,i.e.whether to introduce a new 6GC or reuse the 5GC,falls within SA2's scope. Therefore, we propose using the 5G RAN architecture as the baseline for 6G RAN design

Proposal 2: Use the 5G RAN overall architecture as the baseline for 6G RAN architecture design.

2.1.2 Design Principle for New Services

In addition to legacy services, 6G will also introduce new types of services with distinct characteristics. As analysed in [2], there is a requirement to support efficient data transfer between RAN nodes and the core network for new services e.g. AI, Sensing. In the current 5G architecture, however, although signalling can be transmitted between an NG-RAN node and the core network via NGAP, the control plane is not suitable for transmitting large volumes of data, as its primary design goal is reliability rather than efficiency. Similarly, the conventional user plane presents limitations for new service requirements for the following reason:

- At the CN side of NG interface, user plane data are always sourced from (in downlink) or destined to (in uplink) an external Data Network (DN), necessitating routing through the UPF/MB-UPF. And according to TS 38.300, the user plane must be between the NG-RAN node and the UPF. Both descriptions are not aligned with the needs of new services where the data source/consumer may be a Network Function (NF) within the 6G core. Transiting via the UPF in such cases appears redundant and inefficient.

- At the RAN side of NG interface, user plane data are always sourced from (in uplink) or destined to (in downlink) the UE This is not always aligned with the needs of new services where the data source/consumer may be the RAN itself, rather than the UE.

Therefore, to support efficient data transfer between NG-RAN node and CN, we propose exploring a new data transfer mechanism between RAN and CN, distinct from the current control and user plane mechanisms which may be named as data plane. Direct data transfer between RAN nodes and the corresponding function in CN could be thus supported.

Nevertheless, during the discussion of last meeting, there were some comments that the new data transfer mechanism could also be called as user plane but with some difference with conventional user plane. Although we prefer to introduce another plane to transfer data for new service, we are open for further discussion on the name itself as long as the mechanism could fulfil the requirement.

Proposal 3: To support data transfer for new services, it is proposed to explore a new data transfer mechanism between RAN and CN—for example, by introducing a new data plane between RAN nodes and corresponding core network function.

Regarding data transfer between RAN nodes, there is also a need to support efficient data exchange for new services such as AI. Although a control-plane-based solution was specified in 5G, the variety of use cases in other working groups and emerging 6G requirements call for a more efficient and future-proof mechanism for AI data exchange. Similar to RAN-CN data transfer, the current user plane between NG-RAN nodes is designed for UE-specific data forwarding, which may not be suitable for new service requirements. Therefore, a more flexible mechanism for AI data transfer, potentially similar to the RAN–CN data plane, could be explored

Proposal 4: To  support data transfer for new services between RAN nodes, it is proposed to explore a new data transfer mechanism, such as a new data plane between RAN nodes.

In addition, for the data plane, it is necessary to design a corresponding control signaling transmission mechanism. We propose adopting a service-oriented communication mechansim, mainly based on the following reasons:

- The SBI-based signaling architecture can support an efficient and concise information transmission method. Specifically, direct communication can be established between different network functions. This will undoubtedly simplify the complexity of the overall RAN design and reduce redundancy;

- The SBI-based signaling architecture can well support elastic scalability. It means that the specific function deployment will be purely practical deployment issues rather than architectural requirements, which further enhances the flexibility and adaptability of network functions;

- Since SBA is built on protocols (HTTP) and interactions (API-based) commonly used in cloud deployment, and adapts to the needs of operator, it has a wide range of application scenarios. Moreover, it has been well applied in 5GC, which can ensure that the expansion for new services will not affect the original network architecture;

- The application protocol of the control plane interface should be decoupled from the transport layer. i.e, unlike the legacy N2 interface having an association with SCTP protocol. By using an SBA-based interface, resource-based granularity can be achieved, where service operations and created resources can be addressed via URIs.

Based on the above discussion for control signaling transmission mechanism of data plane, we provide the following architecture proposals for 6G RAN.

Proposal 5:  Direct communication should be supported for control signaling of new services, so as to avoid complex and redundant transport paths.

Proposal 6: A service-oriented communication mechansim should be designed for control signaling transmission mechanism of the data plane.

Proposal 7: The control plane interface for new services should support HTTP service-based protocols, and be decoupled from the transport layer.

2.2  Consideration of 6G RAN function

Compared to the RAN-CN function split in the NG interface, we assume the legacy function split as NG is still applicable for 6G communication related functions, e.g. RRM, RB control, Mobility Control, Measurement, etc. But for the new services, e.g. AI/ML, Sensing, new service functions will be introduced in the 6GC, and corresponding functions will also be included in 6G RAN node. The function split between 6G RAN and 6GC is illustrated with the figure below:

Figure 1. Functional Split between 6G RAN and 6GC

Observation 1: RAN-CN function split as defined for 5G could be taken as baseline for 6G RAN-CN function split.

For the functions of a 6G RAN node, the specifications for an NG-RAN node in 38.300 could be used as a starting point as belowe.:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in both uplink and downlink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF+ at UE attachment when no routing to an AMF+ can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF+(s);

-	Routing of Control Plane information towards AMF+;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state; [FFS]

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity; [FFS]

-	Interworking between 6GR and NR; [FFS]

Since Dual Connectivity and Interworking between 6G-RAN and NR are expected to be discussed in later stages of 6G standardization, these two items can be marked as FFS.

For UEs in RRC-INACTIVE state, discussions are still ongoing in RAN2, so this item may also be marked as FFS

As for the remaining items, we believe they are applicable to 6G RAN nodes

Proposal 8: The functions defined for 5G NG-RAN should also be supported for 6G RAN nodes with RRC-Inactive, Dual Connectivity and Interworking between 6G-RAN and NR marked as FFS

The introduction of new services will necessitate corresponding functions in the 6G RAN node. It should also be noted that certain functions, such as positioning, are already supported in 5G RAN while are not captured in 38.300. We propose its inclusion in the specification as below:

-	 Functions for Sensing, e.g. Sensing related radio resource control, sensing TRP selection, sensing data collection, sensing data processing and report towards the Sensing Function in 6G CN.

-	Functions for AI/ML, e.g. data collection and data transmission between 6G RAN and the AI/ML related function in 6G CN.

-	Support of Positioning, including positioning resource allocation, positioning measurement, positioning report, between 6G RAN and Positioning related function in 6G CN.

Proposal 9: 6G RAN nodes should support functions for new services i.e.AI/ML and sensing, as well as positioning.

It’s assumed that the legacy functions of 6G CN are similar to the functions in 5GC, as hosted by AMF, SMF, UPF. While the functions of AI/ML related function(s), Sensing Function, Positioning related function (e.g. LMF+) in 6GC to be further discussed, coordination with SA2 is needed.

Proposal 10: Capture above function split between 6G RAN and 6GC in the RAN3 TR.

3	Conclusion

Base on the discussion above, we provide the following proposals:

Proposal 1:  Capture the aforementioned architectural diagrams and relevant descriptions in the RAN3 TR.

Proposal 2: Use the 5G RAN overall architecture as the baseline for 6G RAN architecture design.

Proposal 3: To support data transfer for new services, it is proposed to explore a new data transfer mechanism between RAN and CN—for example, by introducing a new data plane between RAN nodes and corresponding core network function.

Proposal 4: To  support data transfer for new services between RAN nodes, it is proposed to explore a new data transfer mechanism, such as a new data plane between RAN nodes.

Proposal 5:  Direct communication should be supported for control signaling of new services, so as to avoid complex and redundant transport paths.

Proposal 6: A service-oriented communication mechansim should be designed for control signaling transmission mechanism of the data plane.

Proposal 7: The control plane interface for new services should support HTTP service-based protocols, and be decoupled from the transport layer.

Observation 1: RAN-CN function split as defined for 5G could be taken as baseline for 6G RAN-CN function split.

Proposal 8: The functions defined for 5G NG-RAN should also be supported for 6G RAN nodes with RRC-Inactive, Dual Connectivity and Interworking between 6G-RAN and NR marked as FFS

Proposal 9: 6G RAN nodes should support functions for new services i.e.AI/ML and sensing, as well as positioning.

Proposal 10: Capture above function split between 6G RAN and 6GC in the RAN3 TR.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- R3-256615  Discussion on general principles and requirements for 6G RAN,  CATT

Annex1	Text Proposal

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

5.3  		Overall RAN architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

The 6G RAN consists of the following logical nodes:

-	6G RAN nodes providing the 6G U-plane, C-plane and D-plane protocol terminations towards the UE;

The logical nodes in 6G RAN are interconnected with each other by means of the Xn+ interface.

For legacy service, the logical nodes in 6G RAN are connected to the 6GC NFs by means of the NG+ interface. The NG+interface supports a many-to-many relation between 6GC NFs and the logical nodes in 6G RAN.

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN within the Control Plane enables the 6GC NFs to access their services by means of a service-based interface N6gr.[FFS]

For new service (e.g, AI, Sensing, etc), the logical nodes in 6G RAN are directly connected to the corresponding NFs in 6GC by means of the Ndp interface of D-plane.[FFS]

The 6G RAN architecture is illustrated in following figure.

Figure 1: 6G RAN overall architecture

5.3.2 	RAN Functions

Editor’s note: The aim of this section is to describe functions in RAN.

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in both uplink and downlink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF+ at UE attachment when no routing to an AMF+ can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF+(s);

-	Routing of Control Plane information towards AMF+;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state; [FFS]

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity; [FFS]

-	Interworking between 6GR and NR; [FFS]

-	Functions for Sensing, e.g. Sensing related radio resource control, sensing TRP selection, sensing data collection, sensing data processing and report towards the Sensing Function in 6G CN. [FFS for detail]

-	Functions for AI/ML, e.g. data collection and data transmission between 6G RAN and the AI/ML related function in 6G CN. [FFS for detail]

-	Support of Positioning, including positioning resource allocation, positioning measurement, positioning report, between 6G RAN and Positioning related function in 6G CN. [FFS for detail]

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>