3GPP RAN WG3 Meeting #130	R3-258450

Dallas, USA, 17th – 21st November 2025

Source:	CATT

Title:	Consideration on internal interfaces within 6G RAN

Agenda Item:	10.4

Document for:	Discussion and decision

- Introduction

This document discusses a few aspects on high-layer internal split of 6G RAN nodes and the principle and general function of inter-6G-RAN-node interface.

- Discussion

  - Principles and general functions of inter-6G-RAN-node interface

Last meeting R3-257234 was agreed, capturing some principles and general functions of the interface between 6G-RAN-node and the 6G CN. We believe a similar TP can be agreed for inter-6G-RAN-node interface as well.

The principle part of R3-257234 is as following:

The general principles for the specification of the 6G RAN-CN interface are as follows: - the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN; - the 6G RAN-CN interface supports control plane and user plane separation; - the 6G RAN-CN interface supports future enhancements; - the 6G RAN-CN interface supports all possible RAN deployment scenarios; - the 6G RAN-CN interface supports RAN sharing between multiple operators; - the 6G RAN-CN interface supports the operation of network slicing; - the 6G RAN-CN interface supports enhanced service awareness in RAN; - the 6G RAN-CN control plane interface supports reliable signalling transmission; - the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

All the abovementioned bullets apply for inter-6G-RAN-node interface, except the last bullet. We propose copying these bullets into the section for inter-6G-RAN-node interface.

Besides the principles above, the following principles for 5G NG interface as specified in TS 38.410 have not been agreed in R3-257234:

-	the NG interface is open;

-	from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

-	the NG interface separates Radio Network Layer and Transport Network Layer;

-	the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

Some bullets were ever proposed for the interface between RAN and CN, but it was rejected due to the possibility of SBI. But for inter-6G-RAN-node interface, most companies agree that it should be a point-to-point interface. So we propose agreeing all these 4 bullets for 6G.

Proposal 1: Copy the principles for 6G RAN-CN interface agreed last meeting into inter-6G-RAN-node interface with minor adaption, except:
-	Remove the bullet “the 6G RAN-CN interface is designed with a clear functional split between RAN and CN”;
-	Add all the bullet discussed for the RAN-CN interface but not agreed last meeting.

For general functions, we propose listing three outmost bullets captured in TR 38.801:

The FFS-C interface supports the following functions: - FFS interface management and error handling function to manage the FFS-C interface; - UE mobility management: function to manage the UE mobility between 6G RAN nodes. The FFS-U interface supports the following functions: - Data forwarding.

The phrase “connected mode” is removed from the name of the bullet “UE mobility management”, due to it is possible to support other RRC state which requires UE context maintainence in RAN, e.g. RRC_INACTIVE state.

The bullet “UE context retrieval” is removed, as it is part of “UE mobility management” (note that it also applies for RRC_CONNECTED for RRC reestablishment).

The bullet “Dual connectivity” “Interference coordination” “Self-optimization” and “Flow control” are all removed since they are unlikely to be supported in the first 6G release.

Proposal 2: Capture the following texts as the functions for inter-6G-RAN-node interface:
The FFS-C interface supports the following functions:
-	FFS interface management and error handling function to manage the FFS-C interface;
-	UE mobility management: function to manage the UE mobility between 6G RAN nodes.
The FFS-U interface supports the following functions:
-	Data forwarding.

  - Benefit of internal split

In the starting phase of 5G, some benefit of standardised internal split of a RAN node was captured in TR 38.801 and TR 38.806, e.g.:

Some of the benefits of an architecture with the deployment flexibility to split and move NR functions between central and distributed units are below: - Flexible HW implementations allows scalable cost effective solutions - A split architecture (between central and distributed units) allows for coordination for performance features, load management, real-time performance optimization, and enables NFV/SDN - Configurable functional splits enables adaptation to various use cases, such as variable latency on transport

Some benefits for the separation of CU-CP and CU-UP common to the analysed scenarios were identified: - Flexibility to operate and manage complex networks, supporting different network topologies, resources, and new service requirements; - Alignment with SDN concept that would result in a functional decomposition of the radio access, based on a partial de-coupled architecture, between user and control plane entities and on network abstractions; - For functions purely handling with CP or UP processes, independent scaling and realization for control and user plane functions operation; - Support of multi-vendor interoperability (e.g., CU-CP and CU-UP could be provided by different vendors). - The gNB deployment with separate CU-CP and CU-UP provides the possibility of optimizing the location of different RAN functions based on the scenario and desired performance. For example, the CU-CP could be placed in a location close to the DU to provide short latency for the critical CP procedures. The CU-UP could be centralized in a regional or national data center, thus favouring cloud implementation. An additional CU-UP could be also placed closer to the DU to provide a local termination point for [...] URLLC traffic. - Support of radio resource isolation and improving resource utilization for network slicing. A slicing instance may cover a geographic area of several ten to several hundred of gNBs. Central RRM may provide slice-level isolation as well as improve resource utilization.

We believe that most of these benefits are still valid for 6G.

In addition, internal split of RAN nodes may also be beneficial for some new services like AI/ML, e.g. centralised training and/or inference may reduce the cases of redundant calculation, as training/inference inputs includes not only data related to served cells but also data related to neighbour cells.

Proposal 3: Acknowledge that it is beneficial to standardise CU/DU-split-like and CU-CP/UP-split-like internal split of RAN node.

NTN is an important deployment scenario planned to be supported from 6G Day 1. Capacity of satellites is usually costly, especially for the vast majority of satellites in a constellation used to cover the globe, so it is beneficial economically to offload as much as function down to the ground or some large, centralised satellite. In addition, the moving native of non-GEO satellites is also troublesome for any logical interface between the satellite and the ground. Performance and uer experience, however, is also an important consideration. Offloading too much function sensitive to round-trip delay, e.g. function in the physical layer, will deteriorate performance and user experience as well.

Internal split of RAN node is thus considerable for NTN deployment. Offloading only the CU part down to the ground or to large, centralised satellites can reduce the cost of covering satellites and facilitate the management of interface between RAN and CN, and maybe consequently, facilitate the use of RRC_INACTIVE state. Offloading only the CU-UP part can also reduce the cost of covering satellites, since the most capacity-comsuming part in CU is the security function in the PDCP layer.

Proposal 4: Acknowledge that NTN may also benefit from high-layer split of RAN nodes.

One major drawback of 5G-era CU/DU split is an extra round-trip delay between the CU and the DU in many RRC procedures. It occurs almost everywhere, from RRC setup to CU-initiated RRC reconfiguration.

Figure 1: An extra F1-C round-trip delay (Step 1&2) in CU-initiated RRC reconfiguration procedure

Enabling DUs to send and receive Control-Plane messages can virtually eliminate such drawback. Nevertheless, there may be some solutions to such enabling, all of them seeminly have impact on RAN2. Coordination with RAN2 may be necessary.

Proposal 5: Confirm that enabling DUs to send and receive Control-Plane messages will mitigate the extra F1-like round-trip delay of many RRC procedures. Coordination with RAN2 may be needed.

  - Splitting point in protocol stack

Change of the splitting point between the CU and the DU is worthy to consider. It is possible to move the ARQ part of RLC toward the CU, or to move the retransmission function in PDCP toward the DU. However, the benefit seen in RAN3 scope is unclear, e.g. moving the ARQ part of RLC toward the CU will make ARQ retransmission more flexible in Dual Connectivity scenarios, but whether Dual Connectivity itself will be supported in 6G is still very doubtful. The major motivation maybe in RAN2, e.g. if RAN2 decide to merge the PDCP retransmission function with the RLC ARQ part, RAN3 will have to consider whether to put the merged retransmission function within the CU or within the DU. Therefore, we prefer waiting progress of RAN2 UP discussion before deciding the splitting point.

Proposal 6: If high-layer split of RAN nodes is to be specified, wait progress of RAN2 UP discussion before deciding the splitting point in protocol stack.

  - Security for internal split

There are some scenarios which (may) require CU-UPs and/or DUs to perform security process, e.g., normal UP security protection at CU-UPs, sending/receiving CP messages at DUs (RRC-like, MAC-CE-like, etc), and sending/receiving AI/ML-related information at DUs.

In 5G, SA3 instructed that entities in different security domains shall not use the same security key. Due to UE mobility, dual connectivity or some other reason, one UE may be sequentially or simultaneously connected to multiple CU-UPs and/or DUs. We are not sure whether all CU-UPs and/or DUs of a given RAN node are within the same security domain one another respectively. If the answer is no, whether there should be some enhancement on security, or the related feature (e.g. sending/receiving CP messages at DUs) should not be used for the cases that CU-UPs and/or DUs of a given RAN node are not within the same security domain. Coordination with SA3 may be necessary.

Proposal 7: If high-layer split of RAN nodes is to be specified, discuss whether all CU-UPs (if applicable) and/or DUs of a given RAN node are within the same security domain one another respectively, and if no, how to handle that case (either enhancing the security architecture, or forbidding them from performing security process). Coordination with SA3 may be needed.

- Conclusion

Proposal 1: Copy the principles for 6G RAN-CN interface agreed last meeting into inter-6G-RAN-node interface with minor adaption, except:
-	Remove the bullet “the 6G RAN-CN interface is designed with a clear functional split between RAN and CN”;
-	Add all the bullet discussed for the RAN-CN interface but not agreed last meeting.

Proposal 2: Capture the following texts as the functions for inter-6G-RAN-node interface:
The FFS-C interface supports the following functions:
-	FFS interface management and error handling function to manage the FFS-C interface;
-	UE mobility management: function to manage the UE mobility between 6G RAN nodes.
The FFS-U interface supports the following functions:
-	Data forwarding.

Proposal 3: Acknowledge that it is beneficial to standardise CU/DU-split-like and CU-CP/UP-split-like internal split of RAN node.

Proposal 4: Acknowledge that NTN may also benefit from high-layer split of RAN nodes.

Proposal 5: Confirm that enabling DUs to send and receive Control-Plane messages will mitigate the extra F1-like round-trip delay of many RRC procedures. Coordination with RAN2 may be needed.

Proposal 6: If high-layer split of RAN nodes is to be specified, wait progress of RAN2 UP discussion before deciding the splitting point in protocol stack.

Proposal 7: If high-layer split of RAN nodes is to be specified, discuss whether all CU-UPs (if applicable) and/or DUs of a given RAN node are within the same security domain one another respectively, and if no, how to handle that case (either enhancing the security architecture, or forbidding them from performing security process). Coordination with SA3 may be needed.

- pCR for TR 38.760-3

## 6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone TSG#112 (June/2026) to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

6.2.1	Interface between 6G RAN nodes

#### 6.2.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the interface between 6G RAN nodes supports the exchange of signalling information between 6G RAN nodes;

-	the interface between 6G RAN nodes supports control plane and user plane separation;

-	the interface between 6G RAN nodes supports future enhancements;

-	the interface between 6G RAN nodes supports all possible RAN deployment scenarios;

-	the interface between 6G RAN nodes supports RAN sharing between multiple operators;

-	the interface between 6G RAN nodes supports the operation of network slicing;

-	the interface between 6G RAN nodes supports enhanced service awareness in RAN;

-	the control plane interface between 6G RAN nodes supports reliable signalling transmission;

-	the NG interface is open;

-	from a logical standpoint, the interface between 6G RAN nodes is a point-to-point interface between 6G RAN nodes. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the two nodes;

-	the interface between 6G RAN nodes separates Radio Network Layer and Transport Network Layer;

-	the Application Protocol of the interface between 6G RAN nodes supports modular procedures design and uses a syntax allowing optimized encoding/decoding efficiency.

#### 6.2.1.2	Functions

The FFS-C interface supports the following functions:

-	FFS interface management and error handling function to manage the FFS-C interface;

-	UE mobility management: function to manage the UE mobility between 6G RAN nodes.

The FFS-U interface supports the following functions:

-	Data forwarding.