3GPP RAN WG3 Meeting #120	R3-258451

Dallas, US, 17 – 21 November 2025

Source:	CATT, Huawei

Title:	Discussion on UL data forwarding

Agenda Item:	9.2.14

Document for:	Discussion and decision

- Introduction

In 38.415, on data forwarding, there are statements as below:

In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall also be used to send control information elements related to the PDU Session from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes

Based on the above statement, for both downlink data forwarding and uplink data forwarding, DL PDU Session Information Procedure is used. This contribution analyzes the drawbacks of this method and makes a proposal accordingly

- Discussion

The initial purpose of UL PDU Session Information procedure and DL PDU Session Information procedure is stated as below:

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN.

The purpose of the Transfer of UL PDU Session Information procedure is to send control information elements related to the PDU Session from NG-RAN to UPF.

Clearly, the DL PDU Session Information procedure carries downlink control information over the NG interface, while the UL PDU Session Information procedure carries uplink control information from the NG-RAN to the UPF. In the context of data forwarding, UL data forwarding refers to forwarding uplink packets while DL data forwarding refers to forwarding downlink packets.

Regarding the statement on UL/DL data forwarding,it was initially introduced in [1] as below.Based on the summary of changes which originally only mentioned packets received from the Anchor UPF, the CR appears to have been intended to clarify that the PDU Type 0 frame is used for DL data forwarding from the source NG-RAN node to the forwarding UPF. However, in the final version of the CR, this principle was extended to apply to both DL and UL data forwarding.

Following the above CR's approach, i.e. applying the DL PDU Session Information procedure to both UL and DL packages, it works for DL data forwarding, however, it introduces several issues for UL data forwarding:

- The source node would need to assign dummy values to mandatory IEs in the DL PDU Session Information when forwarding UL packets to the target node.

- The target node needs to replace the control information received and set corresponding values for UL PDU Session Information when delivering the forwarded UL package to UPF.

- The source node cannot forward QoS monitoring results included in the UL PDU Session Information procedure to the target node. This information may either be discarded or sent to the UPF even if UL packet delivery to the UPF has been stopped.

In contrast, if the UL PDU Session Information procedure is used for UL data forwarding, no special handling is required in both source side and target side. This approach is also more consistent with the original design intention of the UL/DL PDU Session Information Procedure.

Observation: Using the DL PDU Session Information procedure for UL data forwarding not only mandates  special handling of control information in both source and target nodes but may also lead to QoS monitoring information loss as well. In contrast, neither of the issues arise when UL PDU Session Information procedure is used for UL data forwarding.

So, a more reasonable option is to forward DL and UL packages with PDU Type Frame 0 and 1 separately as below:

In the case of downlink data forwarding the DL PDU Session Information procedure shall also be used to send control information elements related to the PDU Session from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes

In the case of uplink data forwarding the UL PDU Session Information procedure shall also be used to send control information elements related to the PDU Session from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes

We propose RAN3 to discuss whether the alternative above is more suitable for UL data forwarding.

Proposal: Discuss whether it is more suitable to use UL PDU Session Information procedure for UL data forwarding.

- Conclusion

Based on the discussion in section 2, we have the following observation and proposal

Observation: Using the DL PDU Session Information procedure for UL data forwarding not only mandates  special handling of control information in both source and target nodes but may also lead to QoS monitoring information loss as well. In contrast, neither of the issues arise when UL PDU Session Information procedure is used for UL data forwarding.

Proposal: Discuss whether it is more suitable to use UL PDU Session Information procedure for UL data forwarding.

- Reference

[1] R3-212754  Correction of PDU Type Frame  Nokia, Nokia Shanghai Bell, Orange, Ericsson