3GPP TSG-RAN WG3 #130                                                    R3-258455

Dallas, US, 17th – 21st Nov, 2025

Agenda Item:	12.2.1

Source:	CATT

Title:	Support of multi-hop UE trajectory prediction/feedback

Document for:	Discussion

Introduction

In last RAN3 meeting, we discussed the relevant issues on support of Multi-hop UE trajectory across gNBs. According to the discussion, some agreements and FFS are achieved [2], as below:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order. The following solutions are considered for supporting multi-hop UE trajectory: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. Focus on cell-based granularity for measured and predicted multi-hop UE trajectory. Study multi-hop UE trajectory for L3 HO. The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network. The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop. Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message? Configuration of measurement feedback: - Data collection initiate procedure How to transfer measurement feedback: • Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1) • Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1) • Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1) • Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

In this contribution, we will continue discuss the potential issues on Multi-hop UE trajectory, our views will be provided accordingly.

Discussion

Multi-hop UE trajectory can be categorized into three major aspects:

- UE trajectory prediction

- Measurement configuration information

- Measurement feedback

Below, we will analyse and discuss these three key aspects in detail.

  - UE trajectory prediction

How to transfer the multi-hop UE trajectory prediction

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

The last RAN3 meeting discussed how to transfer the multi-hop UE trajectory prediction, but no conclusion was reached. In our opinion, the trajectory prediction Information for Multi-hop is a per-UE measurement. Thus, predicted trajectory can only be contained in a UE associated message, i.e. the Handover Request message.

Proposal 1：It is proposed that the multi-hop UE trajectory prediction should be transferred via UE associated message, i.e. Handover Request message.

The UE trajectory prediction Information transferred by intermediate nodes

In last RAN3 meeting, the content of the UE trajectory prediction Information was discussed. For a UE, most companies agreed that only one corresponding trajectory prediction shall be transmitted, rather than a list of UE trajectory prediction. This is reasonable because predictions from remote nodes have no value for the current node’s HO decision-making.

Proposal 2: For a UE, only one corresponding trajectory prediction shall be transmitted during Handover Request message.

As for how the intermediate nodes handle the UE trajectory prediction, it shall be totally up to NG-RAN node implementation and not be specified, which we consider to be the consensus. And for intermediate node enabling trajectory predicting, the trajectory prediction information from the current node itself may override the previous one. For intermediate node not enabling trajectory predicting, the intermediate may just transfer the received UE trajectory prediction.

Proposal 3: The UE trajectory prediction information transferred by intermediate nodes depends on the implementation of the intermediate nodes.

  - Measurement configuration information

How to transfer the measurement configuration information

Configuration of measurement feedback:

- Data collection initiate procedure

In Rel-18/19, we introduce a non-UE association message for measurement configuration, i.e., the data collection reporting initiation procedure. The requesting node configures data collection in per node manner and triggers data collection in the per-UE Handover Request message. The feedback is provided in the per-node Data Collection Report message as a list of UE-related items. Compared with UE-associated configuration and reporting, this mechanism enables UE specific report with an acceptable signalling overload, i.e. it avoids repeating the same reporting configuration once per UE.

However, in multiple-hop scenario, the abovementioned benefits no longer exist. The configuration for trajectory collection, i.e. the UE Trajectory Collection Configuration IE, only includes Collection Time Duration for UE Trajectory and Number of Visited Cells.

Observation 1: For UE trajectory collection, the measurement configuration only includes the Collection Time Duration and Number of Visited Cells.

Assuming there are some UEs handed over from gNB1 to gNB2 and then to gNB3. The length of time stayed in gNB2 varies among UEs, while gNB1 expects feedback after a uniformed time after each UE is handed over to gNB2 (gNB1 may deletes its UE context when the time expires). So the remaining collection time sent to gNB3 should also vary among UEs according to the time stayed in gNB2. Similarly, the remaining number of visited cells sent to gNB3 should vary among UEs according to the number of visited cells in gNB2. Therefore, for multi-hop UE trajectory feedback, both the collection time duration and the number of visited cells sent in hops except the first one should be UE-specific and updated according to the information known in the source node of the considered hop.

Observation 2: For multi-hop UE trajectory feedback, both the collection time duration and the number of visited cells sent in hops except the first one should be UE-specific and updated according to the information known in the source node of the considered hop.

This issue is valid for both hop-by-hop configuration and parallel configuration.

Assuming UE1 and UE2 are handed over from gNB1 to gNB2 and then to gNB3, with only gNB1 serving as the collection node.

Considering the measurement is configured hop by hop:

Figure1 shows serial configuration procedures using non-UE-associated messages.

Figure1: Serial configuration procedures using non-UE-associated messages

Due the different time of UE1 and UE2 staying in gNB2, the remaining collection time of UE1 and UE2 is different. Similarly, the remaining number of visited cells of UE1 and UE2 is different.

If the configuration information sent in the Data Collection Reporting Initiation procedure is per-node, the Handover Request message should include the updated UE-specific configuration information, specifically the remaining time and remaining number of cells, replacing both IEs in the Data Collection Reporting Initiation procedure for the corresponding UE. That is to say, the Data Collection Reporting Initiation procedure is useless.

Otherwise, if the configuration information sent in the Data Collection Reporting Initiation procedure represents the updated UE-specific configuration, the Data Collection Reporting Initiation procedure should be performed once for each UE handed over form gNB2 to gNB3. As illustrated in the figure above, the configuration information of UE1 and UE2 is sent to gNB3 via Data Collection Reporting Initiation procedure, respectively. This introduces significant signalling overhead and loses the benefit of using non-UE-associated messages. Moreover, since gNB3 receives multiple data collection request messages from gNB2, it cannot distinguish which UE the carried configuration information corresponds to.

Observation 3: Sending the UE-specific configuration information in the Data Collection Reporting Initiation procedure introduces significant signalling overhead and loses the benefit of using non-UE-associated messages, and may cause confusion if multiple UEs are handed over simultaneously.

Considering the measurement is configured parallel:

Figure2 shows parallel configuration procedures.

Figure2: Parallel configuration procedures using non-UE-associated messages

As shown in Figure2, gNB1 configures the gNB2 and gNB3 via the data collection initiate procedure. Although gNB3 received the configuration from gNB1 in advance, since UEs stay at gNB2 for a certain period, the measurement configuration information still needs to be updated (e.g., via Handover Request message to transferring the updated information). For parallel configuration, if the configuration information is per node, the Data Collection Reporting Initiation procedure is meaningless. Therefore, for both serial and parallel configuration using non-UE messages, if the configuration information is node-specific, it still needs to be updated via Handover Request message.

Observation 4: For both serial and parallel configuration using non-UE-messages, if the configuration information is node-specific, it still needs to be updated per UE via Handover Request message.

Since anyway the measurement configuration information needs to be updated via UE-associated message, it seems no value to make configuration via non-UE-associated message. It is more suitable to deliver the updated configuration information via UE-associated messages. In conclusion, it is more appropriate for intermediate nodes to deliver the measurement configuration information via UE-associated messages i.e. Handover Request message.

Proposal 4: It is more appropriate for intermediate nodes to deliver the measurement configuration information via UE-associated messages i.e. Handover Request message.

  - Measurement feedback

In last RAN3 meeting, following FFS is discussed:

How to transfer measurement feedback:

•	Option1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

•	Option2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

•	Option3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

• Option4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Based on the agreement of the last meeting, The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network, so option 4 is excluded. In our view, regardless of whether the target node receives the UE trajectory prediction information, the target node to which the UE moves is required to provide UE trajectory feedback to the collecting node. The delivery of predicted UE trajectory information is not bound to the feedback of UE trajectory.

There are gNB implementation impacts if serial feedback is adopted for multi-hop UE trajectory feedback. UE context has to be kept in all of the visited gNBs in order to collect necessary UE trajectory feedback until the timer for collection time duration expires.

Observation 5: For serial feedback UE context has to be kept by all of the gNBs that UE connected to during handover in order to collect necessary UE trajectory feedback.

Once one of the gNBs fails to send UE trajectory feedback, the measured UE trajectory collected after the failed gNB would be missing.

Observation 6: The drawback of serial mode is that once one of the gNB in the feedback chain fails to send UE trajectory feedback, all of UE trajectory feedback collected after the failed gNB would be missing.

For parallel feedback mode, the above issues on keeping UE contexts may be solved. For parallel UE trajectory feedback, one issue to be discussed is how to identify UE at the initial node, e.g., gNB1 configured data collection to gNB2 and gNB3, when UE1 HO to gNB2 and further HO to gNB3, and the gNB3 send feedback to gNB1 directly, how to enable the gNB1 identify the feedback is from UE1.This issue can be solve by signalling enhancement, solutions FFS.

Observation 7: For parallel feedback mode, the issue to be discussed is how to identify UE at the initial node when it receives the UE feedback.

According to above observations, for serial mode solution, observation 5 and 6 are insuperable problems. While for parallel mode solution, these problems could be solved by enhancing XN interface message. Since UE trajectory feedback is crucial for mobility enhancement, reliable feedback should be considered. Therefore, parallel mode solution is slightly preferred.

Proposal 5: Discuss whether Hop-by-hop feedback or parallel feedback should be adopted for multi-hop scenario.

Conclusions

UE trajectory prediction

Proposal 1：It is proposed that the multi-hop UE trajectory prediction should be transferred via UE associated message, i.e. Handover Request message.

Proposal 2: For a UE, only one corresponding trajectory prediction shall be transmitted during Handover Request message.

Proposal 3: The UE trajectory prediction information transferred by intermediate nodes depends on the implementation of the intermediate nodes.

Measurement configuration information

Observation 1: For UE trajectory collection, the measurement configuration only includes the Collection Time Duration and Number of Visited Cells.

Observation 2: For multi-hop UE trajectory feedback, both the collection time duration and the number of visited cells sent in hops except the first one should be UE-specific and updated according to the information known in the source node of the considered hop.

Observation 3: Sending the UE-specific configuration information in the Data Collection Reporting Initiation procedure introduces significant signalling overhead and loses the benefit of using non-UE-associated messages, and may cause confusion if multiple UEs are handed over simultaneously.

Observation 4: For both serial and parallel configuration using non-UE-messages, if the configuration information is node-specific, it still needs to be updated per UE via Handover Request message.

Proposal 4: It is more appropriate for intermediate nodes to deliver the measurement configuration information via UE-associated messages i.e. Handover Request message.

Measurement feedback

Observation 5: For serial feedback UE context has to be kept by all of the gNBs that UE connected to during handover in order to collect necessary UE trajectory feedback.

Observation 6: The drawback of serial mode is that once one of the gNB in the feedback chain fails to send UE trajectory feedback, all of UE trajectory feedback collected after the failed gNB would be missing.

Observation 7: For parallel feedback mode, the issue to be discussed is how to identify UE at the initial node when it receives the UE feedback.

Proposal 5: Discuss whether Hop-by-hop feedback or parallel feedback should be adopted for multi-hop scenario.

The corresponding TP for TR 38.745 is provided on appendix 5.

Reference

- RP-252867 “Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” 3GPP TSG RAN Meeting #109

- RAN3_129bis_agenda_20251017_EOM2

- R3-257329 (TP to TR 38.745) Consideration on Multiple-hop UE trajectory

- TP for TR 38.745

<<<<<<<<<<<<<<<<<<<< First Changes >>>>>>>>>>>>>>>>>>>>

- Scope

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, AI/ML based intra-CU LTM, and other handover enhancements.

The present document provides the description and investigation of new AI/ML based use cases, i.e., multi-hop UE trajectory, AI/ML based intra-CU LTM, and other handover enhancements.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

For Multi-hop predicted UE trajectory across gNBs , only one corresponding trajectory prediction shall be transmitted for a UE.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Impact on interface

The following solutions are considered for supporting multi-hop UE trajectory:

-	The multi-hop UE trajectory prediction should be transferred via UE associated message, i.e. Handover Request message.

-	Transfer the measurement configuration information using UE associated message, i.e. Handover Request message.

Following is the signaling example to transfer the measurement configuration information.

Figure. UE Trajectory Collection Configuration in UE associated message

Step 1: UE connects to gNB1;

Step 2: Data Collection Reporting Initiation procedure is used to configure UE trajectory feedback.

Step 3: UE is handed over to gNB2 and UE trajectory prediction is transferred to gNB2.

Step 4: UE connects to gNB2, and gNB1 keeps UE context to collect UE trajectory feedback.

Step 5: gNB 2sends Handover Request message, including UE Trajectory Collection Configuration.

Step 6: UE connects to gNB3.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>