3GPP TSG-RAN WG3 Meeting #130	R3-258476

# Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1646 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on procedural text for reporting of predicted radio resource status per slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19 RAN3 introduced the predicted radio resource status per slice and predicted slice available capacity as information that may be used by the AI/ML model in the context of AI/ML-based Network Slicing. The predicted radio resource status per slice is included in the 1st bit ‘predicted radio resource status’ of the Report Characteristics for Data Collection IE introduced in Rel-18, while a new bit (10th) has been introduced in Rel-19 for the ‘predicted slice available capacity’. And a new IE Slice To Report List for Data Collection has been introduced in the DATA COLLECTION REQUEST message which needs to be included when ‘predicted radio resource status per slice’ and/or ‘predicted slice available capacity’ are requested to be reported. Based on that: - If bit 10 is set to ‘1’, this mandates the presence of the Slice To Report List for Data Collection IE; - If bit 1 is set to ‘1’ and the Slice To Report List for Data Collection IE is included, this implies that both Rel-18 ‘predicted radio resource status’ and Rel-19 ‘predicted radio resource status per slice’ are reported. To better address case b) above, it is proposed to enhance the receiver behavior’s description in the “Interaction with the Data Collection Reporting procedure” section of the Data Collection Reporting Initiation procedure, to make it clear that, when the Slice To Report List for Data Collection IE is present in the DATA COLLECTION REQUEST message, the Rel-19 ‘predicted radio resource status per slice’ is reported along with the Rel-18 ‘predicted radio resource status’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that, when the DATA COLLECTION REQUEST message includes the 1st bit ‘predicted radio resource status’ of the Report Characteristics for Data Collection IE and the Slice To Report List for Data Collection IE, the reporting node will provide both the ‘predicted radio resource status per slice’ and the ‘predicted radio resource status’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

*** unmodified text omitted ***

The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, also include the Slice Radio Resource Status Item IE.

*** unmodified text omitted ***

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>