3GPP TSG-RAN WG3 #130																	R3-258495

Dallas, US, 17 - 21 November 2025

Agenda Item:	9.2.14

Source: Sony, Ericsson, vivo, Jio Platforms, LG Electronics

Title:	LTM Resource consumption for the target cells

Document for:	Discussion and Decision

# Introduction

In Rel-18, LTM enhancement for intra-CU was specified which provided some mobility benefits of reducing HO latency and interruption time. However, there was a restriction that only mobility between cells of the same gNB (intra-CU) were supported in Rel-18 as shown on Figure 1.

In Rel-19, the benefits of Rel-18 LTM have been extended to support inter-CU scenario [2], i.e., cells of different gNBs as shown on Figure 2.

In this contribution, we discuss how to reduce resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios.

## 2. LTM Resource consumption for the target cells

In Rel-19 LTM, when gNB receives L3 measurement report which identifies the neighbouring candidate cells, the source gNB exchanges the neighbouring candidate cells with the LTM configurations and subsequently provides LTM RRC reconfiguration message to the UE. The configuration, for example, contains Configured Grant (CG) Resources of all the candidate cells. Figure 3 shows all the steps of the LTM procedures.

Figure 3. The overall procedure for inter-gNB LTM (Figure 9.2.3.5.2-2. Signalling procedure for LTM) [2]

As can be seen from the above Figure 3, configured grant (CG) resources for the target candidate cells are allocated in the RRC reconfiguration message. In this process, it is required that multiple CG resources are allocated for multiple candidate cells, although these resources are not used at the end, except one resource belonging to the chosen target cell. Hence, the issue is how to minimize the resource consumption for both intra-CU and inter-CU deployment scenarios.

Basically, in the current LTM procedure as shown on Figure 3 above, the CG resources for the target cell are allocated in the RRC reconfiguration message (in Step 8). In step 14, a UE receives the LTM Cell Switch Command to the target cell, and in step 16, the UE applies CG resources to transmit the first message (i.e., RRCReconfigurationComplete message) to indicate its arrival at the target cell.

In order to minimize the CG resource consumption, when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM Cell Switch Command to the UE for a target cell (for example target cell index 1), the CG resources can be activated only for that target cell (index 1). This means that activation message should be sent from the serving cell to the target cell only.

Observation 1: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

As the activation message is sent from the serving cell to the target cell, some signalling impact is expected in RAN3, but we think implicit signalling would be enough by using existing message of CELL SWITCH NOTIFICATION in step 15 of Figure 1, that is: when target cell receives this message, it should activate the CG resources of that target cell only.

Proposal 1: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Based on proposal 1, it should be noted that in case of Inter-CU scenario, there could be noticeable delay for the target cell to activate the CG resources, in that case, an implementation can handle this issue whereby the serving cell slightly delays sending LTM CSC MAC CE to the UE as latency is better than interruption [3]. This may need slight changes to the current order of step 14 and step 15. We think step 15 should come earlier than step 14 by swapping them, or the order could be up to network implementation, and a note can be added in the spec.

Proposal 2: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

## Conclusions

In this contribution, we have discussed how to reduce the configured grant resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios. We have the following observation and proposals:

Observation 1: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

Proposal 1: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Proposal 2: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

Proposal 3: Agree accompanying CRs for Rel-18 (38.401) and Rel-19 (38.300 and 38.401) [4-6].

## References

[1] RP-234036, “New WID: NR mobility enhancements Phase 4,” RAN#102.

[2] R2-2506225, “Introduction of NR mobility enhancements Phase 4 in TS 38.300”, CR Rel-19 TS 38.300.

[3] R2- 2408259, “Further discussion on Inter-CU LTM”, RAN2 Meeting #127bis, MediaTek.

[4] R3-258496, “Correction for Cell Switch execution”, SONY, Ericsson, vivo, Jio Platforms, LG Electronics, 38.300 Rel-19.

[5] R3-25xxxx, “Correction for Cell Switch execution”, Ericsson, Sony, Jio Platforms, LG Electronics, vivo, 38.401 Rel-18.

[6] R3-25xxxx, “Correction for Cell Switch execution”, Ericsson, Sony, Jio Platforms, LG Electronics, vivo, 38.401 Rel-19.