3GPP TSG RAN WG3 Meeting #130 	R3-258550

Dallas, US, 17th – 21st November, 2025

Agenda Item:	10.4

Source:	BT

Title:	Considerations for 6G Higher Layer Split Architecture

Document for:	Discussion

# 1	Introduction

During the last RAN3#129-bis meeting [1], the Higher Layer Split (HLS) topic was proposed to further study:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

This paper aims to identify some of the considerations associated with the adoption of a HLS in the context of 5G and future 6G deployments, summarising how HLS can address existing pain points while outlining areas for further investigation.

# 2	Discussion

BT sees several key benefits of a HLS architecture to enhance coverage, services and security within an Operator’s network, which would not be feasible if only Lower Layer Split (LLS) and monolithic node architecture deployments are available.

Removing User Plane Cleartext handling at network edge and 3rd party site location within the RAN node.

Currently, in 5G systems, user plane cleartext is handled within a monolithic node where the IPSEC terminates at the NG interface within the gNB before PDCP cyphering is applied. Although SA3 specifications [2] address the associated risks, Operators may opt to centralise the PDCP function to prevent exposure of cleartext user plane data at the edge of the network where physical and network security is less robust.

For 6G, however, if only the LLSs are referenced within 3GPP specifications, centralised PDCP function (or equivalent 6G function) becomes challenging as the LLS demands stringent transmission requirements, which are often unsuitable for rural areas due to limitations in infrastructure and increased site distances.

Observation 1: To enhance security, Operators may decide to centralise the PDCP function (or equivalent 6G function) into Operator’s secure/trusted environment.

Figure 1 highlights the location of PDCP function within different RAN architecture deployments in 5G.

Figure 1 PDCP function within different architecture deployments.

Centralising RAN in Rural/Semi Urban areas.

Centralising the RAN in rural/semi urban areas is difficult because LLS demands strict fronthaul requirements. One of the main challenges with implementing LLS in rural environments stems from the need for a highly reliable, low latency and high capacity transport network. Rural areas often struggle to meet LLS transmission requirements without costly and complex network upgrades, mainly because of greater site distances and limited suitable hub locations—especially when considering geo-redundancy.

Additionally, the high transmission capacity required for wide-band radio solutions, such as mmWave, can limit the inter-site distances for LLS in urban environments, making deployments more challenging.

Observation 2: Centralising the RAN in rural and some urban areas is challenging using the Lower Layer Split due to increased site distances and availability of suitable hub sites.

In these environments, a Higher Layer Split can be beneficial because it has less demanding midhaul requirements (typically 5-10ms) than the Lower Layer Split, while still maintaining certain centralised functions that improve system performance, security, efficiency and operations.

Proposal 1: RAN3 should study how the 6G RAN split architecture can enable centralisation of RAN functions where the Lower Layer Split RAN architecture cannot be deployed within the Operator’s network.

Enabling Low-complexity Indoor solutions.

4G/5G Femto solutions are used by some Operators as a cost-effective, low-capacity solution for enterprise locations such as shops or small offices where traditional in-building systems are often impractical or uneconomical to deploy. Conventional indoor RAN equipment typically requires centralised hardware (BBU) within third-party communications rooms with its own power, heat and space requirements, or necessitates specialised cabling. Although monolithic RAN nodes can be deployed on-premises, they operate in isolation when a centralised function is absent.

Deploying Femto in this deployment scenario is not ideal as the nodes lack coordination without a centralised function and only providing an aggregation function. The CU/DU architecture in regard to Femto was discussed within R19 Study on additional topological enhancements for NR [2], but it was deemed that the F1 interface was unsuitable for a Femto type solution as it was not designed for this purpose.

RAN3 should study how the architecture could support low-complexity DUs hosted at enterprise locations connected over midhaul transmission to CUs from day one. The CU could be placed within the Operator’s secure premises, instead of locating it within the third party's building, allows for centralised/aggregation functions by the CU.

Proposal 2: RAN3 should study how the 6G RAN architecture could support low-complexity DUs connected over midhaul transmission to a CU.

Low Complexity/Lightweight RAN node for Satellites/UAV

Similar to the ‘Low-complexity Indoor solutions’ topic, the requirement for satellites and UAVs is to maintain a simple, lightweight solution for NTN. Using the CU/DU split appears to be an effective approach, as functions related to the CU can be ground based. It may be more appropriate for satellite Operators to advocate these exact requirements, having a common split RAN architecture for all these different deployment scenarios would be desirable.

Proposal 3: RAN3 should study an unified 6G split RAN architecture applicable to both TN and NTN deployments.

BT considers the HLS architecture as an emerging RAN architecture within 5G networks and notes that it is already in use by several Operators globally. BT believes that the HLS should be specified in 6G, but with improvements to enable more deployment scenarios to be fully supported by HLS, rather than being addressed in later releases.

Proposal 4: RAN3 should study the applicability of a Higher Layer Split within 6G RAN architecture.

# 2	Conclusions

Observation 1: To enhance security, Operators may decide to centralise the PDCP function (or equivalent 6G function) into Operator’s secure/trusted environment.

Observation 2: Centralising the RAN in rural and some urban areas is challenging using the Lower Layer Split due to increased site distances and availability of suitable hub sites.

Proposal 1: RAN3 should study how the 6G RAN split architecture can enable centralisation of RAN functions where the Lower Layer Split RAN architecture cannot be deployed within the Operator’s network.

Proposal 2: RAN3 should study how the 6G RAN architecture could support low-complexity DUs connected over midhaul transmission to a CU.

Proposal 3: RAN3 should study an unified 6G split RAN architecture applicable to both TN and NTN deployments.

Proposal 4: RAN3 should study the applicability of a Higher Layer Split within 6G RAN architecture.

# 3	References

- Chair Notes RAN3#129-bis

- 3GPP TS 33.501 Security architecture and procedures for 5G System

- 3GPP TR 38.799 Study on additional topological enhancements for NR