3GPP TSG-RAN WG3 Meeting #130	R3-258565

Dallas, USA, November 17-21, 2025

Agenda Item:	10.4

Source:	THALES, ESA, EchoStar, Lockheed Martin, Novamint

Title:	6G RAN internal functional split for NTN

Document for:	Discussion

# 1	Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting and updated at RAN#109 [1]. It states that internal function split is considered for RAN architecture.

(6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] a) Overall RAN architecture aspects [RAN3, RAN2] b) RAN-CN functional split, interface, protocol stack and procedures [RAN3] c) RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

6GR also aims to harmonize TN and NTN radio interface :

j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

In RAN3#129bis meeting, 6G RAN internal functional split was discussed and following guidelines by Chair to prepare the discussion for the RAN3#130 meeting.

Capture pain points and benefits (and potentially derived requirements) of 5G HLS To be continued...

In this contribution, we will share our views for supporting a flexible internal functional split for NTN.

# 2	Discussion

The split architecture is described for 5G in [2]. In this architecture, the gNB Distributed Unit (DU) provides radio access with the lower layers, and the gNB Centralized Unit (CU) handles the higher layers and connectivity with the core network. Each gNB-DU can be connected to only one gNB-CU at a time (no CP-UP split), but multiple gNB-DUs can be handled by one gNB-CU.
This architecture offers flexibility for deployment in remote areas of terrestrial networks. The F1 interface is used as a transport network layer between the gNB-CU and gNB-DU for both the user plane and control plane. Several options are specified, as illustrated below, with some attracting more interest than others according to latency requirements and other gains.

Figure 1 - functional split options

The support for split architecture in NTN was studied in Rel-16 [3], with split option 2 (F1 interface between the RLC and PDCP layers) considered as the baseline. The split architecture was not specified in the stage 2 specifications, whereas the transparent architecture (introduced in Rel-17) and the regenerative architecture with a fully integrated gNB (introduced in Rel-19) were included. However, split architecture can still be supported through implementation with appropriate adaptations to handle the long delays on the feeder link, such as adjusting timers for RRC and L3, and providing buffering on the F1 user-plane interface.

For 6G, supporting the base station split architecture for NTN offers several significant benefits and deployment options. Some deployment scenarios are described in [4]. Scenario (3) demonstrates some benefits for NTN, such as reducing the computation and memory footprint on the satellite, while limiting throughput and latency constraints on the feeder link are considered as pain points.

|  | Scenario On the satellite On the ground |  | Benefits | Pain points |
| --- | --- | --- | --- | --- |
| (3) | RU+DU | CU + Core network | Limited computation and memory footprint on the satellite while limiting the feeder link throughput and latency constraints | High mobility between DU and CU and inter-satellite handover relies on the ground segment |

Optimizing the network function footprint on satellite payload computation and memory is critical for deployments. For instance, for each UE and for each data radio bearer (signaling radio bearer for control plane), an entity at each protocol layer is instantiated. For the access node (i.e., gNB in 5G), the number of instantiated entities can be quite large, resulting in increased memory and computational cost. For a satellite where the gNB is fully embarked, the satellite payload and the related gNB(s) can handle hundreds of cells (as illustrated below with a LEO-600 field of view with 1,058 beams that can be generated), with terminals in each cell, resulting on a large memory and computational footprint on the satellite payload.

Figure 1 : View field of a LEO-600 satellite with 1058 beams

The satellite constrained SWaP (Size, Weight and Power) has to be used at its best for the user service and, in particular, on the service availability and throughput.

By leveraging the functional split between, NTN deployments can achieve greater flexibility and efficiency in network design and resource allocation. This separation allows part of the baseband processing to be centralized on ground stations or cloud infrastructure while distributing real-time radio functions closer to the satellite or airborne platform. As a result, it reduces latency, optimizes bandwidth usage over satellite links, and enhances scalability by enabling dynamic allocation of processing resources depending on the traffic demand. Moreover, the split architecture facilitates easier upgrades and maintenance by isolating components, thus improving the overall operational resilience and adaptability of NTN systems to varying mission requirements and environments.

Proposal	The 6G RAN architecture should support various base station internal functional splits able to accommodate the NTN characteristics and allow the selection of the minimum set of RAN functions to be embarked on board for the mission purpose.

# 3	Conclusion

Proposal	The 6G RAN architecture should support various base station internal functional splits able to accommodate the NTN characteristics and allow the selection of the minimum set of RAN functions to be embarked on board for the mission purpose.

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

3GPP TS 38.401, “5G; NG-RAN; Architecture description”, Rel-18, May 2025.

3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, Rel-16, May 2021.

R3-257192, “6G RAN overall architecture to support NTN”, Prague, CZ, October 2025.