3GPP TSG-RAN WG3 Meeting #130	R3-258568

Dallas, TX, USA, 17th – 21st November 2025

Title: 	Functions and Architecture for the 6G RAN

Source: 	Ericsson

Agenda item:	10.2.2

Document Type:	Discussion and agreement

# 1. Introduction

In [1] a new Study Item was approved to study aspects of the 6G RAN. One of the objectives of the SID is to derive a 6G RAN architecture that fulfils the objectives of the study. Some of the objectives that are relevant for RAN3 in its early phases of 6G RAN architecture design are reported below:

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. […] The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. […] Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. […] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] […] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] […] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

At the same time, RAN plenary agreed the following requirements for 6G RAN (see [2]):

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

In this paper we take the SID in [1] and the requirements in [2] as a starting point to outline the functionalities to be supported by the 6G RAN, as well as a baseline architecture fit to fulfil the requirements established so far.

# 2. Discussion

## 2.1 Functionalities and baseline architecture for the 6G RAN

As a general starting principle, we do not see the need to change the functional split between the 6G RAN and the CN established in 5G. We believe such functional split is well proven and able to support requirements for 6G. Obviously, new features will be supported in 6G, and for those features new functionalities will be supported at both the 6G RAN and the CN for 6G.

Proposal 1: Capture in the “RAN-CN Functional Split” section of TR38.760-3 that “For 5G functionalities also supported in 6G, the 6G RAN-CN functional split is the same as the RAN-CN functional split for 5G”

When it comes to functionalities under RAN3 responsibility to be supported by the 6G RAN, they can be divided in two areas. Functionalities that can be mapped to the 5G RAN and functionalities that are new to 6G. 
We note that, while some of the functionalities supported in the 5G RAN and listed in TS38.401/TS38.300 could be already agreed today for the 6G RAN, support of other 5G functionalities would depend on progress on the 6G RAN architecture and on which features should be supported by the 6G RAN.

As an example, the following functionalities currently listed in TS38.401/38.300 could be assumed to be supported by the 6G RAN:

- RAN sharing *;

- Support for Non-Public Networks*;

- Support of AI/ML**;

- Routing of User Plane data towards UPF(s)*;

- Routing of Control Plane information towards AMF*;

- Support of Network Slicing*;

*This functionality is confirmed in the requirements agreed in [2]

**This functionality is confirmed in the objectives captured in [1]

Based on the above, the following is proposed.

Proposal 2: Capture in the “RAN Functions” section of TR38.760-3 that the 6G RAN is required to support the following functionalities:

-	NG-RAN sharing;

-	Support for Non-Public Networks;

-	Support of AI/ML;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Support of Network Slicing;

### 2.2 RAN-CN Interface

There is a strong market requirement to enable the 6G RAN and the CN for 6G to be provided by different vendors. For this reason, it is imperative that 3GPP defines an interoperable RAN-CN interface for 6G.

Proposal 3: RAN3 shall define an inter vendor interoperable RAN-CN interface for 6G

One of the fundamental principles to achieve a 6G RAN design that improves the overall RAN performance is to ensure that handling of UE-associated processes such as UE context management, UE configurations, UE state management, UE capability handling etc. shall be carried out by a single logical RAN entity. 
Likewise, we can extend this principle to the logical entity in the CN that would terminate the RAN-CN interface supporting UE associated procedures. Namely, we propose that the RAN shall interface with a single logical entity at the CN for the handling of UE-associated processes such as UE context management, UE configurations, UE state management, UE capability handling etc. We do understand that there could be multiple functions within the 6G CN that would handle the UE (e.g. AMF, SMF), but the principle we would like to stress is that “in the eyes of the RAN” there should be only one logical entity terminating the UE associated procedures over the RAN-CN interface. Such entity is, in 5G, the AMF.

The latter principle has a number of important advantages that we describe as follows.

By letting the RAN to interface with a single CN logical entity, communication over the RAN-CN interface is always “coherent”. Namely, the single logical entity at the CN would ensure the following:

- Avoid receiving of messages at the RAN containing contradicting or non-matching configurations/policies/information. This could be the case if the RAN interfaces with multiple different logical entities each responsible for some level of communication with the RAN

- Avoid race conditions between different procedures. This could be the case if the RAN interfaces with multiple different logical entities sending messages to the RAN that need to follow a well-defined temporal order.

- Avoid exposure of the 6G RAN to a multitude of logical CN entities, requiring maintenance of the connection towards such nodes. The latter would add large overheads due to management of the communication channels towards each of the logical CN entities communicating to the RAN. Furthermore, security implications should be also considered as connections to multiple logical CN entities would imply a larger surface area of attacks towards RAN.

For this reason, we propose the following:

Proposal 4: Capture in the “RAN-CN Functional Split” section of TR38.760-3 that “UE Associated procedures between the RAN and the 6G CN shall be terminated between a single logical entity at the RAN and a single logical entity at the 6G CN”.

Additionally to the above, and to take advantage of the existing and well proven support for 5G functionalities over the current NG interface, we propose that, for those functionalities available in 5G and to be supported in 6G, the 6G RAN-CN interface design should be based on the existing 5G NG interface procedures.

We do not exclude enhancements for such functionalities. However, we believe that this approach allows to exploit the strength of the5G RAN-CN interface design, to maximum reuse of existing implementations and to ensure an easier path to feature parity between 5G and 6G.

Proposal 5: Capture in the “RAN-CN Functional Split” section of TR38.760-3 that “for those functionalities available in 5G and to be also available in 6G, the 6G RAN-CN interface design should be based on the existing 5G NG interface procedures”

## 2.2 RAN internal interfaces

To guarantee interoperability between clusters of 6G RAN nodes from different vendors, it is essential to support a “horizontal” interface between RAN nodes. Such interface has been defined as the “Xn” in 5G.

The Xn, like other horizontal interfaces defined in the past for different generations of mobile networks, has supported numerous functionalities. However, the functionalities we identify as the most essential for the purpose of inter vendor interoperability over a RAN-RAN interface are the following:

- The interface management and error handling function

- The UE mobility management function (including both CP and UP functionalities)

We would, at least at this stage of the 6G study, focus on these functionalities for the RAN to RAN interface and we propose to discuss possible further requirements that would justify the introduction of more functionalities on such interface.

Proposal 6: RAN3 shall define an inter vendor interoperable RAN-RAN interface for 6G, supporting the following functionalities:

- Interface management and error handling function

- UE mobility management function (including both CP and UP functionalities)

## 2.2 Overall 6G RAN architecture

Based on the proposals provided above, we derive a high level representation of the 6G RAN architecture based on principles outlined so far:

Figure 1: Overall high level description of the 6G RAN architecture and interfaces

The figure above shows an architecture derived by taking into account what is in our opinion the essential interfaces to be standardised to achieve multi vendor interoperability.

We would like to point out that the above architecture represents an option that reflects what is vastly supported in 5G deployments today. This architecture option is essential for cases where all RAN functionalities need to be hosted in the same logical entity. 
Hence, it is important to confirm that a RAN architecture based on a single logical RAN node shall be defined by RAN3, and that features/interfaces shall be developed taking this architecture option in mind. 
Converging to the acknowledgement of the architecture in Figure 1 does not preclude RAN3 from discussing other architecture options, but at the same time enables RAN3 to have a baseline architecture on which RAN3 can work. This could, for example, enable RAN3 to move forward with the definition of a RAN node, to agree on the need of a RAN to RAN interface and to determine the RAN entity that would terminate a RAN-CN interface.

Proposal 7: Capture in the “General Principles” section of TR38.760-3 that a RAN architecture based on a single logical RAN node, supporting all RAN protocols and functionalities, shall be defined by RAN3, and that features/interfaces shall be developed taking this architecture option into account. Other architecture options are not precluded.

# 3. Conclusions

In this paper we have derived principles and design guidelines for the overall standardisation of the 6G RAN functions and interfaces. The proposals derived are reported below:

Proposal 1: Capture in the “RAN-CN Functional Split” section of TR38.760-3 that “For 5G functionalities also supported in 6G, the 6G RAN-CN functional split is the same as the RAN-CN functional split for 5G”

Proposal 2: Capture in the “RAN Functions” section of the RAN3 TR on 6G that the 6G RAN is required to support the following functionalities:

-	NG-RAN sharing;

-	Support for Non-Public Networks;

-	Support of AI/ML;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Support of Network Slicing;

Proposal 3: RAN3 shall define an inter vendor interoperable RAN-CN interface for 6G

Proposal 4: Capture in the “RAN-CN Functional Split” section of TR38-760-3 that “UE Associated procedures between the RAN and the 6G CN shall be terminated between a single logical entity at the RAN and a single logical entity at the 6G CN”.

Proposal 5: Capture in the “RAN-CN Functional Split” section of TR38.760-3 that “for those functionalities available in 5G and to be also available in 6G, the 6G RAN-CN interface design should be based on the existing 5G NG interface procedures”

Proposal 6: RAN3 shall define an inter vendor interoperable RAN-RAN interface for 6G, supporting the following functionalities:

- Interface management and error handling function

- UE mobility management function (including both CP and UP functionalities)

Proposal 7: Capture in the “General Principles” section of TR38.760-3 that a RAN architecture based on a single logical RAN node, supporting all RAN protocols and functionalities,  shall be defined by RAN3, and that features/interfaces shall be developed taking this architecture option into account. Other architecture options are not precluded.

It is proposed to agree to the proposals above as well as to the text proposal in the Annex

# 4. References

[1] RP-252912, New SID: Study on 6G Radio

[2] RP-252870, pCR for 38.914 on Requirements for architecture and migration for 6G, NTT DOCOMO (moderator)

# 5. Annex: pCR for TR 38.760-3

# <<<<<<Start of Changes>>>>>>

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

A RAN architecture based on a single logical RAN node, supporting all RAN protocols and functionalities, shall be defined by RAN3. Features/interfaces shall be developed taking this architecture option into account.

FFS whether other architecture options are supported.

<<<<<<Next Change>>>>>>

## 5.3	Overall RAN architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

### 5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN

The following principles shall be fulfilled:

- For 5G functionalities also supported in 6G, the 6G RAN-CN functional split is the same as the RAN-CN functional split for 5G;

- UE Associated procedures between the RAN and the 6G CN shall be terminated between a single logical entity at the RAN and a single logical entity at the 6G CN

- For those functionalities available in 5G and to be also available in 6G, the 6G RAN-CN interface design should be based on the existing 5G NG interface procedures

### 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN

The 6G RAN shall support the following functionalities:

-	NG-RAN sharing;

-	Support for Non-Public Networks;

-	Support of AI/ML;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Support of Network Slicing;

<<<<<<End of Changes>>>>>>