3GPP TSG-RAN WG3 Meeting #130	R3-258581

# Dallas, US, 17 – 21 November 2025

Source: KDDI Corporation

Title:　 Considerations on 6G RAN-CN Interface

Agenda Item:	10.3.1. General principles and requirements

Document for:	Discussion

- Introduction

This document proposes a direction for the 6G RAN–Core Network (RAN-CN) interface, focusing on function scope, interface architecture, and transport protocol. We recommend that RAN3 base its discussions on TS 38.410 functions, aligned with SA2’s 6G Day 1 feature list. For legacy services like voice and mobility, point-to-point (P2P) interfaces and SCTP remain optimal and require no change. SBI and QUIC may be considered for future services, but only after their requirements are clearly defined. This approach balances stability with flexibility for 6G evolution.

- Discussion

  - 6G RAN-CN Interface Functions

SA2’s 6G SID [1] emphasizes the principle that “duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.” This suggests that the division of responsibilities between RAN and CN will remain consistent with 5G. Therefore, it may be reasonable to base our discussions on the NG interface functions defined in TS 38.410 Section 5.

5 Functions of the NG interface 8 5.1 General 8 5.2 Paging function 8 5.3 UE Context Management function 8 5.4 Mobility Management function 8 5.5 PDU Session Management function 8 5.6 NAS Transport function 9 5.7 NAS Node Selection function 9 5.8 NG Interface Management function 9 5.9 Warning Message Transmission function 9 5.10 Configuration Transfer function 9 5.11 Trace function 9 5.12 AMF Management function 9 5.13 Multiple TNL Associations Support Function 10 5.14 AMF Load Balancing function 10 5.15 Location Reporting function 10 5.16 AMF Re-allocation function 10 5.17 UE Radio Capability Management function 10 5.18 NRPPa Signaling Transport function 10 5.19 Overload Control function 10 5.20 Report of Secondary RAT data volumes Function 10 5.21 RIM Information Transfer function 10 5.22 Retrieve UE Information function 11 5.23 RAN CP Relocation Indication function 11 5.24 Suspend-Resume function 11 5.25 Connection Establishment Indication Function 11 5.26 AMF CP Relocation Indication Function 11 5.27 NR MBS Session Management function 11 5.28 Multicast Group Paging Function 11 5.29 QMC support function 11 5.30 MT Communication Handling function 11 5.31 Timing Synchronisation Status Reporting function 12 5.32 AIoT Service Management function 12

List of functions of the NG interface (excerpt from TS38.410)

However, we believe that the design of the 6G RAN-CN interface is closely tied to how the 6G Core Network will be structured in SA2 and might not be able to discuss without inputs from SA2. Currently, SA2 is discussing the 6G Day 1 feature list [2], which is expected to be finalized at the upcoming November meeting. 6G RAN-CN Interface Functions also somewhat depends on how the 6G architecture will be designed in SA2. For instance, a Clean Slate 6G Core might aim to avoid heavy reliance on a centralized AMF and simplify control signaling, whereas an evolved 5G Core approach would reuse much of the 5G Core to minimize introduction costs. This discussion also impacts on the RAN-CN interface functions.

At the October RAN3 meeting, agreement was reached on three specific NG interface functions for 6G (highlighted in yellow above). However, we believe it is premature to discuss the remaining NG interface functions before the 6G Day 1 feature list is finalized by SA2. While several contributions to RAN3#123bis propose architectural options such as SBI or P2P for the RAN–Core Network (RAN-CN) interface, we believe that before delving into architectural discussions, it is essential to first clarify which RAN-CN interface functions should be supported in 6G.

Proposal 1: RAN3 should identify the functions of the NG interface based on the outcome of the 6G Day 1 feature discussions from SA2, starting from the upcoming February meeting.

  - SBI/P2P Interface Discussion

For legacy communication services such as voice, mobility management, and session management, we recommend continuing the use of point-to-point (P2P) interfaces. These services have well-defined and stable interactions between the RAN and Core Network and are already efficiently supported by NGAP procedures. The flexibility and extensibility offered by Service-Based Interfaces (SBI) are more suitable for dynamic and evolving services. For legacy functions like voice call setup or handover, the signaling flow is fixed, and converting it into a service-based API does not provide any functional advantage. In fact, introducing SBA concepts into a traditional RAN-Core interface offers only limited benefits.

For emerging services such as AI/ML and sensing, we are open to discussing the adoption of SBI, but only after the corresponding service requirements and functional specifications are clearly defined. Even in these cases, RAN3 should carefully evaluate how much flexibility and extensibility are actually needed. If the RAN node’s role is simply to forward information to another node, then SBI may not be necessary.

In summary, it is important to remain open-minded. The fact that SBI offers no clear benefit at this time does not mean it won’t evolve or become more relevant in the future. If new evidence emerges—such as a specific legacy procedure that clearly benefits from a service-based approach—RAN3 can reconsider. Until then, the safest and most efficient path for 6G is to prioritize P2P interfaces. Therefore, we propose the following:

Proposal 2: RAN3 should discuss and identify which functions qualify as “legacy communication services,” based on the outcome of the 6G Day 1 feature discussions from SA2.

Proposal 3: RAN3 should focus exclusively on point-to-point (P2P) interfaces for legacy communication services.

  - Transport Layer enhancement (SCTP/QUIC)

At recent RAN3 discussions, there are some proposals to consider QUIC (Quick UDP Internet Connections) as a new transport protocol for base station (gNB) to core network signaling, replacing the current SCTP (Stream Control Transmission Protocol). While QUIC offers modern features (e.g. built-in encryption, 0-RTT setup, path migration), we believe SCTP remains fully sufficient and optimal for the NG-RAN ↔ 5GC control-plane interface. Below we compare the two protocols across key aspects relevant to 3GPP RAN3.

QUIC is an excellent protocol for internet-based applications, offering low latency connection setup, built-in security, and easy NAT traversal. Some companies suggest QUIC could simplify the protocol stack between RAN and core by removing the need for SCTP-specific handling, providing encryption, and aiding future cloud-native deployments. We acknowledge these potential benefits. However, most of those benefits are either already provided by SCTP in our scenario or are not directly applicable to the gNB–core environment:

Low latency and Head-of-Line Blocking: SCTP’s multi-stream capability and ability to bundle data in the 4-way handshake largely mitigate latency concerns on a reliable link. In RAN-CN interface, the extra RTT in setup is manageable, and long-lived associations mean setup happens infrequently. Both SCTP and QUIC protocols avoid HOL blocking for independent streams, so signaling procedures won’t stall each other.

Reliability and failover: Both SCTP and QUIC ensure reliable delivery. SCTP’s proven multi-homing feature directly addresses link failover, which is a key reliability requirement for RAN-CN interface (e.g. dual-homed transport networks). QUIC’s connection migration is more aimed at user devices moving across networks, which is not a typical scenario for network side elements.

Security: QUIC’s always-on encryption is a double-edged sword in our use-case. While it adds confidentiality, it also prevents diagnostic monitoring of signaling (unless additional probe endpoints or key-sharing mechanisms are introduced). Given that 5G NGAP messages often travel over secure, closed links (or already within IPsec tunnels when crossing untrusted domains), mandatory QUIC encryption is not a compelling advantage. SCTP can remain in use with security applied at the network layer as needed.

Complexity: Introducing QUIC means coexisting with SCTP (since legacy LTE interfaces and other 5G options still use SCTP). This adds complexity to network nodes which must then maintain both protocol stacks. The maturity of SCTP in telecom means the “edge cases” and performance under load are well characterized, whereas QUIC is relatively new – especially in a telco control-plane context, which has different traffic patterns than web browsing.

There is currently no strong technical justification for replacing SCTP with QUIC for RAN–Core Network (RAN-CN) interface signaling. SCTP fully meets existing requirements for latency, reliability, and throughput, and does so with minimal operational risk. While QUIC offers advantages in internet-based scenarios, its features do not provide clear incremental value for network-side elements operating over well-managed transport infrastructure. We remain open to exploring QUIC in conjunction with Service-Based Interfaces (SBI) for emerging services. However, we believe this does not warrant the cost and risk of change for legacy communication services.

Proposal 3: RAN3 should continue to focus exclusively on SCTP/IP as the transport protocol for legacy communication services.

- 3. Conclusion

This contribution discusses a direction for the 6G RAN–Core Network (RAN-CN) interface, function, interface architecture, and transport protocol and proposes the following:

Proposal 1: RAN3 should identify the functions of the NG interface based on the outcome of the 6G Day 1 feature discussions from SA2, starting from the upcoming February meeting.

Proposal 2: RAN3 should discuss and identify which functions qualify as “legacy communication services,” based on the outcome of the 6G Day 1 feature discussions from SA2.

Proposal 3: RAN3 should focus exclusively on point-to-point (P2P) interfaces for legacy communication services.

- 4. References

[1] S2-2506096 	Study on Architecture for 6G System		ZTE (Moderator)		SA2#169

[2] S2-2509204	Summary of e-mail discussion on 5G feature list and way forward	Qualcomm Incorporated (moderator)	SA2#171