3GPP TSG-RAN WG3 Meeting #130	R3-258603

Dallas, USA, Nov 17th - Nov 21st, 2025

Agenda item:	14.2

Source:	CMCC

Title:	(TP for TS38.300 BL CR) Discussion on Topology 2 for A-IoT

Document type:	Other

Document for:	Discussion

Introduction

In RAN3#129bis meeting, the following agreements have been made [1]:

- General Arch For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied. NGAP procedures specified for Topology 1 are base line for Topology 2. - NGAP signaling procedures (Inventory and Command) The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as base line. - UE Reader Authorization UE Reader Authorization is performed at 5GC The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB. FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

In this contribution, we would like to provide our further views on the issues related to A-IoT Topology 2.

Discussion

2.1 Relationship with R19 Topology 1

In Release 19, A-IoT topology 1 has been specified to support A-IoT. When it starts specifying topology 2 in R20, some clarification is necessary on the relationship between topology 1 and topology 2. To be specific, the objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified. Therefore, from RAN3 perspective, it is obvious that the A-IoT enabled gNB for R19 and that for R20 are not co-located. In other words, we do not need to consider the scenario where the same gNB supports both Topology 1 and Topology 2 for device 1.

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

2.2 UE-reader authorization

In order to identity UEs which support A-IoT, UE-reader authorization information need to be made available in NG-RAN. In RAN3#129bis meeting, it has been agreed that UE-reader authorization is performed at 5GC and the UE Reader authorization status is provided from the 5GC to the gNB.

From our point of view, the UE-reader authorization procedure can be performed in the following cases: registration, service request, HO, etc. For handover/re-establishment case, we think that supporting the UE Reader authorization status to be provided from the source/old gNB to the target/new gNB via Xn is beneficial for latency reduction than always retrieving from 5GC. To be specific, AMF should include the UE-reader authorization information for A-IoT to NG-RAN during initial context setup, UE context modification, handover preparation and path switch procedures. Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover preparation or Retrieve UE Context procedures. In terms of split architecture, we are open to discuss whether to support it or not.

To this end, we support to define a new IE, which is named “5G UE-reader Authorized”, and introduced at least in the INITIAL CONTEXT SETUP REQUEST message, UE CONTEXT MODIFICATION REQUEST message, HANDOVER REQUEST message, PATH SWITCH REQUEST ACKNOWLEDGE message, HANDOVER REQUEST message, RETRIEVE UE CONTEXT RESPONSE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5G UE-reader Authorized |  |  |  |  |  |  |
| > A-IoT enabled UE | O |  | ENUMERATED (authorized, non-authorized, ...) | Indicates whether the UE is authorized to act as A-IoT enabled UE | - | - |

Proposal 2: The UE Reader authorization status should be provided from source/old gNB to the target/new gNB via Xn.

Proposal 3: Define a new IE “5G UE-reader Authorized” to indicate whether the UE is authorized to act as an UE-reader.

Proposal 4: AMF should include the UE-reader authorization information for A-IoT to NG-RAN in Initial Context Setup, UE Context Modification, Handover Preparation and Path Switch procedures, i.e. introducing the “5G UE-reader Authorized” IE into INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE message(s).

Proposal 5: Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover Preparation or Retrieve UE Context procedures, i.e. introducing the “5G UE-reader Authorized” IE into HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message(s).

2.3 Selection of UE-reader

According to the final TS 38.300 CR, the reader selection related information within inventory procedure is described as following [2]:

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list). If the Requested Service Area Information contains: - neither the Requested AIoT Area List nor the Requested Reader List, the gNB shall select all readers it serves - the Requested AIoT Area List, the gNB shall select readers within the indicated AIoT Area(s) - the Requested Reader List, the gNB shall take the reader list into account to perform inventory.

It is clear that both CN and gNB are involved in the reader selection in R19. Regarding the selection of UE-reader, the related paragraph in TR 38.769 is described as follows [3]:

For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

For topology 2, with the consideration of the mobility of UE-reader, adjacent UE-readers using the same A-IoT air interface resources may cause severe mutual interference. Therefore, interference coordination between UE-readers and coordination of A-IoT radio interface resources become even more important than that in topology 1. Based on above analysis, since gNB is aware of the resource usage of the A-IoT radio interface and approximate position of the UE-reader (e.g., under which beam direction), we think that gNB is responsible for UE-reader selection. Core network can provide some assistance information to the gNB to assist UE-reader selection. RAN3 can discuss the details of UE-reader selection under several potential cases: No assistance information, or Assistance information with area information, or Assistance information with UE-reader information, or Assistance information with both area information and UE-reader information.

- Case 1：No assistance information

Considered this case, we think gNB can have the freedom to select the UE-readers based on its implementation.

- Case 2：Assistance information with area information

In last meeting, some companies proposed to distinguish between the fixed UE-reader(s) and the mobile UE-reader(s). From the operator’s perspective, we agree these two types of UE-readers will be deployed in real network. For mobile UE-reader, RAN3 should discuss whether it can be mapped to A-IoT Area ID(s) or not. While, for the behavior after gNB receives the assistance information with area information, we think it should select the UE-reader(s) which is definitely mapped to the provided area information.

- Case 3：Assistance information with UE-reader inforamtion

For this case, we think the CN sends the list of UEs that have been authorized  as readers to the gNB. Then, the gNB selects the suitable UE-reader from the UE list provided by the CN to perform inventory or command procedure.

- Case 4：Assistance information with both area information and UE-reader information

For this case, we think that take both area information and UE-reader information into consideration when selecting the appropriate UE-readers.

Observation 2: Due to the mobility of UE-reader(s), interference coordination between UE-readers and coordination of A-IoT radio interface resources become even more important than that in topology 1.

Proposal 6: The gNB is responsible for UE-reader selection, and core network can provide some assistance information to the gNB.

Proposal 7: RAN3 can further discuss the details of UE-reader selection under several potential cases: no assistance information, or assistance information with area information, or assistance information with UE reader information, or assistance information with both area information and UE-reader information.

2.4 Mobility of UE-reader

2.4.1 Handover

The handover preparation phase is used to establish necessary resources in an NG-RAN node for an incoming handover, where the UE context transfer, admission control and resources preparation at the target gNB are performed. In handover execution phase, the RAN node assists the UE in performing the handover, and a path switch is performed after the UE successfully handover to the target gNB.

For the handover procedure of UE-reader, the UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

During handover preparation for UE-reader, which performs A-IoT inventory/command, the A-IoT-related capability/information of the UE-reader (e.g., UE-reader Authorization Status, A-IoT radio interface resource, etc.) should also be transmitted to target gNB, so that the target gNB can perform admission control based on this information. Therefore, we think the A-IoT-related capability/information of the UE-reader can be considered as UE context and transmitted within Handover Request message. Similarly, in terms of split architecture (i.e. F1AP interface), some UE Context Management messages (e.g. UE CONTEXT SETUP REQUEST, UE CONTEXT SETUP RESPONSE, UE CONTEXT RELEASE COMMAND, UE CONTEXT MODIFICATION REQUEST, etc.) needs to be enhanced.

Observation 3: The UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

Proposal 8: In handover preparation phase, the A-IoT-related capability/information of the UE-reader can be defined as UE context and can be transmitted in Handover Request message.

2.4.2 Re-restablishment procedure

In the re-establishment procedure, UE context retrieval may be performed between gNBs, and path switch may be performed between the gNB and the CN node. Regarding A-IoT, the UE-reader in RRC_CONNECTED may also encounter problems such as RLF, reconfiguration failure, etc., thus the legacy re-establishment procedure may need to be enhanced. To be specific, the request for A-IoT-related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

Observation 4: In A-IoT, the UE-reader in RRC_CONNECTED may encounter problems such as RLF, reconfiguration failure, etc.

Proposal 9: The request for A-IoT related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT-related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

2.5 Signalling procedure

As discussed in Rel-19 A-IoT study item, the message flow for inventory and command in topology 2 in Clause 6.5.3.1.1 and 6.5.3.2.1 in TR 38.769 are given as follows:.

Figure 1: Message flow for A-IoT Inventory in Topology 2 (RRC-based solution) [3]

Figure 2: Message flow for A-IoT Command in Topology 2 (RRC-based solution) [3]

From our view, the above two message flows can be taken as the baseline for Topology 2 deployment, we think  the inventory request procedure, inventory report procedure, and command request procedure defined in Topology 1 can also apply to for Topology 2 with necessary enhancement. The main enhancement is adding the UE-reader's identifier in the messages, if needed. We are open to discuss which id can be used to represent the UE-reader, e.g. whether to reuse a legacy UE ID or to define a new ID for UE-reader.

Proposal 10: RAN3 can discuss which ID can be used to represent the UE-reader, and which message(s) should be enhanced to include the UE-reader ID.

Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT Topology 2:

Observation 1: The objective of R19 A-IoT is towards D1T1, where indoor A-IoT devices communicate with an indoor base station. In R20, D2T2-B scenario, where UE-reader assists the indoor A-IoT device to communicate with outdoor base station, is going to be specified.

Observation 2: Due to the mobility of UE-reader(s), interference coordination between UE-readers and coordination of A-IoT radio interface resources become even more important than that in topology 1.

Observation 3: The UE-reader can continue to perform the A-IoT procedure under the control of the network after a successful handover.

Observation 4: In A-IoT, the UE-reader in RRC_CONNECTED may encounter problems such as RLF, reconfiguration failure, etc.

Proposal 1: The scenario where the same gNB supports both Topology 1 and Topology 2 for device 1 is not considered in R20.

Proposal 2: The UE Reader authorization status should be provided from source/old gNB to the target/new gNB via Xn.

Proposal 3: Define a new IE “5G UE-reader Authorized” to indicate whether the UE is authorized to act as an UE-reader.

Proposal 4: AMF should include the UE-reader authorization information for A-IoT to NG-RAN in Initial Context Setup, UE Context Modification, Handover Preparation and Path Switch procedures, i.e. introducing the “5G UE-reader Authorized” IE into INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST and PATH SWITCH REQUEST ACKNOWLEDGE message(s).

Proposal 5: Source/Old gNB should include the UE-reader authorization information for A-IoT to NG-RAN to the target/new gNB during Handover Preparation or Retrieve UE Context procedures, i.e. introducing the “5G UE-reader Authorized” IE into HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message(s).

Proposal 6: The gNB is responsible for UE-reader selection, and core network can provide some assistance information to the gNB.

Proposal 7: RAN3 can further discuss the details of UE-reader selection under several potential cases: no assistance information, or assistance information with area information, or assistance information with UE reader information, or assistance information with both area information and UE-reader information.

Proposal 8: In handover preparation phase, the A-IoT-related capability/information of the UE-reader can be defined as UE context and can be transmitted in Handover Request message.

Proposal 9: The request for A-IoT related information can be added to RETRIEVE UE CONTEXT REQUEST message and A-IoT-related capability/information of the UE-reader can be added to RETRIEVE UE CONTEXT RESPONSE message.

Proposal 10: RAN3 can discuss which ID can be used to represent the UE-reader, and which message(s) should be enhanced to include the UE-reader ID.

References

- Chair’s notes, RAN3 chair, RAN3#129bis.

- R3-255964, Introduction of Ambient IoT, RAN3#129.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19), V19.0.0.

Text proposal for TS 38.300 BL CR

**************************Start of change*****************************

3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18].

*Skip unchanged part

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s). Up to three L2 U2N Relay UEs (i.e. one Last U2N Relay and up to two Intermediate U2N Relays including one First U2N Relay) can be configured for serving a L2 U2N Remote UE in multi-hop L2 U2N Relay communication in this release.

U2N Remote UE: a UE that communicates with the network via one or more U2N Relay UEs on an indirect path.

U2U Relay UE: a UE that provides functionality to support connectivity between two U2U Remote UEs.

U2U Remote UE: a UE that communicates with other UE(s) via a U2U Relay UE.

UE-reader: a UE providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.23.

UE-side (AI/ML) model: an AI/ML model whose inference is performed at the UE.

**************************Next change**********************

16.23	Support of Ambient IoT

16.23.1	General

A-IoT radio interface provides the communication between A-IoT device(s) and A-IoT reader, as illustrated in Figure 16.23.1-1. In this release, the A-IoT reader is a gNB-reader or a UE-reader. A-IoT radio interface can support both inventory procedure and command procedure as defined in TS 23.369 [68]. An A-IoT device monitors the R2D message as long as it has sufficient energy.

Figure 16.23.1-1: Architecture supporting the A-IoT radio interface

16.23.2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio, or the gNB supports communication with UE-readers by means of Uu interface.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with UE-readers by means of Uu interface.

-	A gNB serves one or multiple readers.

-	A gNB is aware of the mapping relationship between A-IoT Areas and gNB-readers by means of OAM configuration.

-	An A-IoT Area may include gNB-readers belonging to the same or different gNB.

-	One gNB-reader only belongs to one gNB.

-	One gNB-reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [68], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF). A gNB is aware of direct connectivity with the AIOTF via OAM.

NOTE:	A deployment with both direct connectivity and indirect connectivity is not expected.

NOTE:	A deployment with UE-reader only supports indirect connectivity.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of gNB-readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the gNB-readers supported by the gNB.

The AIOTF may also be aware of the gNB-reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, gNB-readers and A-IoT areas by means of OAM configuration, as needed.

The authorization of UE-reader is performed at core network.

************************End of change*********************