3GPP TSG-RAN WG3 Meeting #130	R3-258608

Dallas, USA, 17th – 21th Oct 2025

Title: 	Discussion on 6G RAN internal functional split and interfaces

Source: 	CMCC

Agenda item: 	10.4

Document Type:	Discussion and Decision

# Introduction

In RAN3#129bis, companies shared their opinions on 6G split architecture, but no consensus achieved. The following discussion will continue as guided by chair as following:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

# Discussion

This discussion paper will focus on the 6G RAN internal functional split and interfaces.

## 2.1 CU-DU split

CMCC has not deployed CU-DU split architecture network for commercial 5G yet. We are neutral to this option if no extra complexity of interworking will be introduced in 6G. In this chapter, we see the pain points and benefits as below:

Pain points: CU-DU split will introduce extra signal transfer. It not only increases the complexity of development for vendor but only increase the complexity of deployment and maintains for operators. Especially for operators who don’t adopt the CU-DU split architecture 5G network, introducing it in 6G will be a new challenge.

Benefit：For some operators already deployed CU-DU split architecture in 5G network, they may re-use the deployment and their experience to maintain. It won’t bring extra cost but avoid some hardware re-investment. And it also improves energy efficiency which is important for operators.

Potential use cases for 6G: In 6G, AI/ML and unified data platform are popular use cases under study. The hardware to support AI training and inference will be a big investment. It may be a good choice to only introduce AI/ML training/inference capability in some centralized entities, and use the existing interfaces to transfer AI models, collect data from nodes near UE. In R19, the F1 interfaces have been enhanced a little bit to support some of the AI/ML use cases.

In 6G, we may also see some potential deployment benefits in vertical industry solution. In R19 NR NTN and IoT NTN, regenerative payload is supported. However to host the whole gNB is still a big cost for satellite operators. In 6G, we may think about hosting the application layer on satellite, while keep the RLC and low layers on the ground. In this way, standardized split architecture may benefit to reduce the cost for NTN, and improve the user experience.

In summary, whether to keep CU-DU split architecture in 6G shall be use cases driven. And the interface design shall be meet the minimal requirement of the 6G new use cases, as simple as possible.

Proposal 1: RAN3 shall study the 6G new use cases first, to see whether the current 5G CU-DU split architecture and basic sets of F1 interface functions can be re-used to meeting the requirements.

Meanwhile, in R18 and R19 AIML journey, we saw that for some sensitive information, it is possible to use the current interface and procedure to transfer between different vendors. It is case by case to consider whether the interfaces need to be standardized.

Proposal 2: In 6G, if CU-DU split architecture is adopted, it is proposed to keep the interface as simple as possible, to avoid introducing the unnecessary complexity in network.

## 2.2 CU-CP and CU-UP split

Pain Points: 6G should re-evaluate the CU-CP and UP split option. Currently there is nearly no deployment to split the CU-UP and CU-UP, nor on E1 interfaces. It introduces extra complexity in network deployment and network operation, but we didn’t see the benefits. To learn the lesson from 5G, we should avoid it in 6G.

Proposal 3: It is proposed to not adopt CU-CP, CU-UP split architecture in 6G, no benefit seen currently to keep it.

## 2.3 Other internal function split and interface consideration

According to current contribution of 6G use cases, we see no need to introduce other internal function splits for 6G. So we suggest to focus on CU-DU split now.

Proposal 4：It is proposed that RAN3 shall not begin to discuss further internal function split and interfaces in 6G after the use cases are confirmed.

# 3	Summary

In this contribution, we provide additional input considerations on 6G RAN internal functional split and interfaces, following proposals are made:

Proposal 1: RAN3 shall study the 6G new use cases first, to see whether the current 5G CU-DU split architecture and basic sets of F1 interface functions can be re-used to meeting the requirements.

Proposal 2: In 6G, if CU-DU split architecture is adopted, it is proposed to keep the interface as simple as possible, to avoid introducing the unnecessary complexity in network.

Proposal 3: It is proposed to not adopt CU-CP, CU-UP split architecture in 6G, no benefit seen currently to keep it.

Proposal 4：It is proposed that RAN3 shall not begin to discuss further internal function split and interfaces in 6G after the use cases are confirmed.

# 4	Reference

[1] RAN3_129bis_agenda_20251017_EOM, chair note