3GPP TSG-RAN WG3 Meeting #130	             R3-258617

Dallas, USA, 17th – 21st November 2025

Agenda Item:	10.2.2

Source:	CMCC, NTT DOCOMO, FiberCop, T-Mobile USA, China Unicom

Title:	(pCR to TR 38.760-3) Considerations on RAN data collection function for 6G RAN

Document for:	Discussion

# Introduction

The 6G RAN SID was approved at RAN#108 that aims to develop a one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study in TR38.914 [1].

One of the objectives of the SID is stated as follows:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

It is noted that the study of overall RAN side architecture to support of various services and functionalities (e.g. AI/ML, sensing, etc) is required in RAN3 and RAN2.

Meanwhile, another objective in the SID is stated as follows:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

…

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Meanwhile, the data transfer design over radio interface also needs to be considered to support various types of data (e.g. AI/ML, Sensing, etc) between RAN2 and RAN3. In this contribution, we will take data collection function as a start point and analyse how the requirements of data collection in 6G AI/ML service will impact the RAN architecture.

# Discussion

As SID explicitly points out the overall RAN architecture aspects and data transfer design are studied in both RAN3 and RAN2, the necessary cooperation is required between two groups to check the progress of each group for 6G network design. For reference, the agreements on the data collection framework reached during the RAN2#131bis meeting are presented as follows:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features - These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

The RAN2's agreements gives RAN3 a good direction where the study on standardized data collection framework for AI/ML should be taken as baseline and the study on further requirements of other non-AI/ML use cases/services is to understand whether the common requirements exist across different use cases.

Observation 1: The RAN2's agreements gives RAN3 a good direction where the study on standardized data collection framework for AI/ML should be taken as baseline and the study on further requirements of other non-AI/ML use cases/services is to understand whether the common requirements exist across different use cases.

From RAN3 perspective, we totally agree the spirit of above RAN2 agreements. Given that AI/ML services have already been extensively explored in 5G-A, the research on the 6G RAN architecture and radio interface protocol will start from AI/ML services to uncover their potential impact on network design in RAN side. Regarding other new services, once the data collection framework for AI/ML is finalized, RAN3 needs to further consider whether such framework for AI/ML can also fulfil the requirements from those services.

Proposal 1: RAN3 should start with studying AI-centric data collection framework.

In 6G study, RAN3 decided to start the AI/ML use cases discussion before the discussion on AI/ML framework. In last meeting, the AI/ML based Network energy saving and AI/ML based mobility optimization are identified and agreed as 6G AI/ML services to be studied in RAN3. To some extent, there is a high degree of consistency in the study on 6G AI/ML use cases and 5G-A use cases. As thoroughly studied in 5G-A, the transferred IEs and data transfer procedures for both use cases have been fully discussed and well defined. Hence, the data transmission mechanism used in new AI/ML use cases for 6G are unlikely to bring disruptive changes on the AI/ML framework. In our position, it is unnecessary to start the use case first and then continue the framework discussion. The data collection design as the basic function of AI/ML service should always be discussed simultaneously with AI/ML use cases since the architecture for the data collection provides the most fundamental support for the operation of AI/ML use cases.

Observation 2: The data collection design should always be discussed simultaneously with AI/ML use cases since the architecture for the data collection provides the most fundamental support for the operation of AI/ML use cases.

In addition, the latest agreements in RAN2 points out the study on termination points (i.e. understand who are producers, consumers of the data, data collection points). This agreement also provides a good guidance for RAN3 to consider the termination points within the 6G network. In 6G, it is assumed that the data can be terminated at any termination points in different domains. Considered it is possible that the AI/ML data could be collected and consumed in RAN, entities within RAN may potentially serve as the data termination points including the data consumer and the data provider. Hence, RAN3 should study the RAN-centric data collection function to solve the case where both the data producer and data consumer are within RAN domain.

Observation 3: RAN3 should study the RAN-centric data collection function to solve the case where both the data producer and data consumer are within RAN domain.

Before delving into 6G AI/ML discussion, we think the 5G-A AI/ML for PHY topic provides a good case to study. There was a lot of discussion on the termination points in different data collection procedure for beam management case. If the model is in the gNB-sided, the model training can be placed at gNB or OAM where the data collection can be gNB-centric and OAM-centric. The typical data content, data size, latency requirement, and data producer, consumer, transfer path for gNB-sided model is listed as following table based on [2].

Table 1. Typical data type for gNB-sided model (e.g. beam management)

| LCM procedure | Typical data content | Typical data size (per data sample) | Typical latency requirement | Producer | Consumer | Transfer path |
| --- | --- | --- | --- | --- | --- | --- |
| Training | L1-RSRPs and/or beam-IDs | ~ tens to hundreds bits | Relaxed | UE | gNB, OAM | UE->gNB, UE->gNB->OAM |
| Inference | L1-RSRPs, and Beam-IDs if needed, for Set B | ~ tens to hundreds bits | Time-critical | UE | gNB | UE->gNB |
| Monitoring | L1-RSRP(s) and/or beam-ID(s) | ~ tens to hundreds bits | Near-real-time | UE | gNB | UE->gNB |

From the table, it can be observed that data may be generated by the UE, consumed by the gNB or OAM for model training, and by the gNB for inference and monitoring. The typical data includes Layer 1 measurement results and beam ID(s). This use case is a typical case where data producer and data consumer are both located in the RAN side, and RAN should have the responsibility to collect and manage the data. The measurement data applied for beam management case is composed of layer1 measurements which is collected, managed and applied solely within RAN side. The core network could not encode the layer1 data, so it is unnecessary to involve the core network entity to collect the layer1 data. In light of above analysis, the RAN-centric data collection function is required without core network in the case that data producer and data consumer are both located in the RAN side.

Proposal 2: The RAN-centric data collection function is required to fulfil the new service requirement in RAN side without core network involvement.

Followed by above principle, the data consumer can initiate the data collection procedure directly to the data provider in each use case, which may result the duplicated data collection with regard to the collection for same type of data in different use cases. For example, the L1 beam level measurements used for high-performance model training are required for both AI/ML-based beam management case and AI/ML for mobility case. In order to avoid such fragmentation, a centralized RAN data collection function will be pursued in 6G which allows reusability of collected data to fulfil different AI use cases.

Proposal 3: A centralized RAN data collection function will be pursued in 6G which allows reusability of collected data to fulfil different AI use cases.

The introduction of RAN-centric data collection function can effectively avoid duplicate data collection, as data already gathered within the RAN domain can be accessed directly without re-initiating the collection procedure. To further enhance the data collection framework within RAN domain, the collected data can be stored in the RAN-centric data collection function acted as a common data repository that allows network entities to request and retrieve the data when needed. In this way, data consumers can access the stored data and data producers can provide data upon request, avoiding per–use-case or per–model-type data collection designs and ensuring scalability and future-proof support for new service cases.

Proposal 4: A RAN-centric data collection function can store data acted as a common data repository that allows network entities to request and retrieve the data when needed.

This RAN-centric data collection function provides a centralized mechanism to collect data for multiple use cases and to expose the data when required. Currently, there are two potential deployment options for this centralized function are illustrated below: integrated within the 6G base station or deployed independently from 6G base station. RAN3 needs to discuss and compare two options with analysing the potential impact after the RAN data collection function is determined to be introduced.

Option 1: Integrated within the 6G base station

Option 2: Deployed independently from 6G base station

From our perspective, the Option 2 for independent deployment is more suitable for operators which enables cross-vendor data collection and distribution compared to integrated within the base station. Besides, independent deployment from base stations can weaken the network vendors’ control over data, thereby enabling UE vendors and chipset vendors to rapidly deploy the AI ecosystem and minimizing negotiations on data ownership.

Considering that the collected data must remain manageable and controllable throughout its lifecycle, operators are better positioned to protect and manage the data before sharing it with other network, chipset, or UE vendors. This approach ensures that the operators maintain full control and governance over data usage and exposure. From operator’s perspective, the location of RAN-centric data collection function is more suitable to be deployed in a location isolated from the base station.

Proposal 5: RAN3 needs to discuss and compare two following options regarding the location of RAN data collection function:

- Integrated within the 6G base station;

- Deployed independently from 6G base station;

Proposal 6: From operator’s perspective, the location of RAN-centric data collection function is more suitable to be deployed in a location isolated from the base station.

# Conclusion

In this contribution, we take data collection function as a start point and reflects our perspective as operators on the RAN data collection function for 6G RAN. The following observation and proposal are listed below:

Observation 1: The RAN2's agreements gives RAN3 a good direction where the study on standardized data collection framework for AI/ML should be taken as baseline and the study on further requirements of other non-AI/ML use cases/services is to understand whether the common requirements exist across different use cases.

Observation 2: The data collection design should always be discussed simultaneously with AI/ML use cases since the architecture for the data collection provides the most fundamental support for the operation of AI/ML use cases.

Observation 3: RAN3 should study the RAN-centric data collection function to solve the case where both the data producer and data consumer are within RAN domain.

Proposal 1: RAN3 should start with studying AI-centric data collection framework.

Proposal 2: The RAN-centric data collection function is required to fulfil the new service requirement in RAN side without core network involvement.

Proposal 3: A centralized RAN data collection function will be pursued in 6G which allows reusability of collected data to fulfil different AI use cases.

Proposal 4: A RAN-centric data collection function can store data acted as a common data repository that allows network entities to request and retrieve the data when needed.

Proposal 5: RAN3 needs to discuss and compare two following options regarding the location of RAN data collection function:

- Integrated within the 6G base station;

- Deployed independently from 6G base station;

Proposal 6: From operator’s perspective, the location of RAN-centric data collection function is more suitable to be deployed in a location isolated from the base station.

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R1-2310681, Reply LS on Data Collection Requirements and Assumptions

# pCR to TR 38.760-3

### 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

The 6G RAN architecture support following functions:

-	RAN data collection function: The functionality to control data within the RAN;