3GPP TSG-RAN WG3 Meeting #130	R3-258630

Dallas, Texas, USA, November 17th – 21th, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	(TP to BL pCR to TR 38.765) Discussions for ISAC network architecture

Document for:	Other

# Introduction

Rel-20 ISAC study (5G Advanced) for RAN WGs was agreed at RAN#108 and further revised in RAN#109 [1], with the following objectives:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For the architectural aspects, the initial discussions on RAN3-129bis progressed as follows, with the first TP for the network logical architecture captured in TR 38.765 [2]:

Capture logical architecture for ISAC

RAN3 focus on sensing protocol in coordination with SA2

To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

# 7 Network architecture Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied. This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture. Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF. Figure 7.1 Logical architecture for ISAC This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF. Editor’s Note: The details of "Nx" interface need further discussion and decision.

Considering that SA2 has not concluded their study yet (only interim agreements so far), we think it could be premature for RAN3 to discuss sensing protocol aspects at this stage. Therefore, in this contribution, we raise several architectural aspects that can be discussed within RAN3 and propose the corresponding TP in Annex.

# Discussion

## CN-side entity in our ISAC logical architecture

Currently, the figure captured for our ISAC logical architecture [2] shows that gNB is directly interfaced with a SF (Sensing Function) through an interface called “Nx”, with a short description stating that it is independent of whether it is direct or via the AMF between the gNB and the SF.

While we understand that the intention was not to capture only the direct interfacing option, we think that directly referencing SF in our logical architecture may be inappropriate, as it implicitly fixes the interface endpoint as SF even though AMF may participate in certain procedures for ISAC operations (for which have not been concluded by SA2).

We believe that our architectural diagram should serve as a common basis for capturing and describing multiple options during this study. From the solutions captured in SA2 [3], SE (gNB) and SF (in 5GC) could be directly interfaced, or direct interface may only be setup for sensing measurement configuration or reporting purpose, or AMF may act in the middle for bridging between them while performing some discovery or selection of SE instead of SF. To cover all these possibilities while avoiding dependency with SA2, it would be more appropriate to represent this entity generically as a “Sensing CN node”, rather than a specific “SF” in our logical architecture.

This approach is aligned with what RAN3 did for Rel-19 A-IoT, where we captured the CN-side counterpart as “A-IoT CN node” (as defined in TS 38.300 by R2-2506490):

3.2 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2]. 2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18]. A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, i.e. BRID or DAA. Access occasion: a time-frequency resource for A-IoT device(s) to transmit A-IoT MSG1 (i.e., the Random ID message) during an A-IoT CBRA procedure. Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.18. A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF. A-IoT device: a device that supports A-IoT radio interface towards A-IoT reader, as defined in 16.x. A-IoT MSG1: first D2R message transmission in the A-IoT CBRA procedure, as defined in 16.x. A-IoT MSG2: R2D message in response to A-IoT MSG1 in the A-IoT CBRA procedure, as defined in 16.x. A-IoT reader: reader providing A-IoT protocol terminations towards the A-IoT device, as defined in 16.x. Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

By adopting a similar abstraction, we believe that RAN3 can effectively avoid dependency with SA2 while being open for diverse stage-3 designs (including protocols) to be evaluated under this study item.

Proposal 1: RAN3 to re-capture the entity in 5GC that SE (gNB) interacts with, by defining “Sensing CN node” with capturing the following definition (similar to “A-IoT CN node” in Rel-19):

- Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF.

## Other architectural discussion points

One other aspect is about terms and definitions of SE (gNB) and SF (residing at 5GC) in [3]:

## 3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Sensing Service Consumer: The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result. Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver. NOTE 1: In this release, the Sensing Entity is only referring to gNB. Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE 2: The Sensing Function cannot be a Sensing Entity. Sensing Service: Capability to collect and provide information about object and/or characteristics of the environment using radio signals. gNB-based Sensing: it indicates gNB acting as Sensing Entity. This doesn't imply any assumptions for the location of the sensing result generation. The terms defined in TS 22.137 [2] apply to this study: - Sensing assistance information. - Sensing contextual information. - Sensing result. - Target sensing service area. - Sensing receiver. - Sensing transmitter. Editor's note: The terminology will be further revised during the study work, if needed.

It is already stable that a SE can only be a gNB, and SF cannot be a SE. But it is still unclear whether a SF (normally assumed to reside in 5GC) can also be deployed at gNB.

RAN has already agreed not to study inter-gNB coordination in Rel-20. If the network architecture for gNB-based sensing allows a SF to be located in gNB (not acting as a SE), it may require inter-gNB coordination between the gNB hosting SF and another gNB acting as an SE. While it looks obvious that SF can only reside in 5GC, from RAN3 perspective, it would be good to agree that, in Rel-20, a SF can only be located in 5GC and capture this clearly in our TR.

Proposal 2: RAN3 agrees that, in Rel-20, Sensing Function (SF) shall not be located at gNB, as it may require inter-gNB coordination with another gNB acting as a Sensing Entity (SE).

Another aspect for discussion could be on whether our architecture design for overall signalling flow and interactions between gNB and 5GC should be different depending on the sensing type (e.g. mono-static vs bi-static) or the specific use case.

Currently, Rel-20 focuses on gNB-based mono-static sensing and targets the UAV use case. However, this scope was constrained mainly due to practical considerations and limited study timeframe. The gNB-based bi-static sensing, where sensing transmission and reception are executed by different gNBs, was excluded from RAN1 evaluation and may be discussed in RAN3 at a later phase only from architecture perspective. The inter-gNB coordination was also excluded due to its architectural and procedural complexity, especially given that the mono-static sensing alone presents significant evaluation workload, and also considering that the role of this study is to lay the feasibility groundwork for ISAC in future 6G standardization.

From similar reasons, among the various sensing use cases defined in SA1’s TR 22.837 [4], UAVs were selected as the representative (and “only”) use case for study under Rel-20 ISAC in RAN, considering its practical tractability, technical feasibility, and standardization efficiency:

- UAVs provide well-defined targets for sensing system design. Compared to humans or vehicles, which involves complex, multi-path characteristics, UAVs are relatively simple physical objects, enabling easier modelling, performance metric definition and calibration, thus making them well-suited for mono-static sensing.

- UAVs were already considered in Rel-19 study (FS_Sensing_NR [5]) for channel model calibration, making them readily usable in Rel-20 RAN1 simulations. Moreover, UAVs are closely tied to growing industrial applications (aerial surveillance, disaster response, and autonomous delivery, etc.), which ensures clear market demands.

Moreover, needless to say, “Sensing” is an integral component of 6G RAN design, and UAV represents one of the most promising use cases to be studied in 6G [6]. For 6G, UAV use cases are also being addressed in TR 22.870 [7] and referred as “Uncrewed Aerial Vehicles”, as part of SA1’s New Study on 6G Use Cases and Service Requirements (FS_6G-REQ, SP-241391).

Furthermore, from a sensing function point of view, regardless of use cases, the purpose and operational requirements can be categorized into four fundamental objectives: (1) Detection (2) Localization (3) Tracking (4) Classification, to which most sensing use case can be mapped onto.

Given these considerations, we believe that our design for overall signalling flow and interactions between gNB and 5GC for sensing is better to be forward compatible and applicable the same regardless of which sensing type is used or which use case is considered.

Proposal 3: RAN3 designs the ISAC network architecture and signalling interactions between gNB and 5GC in a unified way that is agnostic to sensing type (e.g. mono-static vs bi-static) or a specific use case.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 to re-capture the entity in 5GC that SE (gNB) interacts with, by defining “Sensing CN node” with capturing the following definition (similar to “A-IoT CN node” in Rel-19):

- Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF.

Proposal 2: RAN3 agrees that, in Rel-20, Sensing Function (SF) shall not be located at gNB, as it may require inter-gNB coordination with another gNB acting as a Sensing Entity (SE).

Proposal 3: RAN3 designs the ISAC network architecture and signalling interactions between gNB and 5GC in a unified way that is agnostic to sensing type (e.g. mono-static vs bi-static) or a specific use case.

Regarding Proposals 1 and 2, a text proposal for BL pCR to TR 38.765 [2] is provided in Annex.

# Reference

[1] RP-252819, New SI: Study on Integrated Sensing And Communication (ISAC) for NR (RAN1-led)

[2] R3-257332, “(pCR to TR 38.765) TR skeleton for Study on Integrated Sensing And Communication (ISAC) for NR”, China Telecom, Xiaomi

[3] TR 23.700-14, Study on Stage 2 for Integrated Sensing and Communication (Rel-20), SA2

[4] TR 22.837, Study on Integrated Sensing and Communication (Rel-19), SA1

[5] RP-242348, Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

[6] RP-252912, SID: Study on 6G Radio

[7] TR 22.870, Study on 6G Use Cases and Service Requirements (Rel-20), SA1

# Annex (TP to BL pCR to TR 38.765 [2])

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP RP-252912: “New SID: Study on 6G Radio”

[3]	3GPP TR 38.914: “Study on 6G Scenarios and Requirements”

[X]	3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”

# Definitions of terms, symbols and abbreviations

## 4.1			Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Sensing CN node: The core network node which the gNB connects to 5GC for sensing operation. More specifically, it is the SF (Sensing Function) in case the gNB directly connects with the SF, and it includes both AMF and SF in case the gNB connects with the SF via AMF.

Sensing Entity: defined in TR 23.700-14 [X].

Sensing Function: defined in TR 23.700-14 [X]. In this release, the Sensing Function cannot be hosted at gNB.

## 4.2			Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 4.3			Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the Sensing CN node.

Figure 7.1	Logical architecture for ISAC

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

# RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The Sensing CN node sends sensing request to the gNB.

2.	The gNB sends sensing response to the Sensing CN node.

3.	If requested in Step 1, the gNB sends sensing report to the Sensing CN node.