3GPP TSG-RAN WG3 Meeting #130	R3-258640

Dallas, US, 17th ~21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0503 | Rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to CLI mitigation in SBFD operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In Sub-band full duplex (SBFD) operation, for gNB-to-gNB CLI, when the gNB-DU reports gNB-to-gNB CLI related information of its serving cells to the gNB-CU, this behavior is independent from whether CLI is detected is not; - The gNB-CU just needs to forward gNB-to-gNB CLI related information received from neighboring gNBs to each concerned gNB-DU. For intra-CU, there is no need to do such forwarding, since gNB-CU will coordinate among its serving gNB-DUs. - The information concerning SRS resources potentially causing UE-to-UE CLI should come from neighbour gNBs. - Editorials |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To remove the condition of “if CLI is detected.” for the case of gNB-to-gNB CLI, when the gNB-DU reports gNB-to-gNB CLI related information of its serving cells to the gNB-CU; - To remove “from served gNB-DUs” for the case of gNB-to-gNB CLI, when the gNB-CU needs to forward gNB-to-gNB CLI related information to each concerned gNB-DU; - Add the description of “received from neighbour gNBs” when the gNB-CU signals to the gNB-DU information concerning SRS resources potentially causing UE-to-UE CLI. - Ocnfigurations => configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification texts for gNB-to-gNB CLI and UE-to-UE CLI mitigation remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmitted to RAN3#130 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

In Sub-band full duplex (SBFD) operation, gNB-to-gNB CLI and/or UE-to-UE CLI may also be present. In case of split gNB architecture, the gNB-DU reports gNB-to-gNB CLI related information of its serving cells to the gNB-CU. The gNB-CU forwards gNB-to-gNB CLI related information received from neighboring gNBs to each concerned gNB-DU. The gNB-DU should evaluate the received information and may mitigate interference if necessary.

For the case of detection of UE-to-UE CLI, a gNB-DU that has activated SBFD operation may provide SRS resource configuration to the gNB-CU. A gNB-DU serving potential victim UEs may indicate to a gNB-CU that SRS resource configurations of neighbour cells are needed. The gNB-CU may signal to the gNB-DU information received from neighbour gNBs concerning SRS resources potentially causing UE-to-UE CLI. For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.

<< End of Changes >>