3GPP TSG-RAN WG3 Meeting #130	R3-258645

Dallas, US, 17-21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MRO for S-CPAC and CHO with candidate SCG for 37.340 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For S-CPAC - If the conditional intra-SN PSCell change failure occurs in the last serving SN, besides the last serving SN, the initiating node is also needed to perform optimization on PSCell selection due to wrong candidate PSCell selection. The current description is not correct. However, this chapter is only for Conditional PSCell addition or change failure. There is no need to capture non conditional case. - The text “MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC or CHO with candidate SCG(s) in case of conditional mobility, if CPC whether it is MN initiated or SN initiated” miss the S-CPAC case. The detailed detection mechanisms are different between “CPA or MN initiated CPC or CHO with Candidate SCG(s)/ SN initiated CPC” and “MN/SN initiated S-CPAC”. There are descriped by two different paragraphs. For CHO with candidate SCG 1. For following text “The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC or CHO with candidate SCG(s) procedure”. For the case of SCG failure occurs shortly after successful CHO with candidate SCG, target MN as last serving node can detect CPC failure type and forward to the node which initiates CHO with candidate SCG(s). Therefore, it should be the last serving MN detect CPC failure type. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the SON function. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the description on intra-SN PSCell change failure. - Add the description on S-CPAC. - Clarify the last serving node carry out the failure type of CHO with candidate SCG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 descriptions of S-CPAC and CHO with candidate SCG are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration/CHO with Candidate SCG(s) configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

For Too late CPC execution and CPC/CPA execution to wrong PSCell, there are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC, or by the candidate or target MN provided CHO with Candidate SCG(s) configurations or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN/MN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or the candidate or target MN provided CHO with Candidate SCG(s) configuration or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. The MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC or CHO with candidate SCG(s) or S-CPAC in case of conditional mobility, if CPC/S-CPAC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPACprocedure. In case of CHO with candidate SCG(s), the detection mechanisms are carried out in the MN node that served the UE before the reported SCG failure:

-	Too Late CPC Execution: if CPC/S-CPAC/CHO with Candidate SCG(s) is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or CHO with Candidate SCG(s) if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For MN initiated S-CPAC, the MN performs the root cause analysis:

-	For initial S-CPAC failure during a MN initiated S-CPAC, the forwarding mechanism is the same as that for MN initiated CPC.

-	For a following S-CPC failure during an MN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, the MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

-	For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

-	For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>