3GPP TSG-RAN WG3 Meeting #129bis	R3-257280

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	9.2.5

Source:	Moderator (Ericsson)

Title:	Summary of offline discussion on CSI-RS Resource Configuration Release

Document for:	Discussion and Decision

# 0	Introduction

This contribution aims to discuss about the handling of CSI-RS resource configuration release in CB#12.

CB: # 12_R19Mob_CSI-RSRscConfigRelease

- Discuss whether LTM modification is supported for inter-CU LTM in Rel-19?

- No new procedure

(Ericsson - moderator)

# 1	For the Chair’s Notes

Capture the following questions in the meeting minutes and mark this topic TBC.

- Whether the modification of LTM configurations is allowed over network signalings?

- If yes, which configuration needs to be modified?

- When the LTM modification can be triggered? For example, during or after LTM preparation?  Is it possible to modify during LTM execution or in subsequent LTM?

- If there is any configuration that can be modified, which way is preferred?

  - Existing LTM Cancel procedure, and the source re-initiate the configuration procedure

  - New procedure?

# 2	Discussion

Option 1: RAN3 agrees that it is possible to modify LTM configurations, i.e., which will require the RRC reconfiguration to the UE, being triggered by the candidate network node. Details will be discussed case by case.

Option 2: send an LS to RAN1 and RAN2 to say that network signalings don’t support the modification of LTM configurations for inter-CU LTM.

A draft LS is in R3-257281.

## 2.1	Modification of LTM configurations

From Rel-18, RAN3 has agreed that the LTM configurations to be configured for the UE can be added, modified, and released, before cell switch. However, R3-257127 mentioned that,

LTM modification (neither source gNB initiated, nor candidate gNB initiated) was never discussed for inter-CU LTM without SCG case in the WI phase.

It is unclear what “LTM modification” is referring to. Is it only for CSI-RS resource configuration or any other LTM related configurations. In principle, UE can be reconfigured with the add/mod and release list, thus we need to discuss whether network design is able to support such.

In current TS 38.300, it says that the LTM Configuration Update message can be used to update the LTM configurations of candidate cells.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration, the LTM configuration ID mapping list and the LTM security information.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration. The candidate gNB may include the CSI report configuration for CSI acquisition of the candidate cell(s).

Q1: Companies are invited to share their opinions on whether the LTM configurations for the UE can be modified or released during preparation phase:

## 2.2	Specs impacts

If it is concluded that no modification to the LTM configurations will be supported, the UE will be unable to release the CSI-RS resource configuration. Then the stage-3 CRs and an LS to RAN2 will be required.

In case RAN3 agrees to align with the RAN2 design, a stage-2 CR should be prepared. Please find the drafts in the folder.

Q2: Whether LS reply is needed to RAN2

Two sets of CRs and LS are provided in the drafts folder.

- Set 1: an LS to RAN2, with endorsed stage-3 CRs to remove the release lists for CSI-RS resource configuration

- Set 2: stage-2 CR to add a note

Q3: Which set of CRs can be agreed

# 3 Conclusion