3GPP TSG-RAN WG3 Meeting #130	R3-258713

# Dallas, US, 17th – 21st Nov,2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1549 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Essential correction on inter-CU LTM in DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,Huawei, China Telecom, Nokia, Samsung, NEC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The new S-NG-RAN node to M-NG-RAN node Container IE inside LTM Candidate PSCell Change Information Required IE was introduced in LTM, therefore, the original S-NG-RAN node to M-NG-RAN node Container IE shall be ignored. - The presence of TCI States Configurations List IE and SSB information IE in LTM Candidate PSCell Prepared List IE are set as optional. However, those two IEs shall be mandatory for LTM. - The CSI-RS Report Configuration for Early CSI Acquisition IE is missing in S-NODE MODIFICATION REQUEST ACKNOWLEDGE, and corrects the typo of this IE name. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add semantics description for the original S-NG-RAN node to M-NG-RAN node Container IE to indicate it shall be ignored when LTM is configrued. - Change the presence of TCI States Configurations List IE and SSB information IE in LTM Candidate PSCell Prepared List IE from O to M. - Add CSI Report Configuration for Early CSI Acquisition IE in S-NODE MODIFICATION REQUEST ACKNOWLEDGE, and correct the IE name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTM in DC can not be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.6, 9.1.2.11, 9.2.3.243, 9.3.5 |  |  |  |  |  |  |  |  |
| .4 |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1 Re-sunmit to RAN3#130 V2 revised based on online discussion |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| ----------------skipped----------------- |  |  |  |  |  |  |
| LTM Candidate PSCell Information Update Acknowledge | O |  |  |  | YES | ignore |
| >LTM Candidate PSCell Prepared List | O |  | 9.2.3.243 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Report Configuration for Early CSI Acquisition | O | OCTET STRING | Includes the ltm-CSI-ReportConfig as defined in TS 38.331 [10]. |  |  | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present or the LTM Candidate PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. This IE shall be ignored if the Conditional PSCell Change Information Required IE or LTM Candidate PSCell Change Information Required IE o is present. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-NG-RAN node provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Source PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| LTM Candidate PSCell Change Information Required | O |  |  |  | YES | reject |
| >LTM Request Indication | M |  | ENUMERATED (request, ...) |  | – |  |
| >Multiple S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the candidate SN may prepare. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| >>>Suggested LTM Candidate PSCell List | O |  | 9.2.3.242 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

9.2.3.243	LTM Candidate PSCell Prepared List

This IE provides the information related to the prepared LTM candidate PSCell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM Candidate PSCell Prepared List |  | 1 |  |  |
| >LTM Candidate PSCell Prepared Item |  | 1 .. < maxnoofLTMCells> |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>LTM No Security Change ID | M |  | INTEGER (0.. 9, …) | Corresponds to the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10] |
| >>TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| >>SSB Information | M |  | 9.2.3.228 |  |
| >>Early Sync Information | O |  | 9.2.3.218 |  |
| >>LTM CFRA Resource Information | O |  | 9.2.3.232 |  |
| >>CSI-RS Resource Configuration for Layer 1 Measurements | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>CSI-RS Resource Configuration for Early CSI Acquisition | O |  | CSI-RS Resource Configuration 9.2.3.224 |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedReportIntervalMDT,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-LTEA2XUEPC5AggregateMaximumBitRate,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

-----------------skipped----------------

LTMPSCellInformation-UpdateReqAck ::= SEQUENCE {

lTM-CandidatePSCellPreparedList			LTM-PSCell-Prepared-List											OPTIONAL,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

cSI-RSReportConfigurationForEarlyCSIAcquisition		OCTET STRING											OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqAck-ExtIEs} }			OPTIONAL,

...

}

LTMPSCellInformation-UpdateReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-----------------skipped----------------

LTMPSCellInformation-ChangeRequired ::= SEQUENCE {

lTM-RequestIndication					LTM-RequestIndication,

multipleSNChangeRequired-List			MultipleCandidateSNChangeRequired-List,

cSI-ResourceConfiguration				CSIResourceConfiguration											OPTIONAL,

lTM-ConfigurationIDMappingList			LTMConfigurationIDMappingList										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMPSCellInformation-ChangeRequired-ExtIEs } }		OPTIONAL,

...

}

LTMPSCellInformation-ChangeRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

...

}

-----------------skipped----------------

LTM-PSCell-Prepared-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Prepared-Item

LTM-PSCell-Prepared-Item ::= SEQUENCE {

pscell-id											NR-CGI,

lTM-NoSecurityChangeID								LTM-NoSecurityChangeID,

tCI-StatesConfigurationsList						OCTET STRING,

sSBInformation										SSBInformation,

earlySyncInformation								EarlySyncInformation									OPTIONAL,

lTMCFRAResourceInformation							LTMCFRAResourceInformation 								OPTIONAL,

cSI-RSResourceConfigurationForLayer1Measurements	CSI-RSResourceConfiguration								OPTIONAL,

cSI-RSResourceConfigurationForEarlyCSIAcquisition	CSI-RSResourceConfiguration								OPTIONAL,

complete-CandidateConfigurationIndicator			ENUMERATED {complete, ...}								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTM-PSCell-Prepared-Item-ExtIEs} } 						OPTIONAL,

...

}

LTM-PSCell-Prepared-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}