3GPP TSG-RAN WG3 Meeting #130	R3-258834

# Dallas, US, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1353 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Nokia, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In NR Femto, the new GW Context Release Indication IE is introduced in UE context release request message, however the corresponding procedural text is missing. It is better to add procedural text for the new IE to make the specification clearer. - The Target RAN Node ID IE is introduced in NR Femto. In case of NG handover towards an NR Femto node connected to an NR Femto GW, to allow the NR Femto GW to identify the target NR Femto node, AMF includes the Global RAN Node ID of the target NR Femto node in the HANDOVER REQUEST message, which is specified in TS 38.300. Suggest to add the reference. - In the semantics description of GW Context Release Indication IE, it described that “This IE indicates to the AMF that the gNB has successfully performed an Xn HO for the UE to a target gNB.”. The main point is if receiving the indication, the NR Femto GW shall release the UE context. No need to specify this indication is sent due to which scenario / what the Femto GW consider about. Since the IE description is clearly enough, suggest to remove the semantics description of GW Context Release Indication IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 8.3.2.2, add corresponding procedural text for the new GW Context Release Indication IE. - In clause 8.4.2.2, add the reference TS 38.300 to the description for Target RAN Node ID IE. - In 9.3.1.274, remove the semantics description of the GW Context Release Indication IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2, 8.4.2.2, 9.3.1.274 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-256953 Rev 1: Remove the changes already endorsed by other document. Rev 2: Revision of R3-257225, resubmit to RAN3#130. Rev 3: Revision of R3-258041, Update coversheet (meeting info). Rev 4: Revision of R3-258706, Update coversheet.s |  |  |  |  |  |  |  |  |

----------------Start of Changes ---------------

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: UE context release request

The NG-RAN node controlling a UE-associated logical NG-connection initiates the procedure by sending a UE CONTEXT RELEASE REQUEST message towards the affected AMF.

The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value, e.g., "TXnRELOCOverall Expiry", "Redirection", for the requested UE-associated logical NG-connection release.

If the PDU Session Resource List IE is included in the UE CONTEXT RELEASE REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the GW Context Release Indication IE is included in the UE CONTEXT RELEASE REQUEST message, the AMF shall, if supported, consider that the UE context may be released as specified in TS 38.300 [8].

-------Skip unchanged---------

-----------------Next Change-------------------

#### 8.4.2.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the MBS Support Indicator IE is included in the Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message, the SMF shall, if supported, handle this information as specified in TS 23.247 [44].

If the AMF UE NGAP ID 2 IE is included in the HANDOVER REQUEST message, the target gNB shall, if supported, store this information in the UE context and use it for subsequent Xn handovers.

If the Target RAN Node ID IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for routing the HANDOVER REQUEST message, as specified in TS 38.300 [8].

If the ECN Marking or Congestion Information Reporting Status IE is included in the Handover Request Acknowledge Transfer IE, the SMF shall, if supported, use it to deduce if ECN marking at NG-RAN or ECN marking at UPF or congestion information reporting is active or not active as described in TS 23.501 [9].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-----------------Next Change-------------------

#### 9.3.1.274	GW Context Release Indication

This information element is set by the gNB to provide an indication that the AMF may release any resources related to the signalled NG UE context (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GW Context Release Indication | M |  | ENUMERATED (true, …) |  |

-----------------End of Changes ---------------