3GPP TSG-RAN WG4 Meeting #116bis                                   R4-2513022

# Prague, Czech Republic, October 13-17, 2025

Title:	Qualcomm views on 6G Spectrum

Source:	Qualcomm Incorporated

Agenda item:	8.6

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# 1.	Introduction

In this paper we discuss and propose 6GR Spectrum issues in accordance with the chair guidance in [3]. We discuss and provide input to regulatory survey, bands and band combinations work, frequency ranges and irregular bandwidths.

# 2.	Discussion

## 2.1	Spectrum Regulatory Survey for 6GR

In order to provide a comprehensive survey of recent and ongoing regulatory activities regarding spectrum for 6G, we will summarize three pillars, namely, 1) WRC-23 outcome regarding spectrum for International Mobile Telecommunications (IMT), 2) Ongoing WRC-27 work regarding IMT identification, and 3) Ongoing work on CEPT on upper 6GHz. In our view, it is still premature to derive conclusions on spectrum and its availability for 6G, but focusing on the above three pillars, it helps to understand future directions for 6G spectrum and its related components.

WRC-23 outcome

Among decisions in WRC-23, held in Abu Dhabi, WRC-23 identified spectrum for IMT, which will be crucial for expanding broadband connectivity and developing IMT mobile services. Main WRC-23 decisions regarding spectrum for IMT identification can be summarized as follows:

- Upper 6 GHz (6.425-7.125 GHz): IMT identification in region 1, with footnotes in region 2 and region 3 with technical conditions (i.e., EIRP mask to protect Fixed Satellite Services)

- 10-10.5 GHz: IMT identifications in some region 2 countries with technical conditions​

- 3.3-3.4 GHz: identified to IMT in several countries in region 1, expanding its reach, identified to IMT in region 2​.

- 3.6-3.8 GHz: identified to IMT in region 2 with primary mobile allocation in 3.6-3.8 GHz in region 1 and IMT identification in3.6-3.8 GHz in the Middle East and Africa​

- UHF bands: 600 MHz band identified for IMT in Middle East with secondary mobile allocation in Europe in 470-694 MHz and secondary mobile allocation in some African countries in 614-694 MHz​.

WRC-27 Studies

WRC-23 agreed a WRC-27 agenda item 1.7 related to IMT, where ITU-R is conducting preparatory studies with the main objective to study the sharing and compatibility and develop technical conditions for the use of IMT in the following three frequency ranges, 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz. RAN4 has already in Rel-19 study item (FS_NR_IMT_4400_7125_14800MHz) provided its response on the technical parameters (i.e., RF parameters and AAS modelling). The output of RAN4 has been documented in TR 38.922. In the meantime, work is ongoing in ITU-R (specifically Working Party 5D) to study the sharing of IMT services with other services in the abovementioned frequency ranges. The outcome of this agenda along with other IMT-related decisions will be decided in WRC-27.

CEPT work on upper 6GHz

Based on the outcome of WRC-23 regarding upper 6GHz, the European Commission issued a mandated “to study feasibility of and develop least restrictive harmonized technical conditions” for the 6425-7125 MHz frequency band”. The harmonization of this band is supposed to be finalized by July 2027.

Observation 2.1-1: There are several target candidate bands for 6G, including the upper 6 GHz and the WRC-27 bands currently investigated at ITU-R (i.e. 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz). The regulatory discussions for some of these bands are still ongoing.

## 2.2	Band/Band combination definition

### 2.2.1 	Bands for 6GR

In 4G to 5G shift, the new bands n77/n78 and n79 were essentially new frequency ranges for FR1 thus the band definition and RF requirement framework received a lot of attention during 5G SI. The same attention must be given for the study for 6G bands operating at new frequencies when regulatory status permits.

We estimate that eventually most 5G bands will be specified as 6G bands since same happened in 5G where most 4G bands were specified as 5G bands. The group should consider how to make this big effort as smooth as possible. For instance, the group should consider if there should be a dedicated request window during which the requests to define existing 5G band also as 6G band for first normative release should be placed to avoid new requests every Plenary cycle that would lead to impossible load management task for the RAN4 leadership. Bands not associated with band-specific NS values are likely straightforward to specify whereas bands with specific NS-values require more work.

One thing to consider are the naming conventions for 6G bands, including the 6G denominator– in 4G it was “B”, in 5G “n”, 6G is waiting for the denominator as well as the denominators for single carrier and intra-band CA.  It would be preferrable to conclude naming conventions earlier rather than later because they would be needed also in band combination studies.

In the first release of 5G, bandwidths only up to 20 MHz with a small number of exceptions where specified for FDD bands. In subsequent releases both Intra-band contiguous CA and wider single CC BWs were specified. We suggest specifying all relevant BWs from the beginning for each band to avoid the need of adding new BWs later and to avoid the need of Intra-band contiguous CA for cases which can be supported by single carrier.

Proposal 2.2.1-1: Study and conclude naming conventions for 6G bands including Single Carrier and Intra-band CA as priority

Proposal 2.2.1-2: Study the process how to migrate requirements of existing 5G bands to 6G band requirements to avoid inefficiencies during WI phase

Proposal 2.2.1-3: Account regulatory timelines and outcomes in studying the characteristics of 6G bands for new frequencies/regions

### 2.2.2 	Band combination definition and simplification for 6GR

Pending RAN/RAN1 decisions on which aggregation options are supported in addition to 6G Carrier Aggregation and also other RAN1 decisions regarding all detailed spectrum aggregation matters, the group should study if band combinations in 6G specification could be specified in RAT/Aggregation method agnostic way to avoid “duplicated” listing combinations for CA/Dual Connectivity/Dual Stack. We suggest this because having dedicated tables for different aggregation options would be very laborious eventually. The band combination definition and specification methods must be flexible enough to fully account the outcome of RAN1 work.

As one part of the study, RAN4 should consider whether the practice in 4G and 5G era, that was to define virtually all possible permutations for band combinations, is still suitable (which today is up to 6 FR1 bands with 6 or more CC’s in NR CA), or if some changes to that would be beneficial for instance one aspect in the RAN4 specification could be the treatment of Intra-band CA within a band. The group should seek a solution for a generic way to show intra-band CA support for a band within Inter-band combination. This alone would reduce the number of band combination count significantly as one would not need to separately list Inter-band combination with “A”, “B”, “C”, “2A”, 3A”, or whatever they are called, for constituent bands.

At least the following topics should be included in the study.

Proposal 2.2.2-1: Pending RAN/RAN1 outcomes on the spectrum aggregation options(s) and RAN1 outcomes on detailed spectrum aggregation matters, study how to specify band combinations in a simple way, striving to avoid duplicate band combination tables in case more than one of CA, Dual Connectivity, Dual Stack are supported

Proposal 2.2.2-2: Study whether the approach of specifying all permutations for band combinations separately up to the maximum number of bands and CC’s per operator is still suitable, or if some simplification in some form could be used

The TX and RX allowances, ∆TIB and ∆RIB, have been specified since LTE Rel-10. During the first years of CA they were important as the RF technology, especially in RFFE was still evolving so margins would have been too tight without these allowances. Another aspect was that these additional deltas somehow depicted the performance difference of CA UE vs non-CA UE. However, during the past years the relevance of these has decreased a lot, up to the level that many companies don’t even follow what ∆TIB and ∆RIB numbers are used for new band combinations in 3GPP, let alone internal link budgets. In addition, virtually every smartphone supports CA nowadays. For 6G, the RAN4 group should check whether these allowances are still needed or not in the specifications.  If they are still found necessary, some generic solution to define those allowances could be used. Our current view is that these are not needed anymore in 6G.

Proposal 2.2.2-3: Study if ∆TIB and ∆RIB are still needed, or if they could be discontinued in 6G. If one or both ∆TIB and ∆RIB  are still needed, study how to specify those using generic method

The group has made tremendous progress in simplifying MSD specification process over the past two years (HPUE LUT), but still there is room for further improvements. For instance, instead band combination specific PC3 MSD requirements there could be general MSD according to each MSD mechanism, using appropriate granularity. As an example, instead of having >>10 band combination specific MSD requirements for LB-MB harmonics, there could be one generic requirement for all LB-MB harmonics for each Power Class. Defining generic MSD’s for different aggressor mechanisms will have different level of challenge. While it would be probably relatively easy to agree generic requirements for UL Harmonics and Harmonic Mixing, and even for Inter-modulations, it might be very challenging if not impossible to define generic MSD’s for Cross-band cases since parametrization would be an ultimate challenge.

Now it is also perfect time to study which kind of MSD requirements are needed overall, one could easily reconsider if e.g. 1+1 RB type of IMD cases or triple-beat are necessary or not.

Proposal 2.2.2-4: Study how to specify REFSENS exceptions (MSD’s) for each Power Classes in more generic way, avoiding band combination specific requirements when possible

Proposal 2.2.2-5: Study which kind of MSD requirements are needed, including considerations whether to drop some of most extreme currently specified cases in 6G

Please note that we have also considered MSD framework in our 6G general and UE RF submission [4].

## 2.3	Frequency ranges

## The study on frequency ranges includes two parts: naming conventions and the actual frequencies supported by each frequency range. For the latter starting from low frequencies, one example way could be to consider extension of the range which was named FR1 in 5G by up to the limit where devices are still implemented in “FR1 style” i.e. using omnidirectional antennas. Another example way could be to consider ~6-8GHz as new FR, given that the band n104 which spans from 6.425GHz to 7.125GHz has not been deployed as 5G band but is very promising frequency for 6G. The baseline implementation for this range/these ranges is based on assumption that the conducted performance wan be verified from antenna connectors like in 5G FR1.

## It could be meaningful to maintain the frequency ranges set for FR2 as there the likely implementation assumption is beamforming. The range in between the “FR1 like” and “FR2” allows more than one baseline implementation (omnidirectional, directional) depending on the exact frequency and use case.

## In our view, there is no urgency to agree the exact frequency ranges and naming conventions especially because the regulatory work is on-going which will impact to applicable frequencies. Meanwhile, these aspects can be studied.

Observation 2.3-1: The study on frequency ranges includes two parts; naming conventions and the actual frequencies supported by each frequency range

Proposal 2.3-1: Study the naming conventions and boundaries for the frequency ranges accounting the frequencies used first time in 6G and the outcomes of the ongoing regulatory work

## 2.4	Irregular Bandwidths for 6GR

In 5G, the said irregular bandwidths are 3MHz, 6MHz, and 7MHz accounting developments up to Rel-20. Especially the agreement to specify 7MHz as first irregular BW between the existing BW’s (5MHz and 10MHz) took several years. There was a dedicated Rel-17 study how to handle irregular spectrum without clear conclusion, then after one release in between it was eventually agreed to be specified in Rel-19. All in all, it turned out to be challenging to add new irregular bandwidths. In our understanding RAN/RAN1 will consider the minimum supported BW for 6G, which RAN4 should obviously account as naturally RAN4 cannot consider any smaller BW’s than that.

For 6G, the concept/framework  for supporting irregular bandwidths, including the granularity and how the respective requirements are specified, should be carefully studied from the beginning.

Proposal 2.4-1: The concept/framework for supporting irregular spectrum holdings should be studied from the beginning of 6G.

# 3.	Conclusion

On Spectrum Regulatory Survey for 6GR we observed that

Observation 2.1-1: There are several target candidate bands for 6G, including the upper 6 GHz and the WRC-27 bands currently investigated at ITU-R (i.e. 4400–4800 MHz, 7125–8400 MHz, and 14.8–15.35 GHz). The regulatory discussions for some of these bands are still ongoing

About bands for 6GR Bands we made the following proposals

Proposal 2.2.1-1: Study and conclude naming conventions for 6G bands including Single Carrier and Intra-band CA as priority.

Proposal 2.2.1-2: Study the process to migrate requirements of existing 5G bands to 6G band requirements to avoid inefficiencies during WI phase.

Proposal 2.2.1-3: Account regulatory timelines and outcomes in studying the characteristics of 6G bands for new frequencies/regions.

For Band Combination Definition and Simplification for 6GR we proposed the following

Proposal 2.2.2-1: Pending RAN/RAN1 outcomes, study how to specify band combinations in a simple way, avoiding duplicate tables for CA, Dual Connectivity, Dual Stack.

Proposal 2.2.2-2: Study whether specifying all permutations for band combinations is still suitable, or if simplification is possible.

Proposal 2.2.2-3: Study if ∆TIB and ∆RIB are still needed, or if they could be discontinued in 6G. If one or both are needed, study how to specify them using a generic method.

Proposal 2.2.2-4: Study how to specify REFSENS exceptions (MSDs) for each Power Class in a more generic way, avoiding band-specific requirements.

Proposal 2.2.2-5: Study which kinds of MSD requirements are needed including whether to drop some of the most extreme currently specified cases in 6G.

For Frequency Range discussions we observed that

Observation 2.3-1: The study on frequency ranges includes two parts: naming conventions and the actual frequencies supported by each frequency range.

And then proposed that

Proposal 2.3-1: Study the naming conventions and boundaries for the frequency ranges, accounting for first-time 6G frequencies and ongoing regulatory outcomes.

For Irregular Bandwidths for 6GR we proposed

Proposal 2.4-1: Study the concept/framework for supporting irregular spectrum holdings from the beginning of 6G.

# 4. 	References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on 6G general and UE RF”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025