3GPP TSG-RAN WG4 Meeting #116bis	R4-2513025

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	Views on 6G SI demodulation topics

Agenda item:	8.8

Document for:	Discussion

Introduction

In this contribution we highlight our views on the 6G SI content for topics concerning RAN4 demodulation performance requirements. We propose several objectives to consider in this study, that we believe would benefit RAN4 specifications and the ecosystem in general for the era of 6GR.

Discussion

Study the foundational assumptions for 6G demodulation requirements

There are several foundational assumptions that have been made to create demodulation performance requirements in previous generations. We think that some of them should be re-thought, and some others need to be re-confirmed or re-aligned within the group.

Spatial channel model for MIMO features

Following the conclusion of the 5GA single user spatial channel model (SCM) study in Rel-19 [1] and the start of the corresponding work item in Rel-20 [2], RAN4 should use the rCDL channel model (or an evolution thereof) as the baseline for MIMO requirements in 6GR.	
Doing so will guarantee that in the field the UEs and NW will make a reasonably efficient use of MIMO features specified, e.g., providing appropriate channel state information to the network such that the network can make optimal use of the available MIMO layers. Reliable MIMO feature performance requirements need to be set under adequate test conditions that mimic MIMO effects, e.g., spatial effects, observed in deployment as much as possible. The legacy TDL channels do not exhibit these effects nor match the expected statistics of a measured channel.	
The rCDL SCM developed in 5GA was found to be suitable for this task, and it aligns with RAN1/2 assumptions during MIMO feature development. Additionally, with this increase in requirement quality, it shall also be possible to reduce the number of requirements overall, to guarantee a reliable demodulation and CSI reporting implementation performance for all NW/UE combinations in the field.

We note that non-MIMO features may not require requirement setting using SCMs. Hence, we propose:

6GR demodulation and CSI reporting performance requirements for MIMO features shall be set using the rCDL as baseline propagation condition. The overall number of requirements may be limited, when rCDL is used.

This is especially relevant for MIMO features with >1 layer being transmitted in the same time/freuqency resources. For single layer cases the legacy models can be sufficient for some goals and use cases (e.g., single layer PDCCH). However, a complete feature coverage using a SCM as baseline is preferred for comparability. Additionally, a limited number of TDL requirements for comparison with 5G may be considered.

Additionally, a limited number of TDL requirements for MIMO features with single and multi-layer transmission within the same time/frequency resources may be considered.

Spatial channel model for AIML use cases

This agenda item only covers non-AIML demod topics, but this quick section is covering our opinion to not make AIML provisions for the SCM in the demodulation topic, i.e., we see it in line with the guidance to not have AIML in the scope.

As discussed in 5GA, the SCM need additional consideration before they are applied to AIML use cases. Especially the need for random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM is needed for features involving prediction performance.

The rCDL SCM derived in 5G, can fulfil the needs of AIML use cases via application of the legacy beamsteering framework, or via mu_desired table evolution, or via direct DoA/DoD angle evolution. However, such extensions shall be handled by the features that use the channel model, and not by the demod-led study, to save time units.

AIML extensions to the SCM framework shall be studied by the AIML 6GR study, if needed.

SNR operating point range extension

Deployed UEs regularly see up to 35dB of receive SNR. However, in 5G we have applied a receive (soft) SNR limitation of 20dB to both UE and BS requirements. On the UE side this limit is non-obviously implemented, via a test equipment (TE) TxEVM assumption for impaired results derivation. However, the commonly used TxEVM values of “6% at QPSK, 6% at 16QAM, 6% at 64QAM, 3% at 256QAM, and 2.5% at 1024QAM” were grossly underestimating the true TxEVM performance of the TEs, as crest factor reduction is not employed in TEs, furthermore these values disregard that the TE TxEVM is independent from the used modulation order (assuming same power).	
Additionally, RAN4 demod requirements are traditionally decoupled from RF requirements and impairments, such as to not conflate demodulation and RF impact on the performance. Hence it is difficult to understand, why such TxEVM was considered in 5GNR [3].

The TxEVM performance of practical TEs is below 1.5% in FR1 without carrier aggregation, and independent of modulation order. Using overly conservative TxEVM values in impaired simulations to derive performance requirements, means the DUT will be subjected to a much better effective SNR in the test than in the simulations (at the baseband receiver and for upper SNR limits).

It is understood that:

SNR limitations were imposed by consideration of worst-case configuration for the test equipment, e.g., OTA setup (with noise power transmission amplification), full power, full resource allocation, full CA, max bandwidths, and then enforced also on the majority of non-limited configuration, e.g., conducted test without carrier aggregation and with limited RE time/frequency domain allocation

Ultimately, the direct (BS) and indirect (UE) limitation of SNR operating point ranges, means that 5GNR does not have requirements for high TPUT, high MCS, and other highly modem implementation stressing configurations. Furthermore, RAN4 demod has often chosen requirements configurations to lower the SNR operating point, that result in requirements outside the design operating points of the modems.

As such we propose

RAN4 shall abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

Handling of UE classifications and applicability

According to initial agreements and discussions in RAN1 [4], it is likely that UE classifications will be a concept in 6GR.

Pending further RAN1/2 decisions, UE classifications can be used to replace (or very much limit) usage of applicability rules, which have made the 5GNR requirements applicability incredibly difficult to follow and quite contentious; in 5GA a large portion of discussion time was spent heckling over legacy and new applicability rules.

It is not yet clear, whether there will be baseline features for all UE classifications or all UE classifications will have fully independent feature sets. Nevertheless, RAN4 should define demod performance requirements in line with UE classification definition rules, while keeping an eye on future expandability.	
In any case, by default all mandatory features per UE classification shall have mandatory demod requirement(s), and optional features should have mandatory requirements, if the feature is supported.

RAN4 to study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification.

Handling of feedback-less channels/signals

In 5GNR demod requirements have been plagued ever more by broadcast and feedback-less channels/signals being declared non-testable. For example, PBCH/SIB was the first one in Rel-15, then common search space PDCCH, PRACH Msg4, and parts of NTN, were to the long list of non-testable channels and signals; current discussions may further expand the list with the new CJT reporting signalling discussed in MIMO_ph5 and parts of LPWUS.	
In 6GR the number of feedback-less channels and signals may increase, especially due to power saving targets. As such RAN4 should study and agree to assume broadcast and feedback-less channels/signals to be testable.

One argument used in 5GNR to make the non-testable declaration was the lack of UE feedback to the system simulator (via 3GPP or other interfaces), so that the TE could not derive the performance indicators for the UE. There are many approaches to overcome this in 6GR, for example

- Test modes/test loops.

- BS testing style test harness.

- KPI derivation at DUT and feedback to TE/SS.

- MDT style interfaces.

- Indirect inference of performance indicators.

- Test UEs.

- Etc.

Seeing the number of potential solutions to the 5GNR perceived problem, we propose to:

RAN4 to assume broadcast and feedback-less channels/signals to be testable. RAN4 to recommend to RAN5 to define needed test solutions.

Performance requirements vs. functional requirements

In technical specifications, there are two types of requirements: Functional and non-functional, i.e., performance, requirements. The prior ones are defined to define support for specific use cases and functionalities. The latter ones are defining quantitative performance metrics to define “how well” a feature’s implementation needs to work or be acceptable for the whole system.	
It is our understanding that RAN4 defines minimum performance requirements to prevent a race to the bottom between NW and UE implementation, as well as, system configuration, due to inherent complexity vs. performance trade-offs in product implementations. Additionally, minimum performance requirements aid in NW planning, (NW/UE) troubleshooting, and (NW/UE) product development. Interoperability between networks and devices is guaranteed using performance requirements, and interoperability device test costs are reduced.

Over the course of 5GNR we have repeatedly heard statements like “we are only doing functional requirements in any case”, albeit that statements mentioning functional requirements are captured very rarely in meeting minutes (e.g., [5]) and usually elicit strong reactions from the chair of the session.	
Furthermore, various online exchanges (e.g., during Rel-16 NR-U) have evoked the notion that having RRM performance requirements specified is often enough to somehow functionally guarantee demodulation “performance”.	
Given our understanding about the reason for using non-functional performance requirements in RAN4 demodulation and CSI reporting requirements given above, we do not think that such statements have a place in RAN4.

We see it useful to re-discuss the mission of RAN4 demod and the goal of minimum performance requirements in the 6G SI, and to collectively re-affirm the mission of RAN4 DMD to produce performance requirements.

RAN4 to reaffirm the mission of RAN4 DMD to produce performance requirements, not functional requirements, and to refrain from seeing RRM performance requirements as “good enough” functional tests for demodulation performance.

There is a potential overlap between RRM reporting performance requirements and CSI reporting requirements (and TA demodulation requirements). RAN4 should also re-discuss the separation between RRM and Demod performance tasks. Often the groups seem to be distinguished via (e.g., timing or power) reporting accuracy in RRM versus reporting impact on demodulation performance in Demod. However, this causes some explicit overlap that RAN4 should either explicitly acknowledge as intended (and performance requirements in one domain not replacing the same in the other domain) or resolve the overlap via explicit splitting of responsibilities.

RAN4 to discuss appropriate performance requirement task separation between RRM and Demod when reports are involved.

Higher layer aspects in demodulation requirements

Current demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects. This leads the DUT to be tested at operating points that are not in line with deployment and do not match receiver implementation performance in the presence of deployment resource allocation and scheduling.
Additionally, this makes it impossible to predict and lower bound performance scaling in deployment environments.

5GNR demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects.

In the past (e.g., Rel-16/Rel-17 enhanced MIMO codebooks) the test equipment complexity was sometimes quoted as a limiting factor, but promised further studies never came to be.

There are two levels of “higher” layer deployment aspects that shall be taken into account in 6GR demodulation performance requirements:

- TE applies DUT feedback.	
E.g., RI, PMI, etc.

- TE takes decisions via known algorithms.	
E.g., SU/MU scheduling, dynamic resource allocation/slots, applying timing offset reports (CJT), OLLA, etc.

RAN4 to study inclusion of higher layer aspects in demodulation requirements via increased and dynamic application of DUT feedback in the TE.

RAN4 to study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., SU/MU scheduling, dynamic resource allocation/slots, applying timing offset reports (CJT), OLLA, etc.

5GNR ATP extension with OLLA

5GNR made a first foray into employing a known TE algorithm to include the outer loop into link adaption. This arguably does not constitute higher layer aspects but still drives towards more dynamic and deployment aligned performance requirements.

During the Rel-17 discussions about “Application Layer Throughput” (ATP) it was proposed to also include OLLA, when defining requirements for ATP. One of the main arguments to include OLLA was that OLLA is always implemented in the NW side, hence existing results without OLLA would not represent actual deployment scenarios.	
In addition, including OLLA would secure the UEs implementation also being able to handle situations where the UE requested CQI would not be set by the NW, if it does not result in approximately 10% BLER, which is only one target used for a deployment case.	
It can also be noted that UE is designed for certain operating point and should not be forced into non-optimal MCS/SNR combinations. Including OLLA in the simulations would partially correct transmissions towards the 10% (or any other agreed) BLER operation point as intended by the CQI definition.

In the previous WI, including OLLA was not ultimately postponed as:

- Including OLLA would require an agreement into the implementation to be used in the TE. RAN4 Demod has not done this before without it already being specified by RAN 1 (i.e. PMI and CQI requirements). It will take time to agree on the “behaviour” of the base station/TE for handling OLLA.

- Due to the expected complexity of aligning and defining OLLA for the TE there was no time to further study in the WI.

Including OLLA will provide more deployment aligned requirements. Further study is required to analyse and define a minimum performance procedure to implement OLLA in TE.

In RAN4#106, Nokia provided a proposal for a simplified model of OLLA (see [6]) based on academic publications, which we believe is a good starting point for discussing a 6G implementation in the TE. The same model shall then be used in simulation configuration enabling full simulation including OLLA implementation.

Study the impact of including OLLA in ATP requirements with relation to actual deployment. Compare results with existing ATP requirements defined without OLLA. Use proposed OLLA model from [R4-2300703] as starting point.

Study how a simple implementation model for OLLA can be defined. Target of the model is implementation in TE.

Other

Additional potential tasks for a 6GR demod-led study objective have been informally discussed in the past. Here we list those that we see as potentially interesting.

UE Baseline and advanced receivers

It may be beneficial to have both baseline (e.g., linear) and advanced (e.g., non-linear) receiver requirements from day 1, but only for devices with a low number of Rx branches (i.e., UEs). For devices with many Rx branches (e.g., BSs) the performance between linear and non-linear algorithms decreasing with the number of branches (assuming constant number of transmitted layers) and algorithmic complexity for R-ML is increased exponentially with the number of branches. 	
Potential intermediate massive MIMO receiver implementation may differ greatly between architectures and thus will not be able to be aligned in RAN4 demod.

RAN4 to consider having both baseline and advanced UE receiver requirements at day 1.

Conclusion

In this contribution, we have presented our views on the 6G SI content for topics concerning RAN4 demodulation performance requirements. We propose several objectives to consider in this study, which we believe would benefit RAN4 specifications and the ecosystem in general for the era of 6GR.	
The following proposals have been made:

== Spatial channel model for MIMO features

- 6GR demodulation and CSI reporting performance requirements for MIMO features shall be set using the rCDL as baseline propagation condition. The overall number of requirements may be limited, when rCDL is used.

Additionally, a limited number of TDL requirements for MIMO features with single and multi-layer transmission within the same time/frequency resources may be considered.

== Spatial channel model for AIML use cases

AIML extensions to the SCM framework shall be studied by the AIML 6GR study, if needed.

== SNR operating point range extension

- SNR limitations were imposed by consideration of worst-case configuration for the test equipment, e.g., OTA setup (with noise power transmission amplification), full power, full resource allocation, full CA, max bandwidths, and then enforced also on the majority of non-limited configuration, e.g., conducted test without carrier aggregation and with limited RE time/frequency domain allocation

RAN4 shall abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

== Handling of UE classifications and applicability

RAN4 to study demod requirement handling for UE classifications and agree on questions such as a baseline set of requirements for all devices vs. individual requirements for each UE classification.

== Handling of feedback-less channels/signals

RAN4 to assume broadcast and feedback-less channels/signals to be testable. RAN4 to recommend to RAN5 to define needed test solutions.

== Performance requirements vs. functional requirements

RAN4 to reaffirm the mission of RAN4 DMD to produce performance requirements, not functional requirements, and to refrain from seeing RRM performance requirements as “good enough” functional tests for demodulation performance.

RAN4 to discuss appropriate performance requirement task separation between RRM and Demod when reports are involved.

== Higher layer aspects in demodulation requirements

5GNR demodulation and CSI reporting requirements are often neglecting to dynamically react to DUT feedback and to consider scheduling aspects.

RAN4 to study inclusion of higher layer aspects in demodulation requirements via increased and dynamic application of DUT feedback in the TE.

RAN4 to study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., SU/MU scheduling, dynamic resource allocation/slots, applying timing offset reports (CJT), OLLA, etc.

== 5GNR ATP extension with OLLA

Including OLLA will provide more deployment aligned requirements. Further study is required to analyse and define a minimum performance procedure to implement OLLA in TE.

Study the impact of including OLLA in ATP requirements with relation to actual deployment. Compare results with existing ATP requirements defined without OLLA. Use proposed OLLA model from [R4-2300703] as starting point.

Study how a simple implementation model for OLLA can be defined. Target of the model is implementation in TE.

== UE Baseline and advanced receivers

RAN4 to consider having both baseline and advanced UE receiver requirements at day 1.

- References

- RP-252186 TR 38.753 v2.0.0, Nokia, RAN#109

- RP-252957 WID: NR demodulation performance: Phase 6, Apple, RAN4#109

- R4-2318350 Discussion on UE Demodulation for non-colocated FR1 intra-band NR-CA, Nokia, RAN4#109

- R1-2506618 FL summary#3 on overview of 6GR air interface, NTT Docomo, RAN1#122

- R4-1913101 RAN4 #92-Bis Meeting Report, ETSI MCC, RAN4#93

- R4-2300703 On Absolute physical layer throughput requirements with link adaptation, Nokia, RAN4#106.

Note that in 5GNR some hard-fought exceptions to the 20dB limit exist. For example, a 30dB SNR for the non-collocated work-item which required a large power difference between the two sources. Such exceptions required extensive discussion and constraints (in this case AWGN channel, or reduction of AWGN_offset on the BS side).	
Additionally other high SNR test cases are marked as non-testable in RAN5, especially for OTA.