3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513029

Prague, Czech Republic, 13 Oct 2025 - 17 Oct 2025

Source:	Charter Communications, Cable Labs

Title:	Adding 7 GHz Small Cells to 6G Study

Agenda item:	8.6 6G Study Items: Spectrum

Document for:	Agreement

Introduction

The 2023 World Radio Communication Conference (WRC-23, 20 November to 15 December 2023) approved agenda item 1.7 for the 2027 World Radio Communication Conference (WRC-27, Administrative Circular CA/270). ITU-R Working Party (WP) 5D is invited to conduct and compare the sharing and compatibility studies to ensure the protection of services to which the frequency band is allocated on a primary basis for these frequencies: 7125 – 8400 MHz.

In this contribution, we discuss our considerations for adding Small-Cell in the 7125 – 8400 band.

Discussion

Figure 1. Spectrum allocation in the 7125 – 8400 MHz band in the USA.

In the USA, the 7125 – 8400 MHz band is currently licensed to national security related links. The FCC spectrum allocation is shown in Figure 1. This band is occupied by military radar, satellites including both geostationary orbit (GSO) and non-GSO (NGSO), fixed service (FS), and earth exploration service (EES). Similarly, a sizeable portion of this band is occupied in many European countries by NATO links. Therefore, it is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G), instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available.

Proposal 1: The 7125 – 8400 MHz band is currently licensed to national security related links. It is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G), instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available, such as using power class 5 (PC5) for UE. High-power UMa BS could risk sharing and reduce available bandwidth.

The 7125 – 8400 MHz band will be considered by operators as microcells or femtocells. Outdoor pole-mount or strand-mount micro small-cell BSs and indoor BSs typically use non-AAS or a small-size array due to the cost and physical dimension limitations. Non-AAS parameters are of interest for the 7125 – 8400 MHz.

Proposal 2: Non-AAS and AAS with small array sizes (e.g., 2×2, 2×4, or 4×4) parameters are of interest for the 7125 – 8400 band for medium-range and local-area microcells. The BS antenna gain for non-AAS is 6 dBi with an omnidirectional pattern in the azimuth plane.

High-power macro-cells are supposed to provide good coverage (e.g., over 95% of the area). 7 GHz frequency signals have larger path loss, building entry loss (BEL), vegetation attenuation, etc. 7 GHz frequency signals typically have few multipath components. The propagation conditions are more challenging at these frequencies. Macro-cell deployment in the 7125 – 8400 band may not be feasible. RAN4 will need to study the feasibility.

Proposal 3: Macro-cell deployment in the 7125 – 8400 band may not be feasible due to large losses in these high frequencies. RAN4 will need to study the feasibility: what is the maximum urban macro (UMa) inter-site distance (ISD) to achieve 95% coverage (> -10 dB SINR) in both DL and UL by considering the aggregated co-channel interference from all the neighbouring Macro cells (CCI)? If the UMa ISD limit is too small, RAN4 shall suggest ITU-R WP 5D that these high-frequency bands are not applicable for the macro scenario.

Proposal 4: We propose to add Micro-Cells to Rel-20 to the current Macro-cell in the 7125 – 8400 band.

Conclusion

The following Observations and Proposals were made:

Proposal 1: The 7125 – 8400 MHz band is currently licensed to national security related links. It is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G), instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available, such as using power class 5 (PC5) for UE. High-power UMa BS could risk sharing and reduce available bandwidth.

Proposal 2: Non-AAS and AAS with small array sizes (e.g., 2×2, 2×4, or 4×4) parameters are of interest for the 7125 – 8400 band for medium-range and local-area microcells. The BS antenna gain for non-AAS is 6 dBi with an omnidirectional pattern in the azimuth plane.

Proposal 3: Macro-cell deployment in the 7125 – 8400 band may not be feasible due to large losses in these high frequencies. RAN4 will need to study the feasibility: what is the maximum urban macro (UMa) inter-site distance (ISD) to achieve 95% coverage (> -10 dB SINR) in both DL and UL by considering the aggregated co-channel interference from all the neighbouring Macro cells (CCI)? If the UMa ISD limit is too small, RAN4 shall suggest ITU-R WP 5D that these high-frequency bands are not applicable for the macro scenario.

Proposal 4: We propose to add Micro-Cells to Rel-20 to the current Macro-cell in the 7125 – 8400 band.

- References

- R4-2400333, Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D.

- RP-240765, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.

- R4-2405729, Technical parameters for the 7125 to 8400 MHz and 14800 to 15350 MHz IMT bands for sharing studies, CableLabs, Charter Communications, Cox Communications

- RP-252888 7GHz Sep plenary