3GPP TSG-RAN4 Meeting  #116bis	R4-2513035

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.3

Source:	Apple

Title:	On 6G system parameters

Document for:	Discussion

# 1	Introduction

At RAN#108, 6G WG SI was approved [1], with the following RAN4 objective:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

From the above objectives, and following RAN4 chair’s guidance in [2], RAN4 system parameters discussion would include the following scope:

In this contribution, we share our views on system parameters.

# 2	Discussion

## 2.1 Waveform

New waveforms are being studied in RAN1 for potential performance gains such as PAPR reduction, high scheduling flexibility, robustness against high doppler shift. Among those candidate waveforms, low PAPR waveforms are important to study because they help to increase UL coverage, which is one key target of 6G radio.

RAN4 plays a critical role in evaluating such low PAPR waveforms, since any realistic assessment of the gain would involve the consideration of PA models and compliance with RAN4 requirements. Therefore, while RAN1 is studying the candidate waveforms, RAN4 can focus on the evaluation methodology. When the time come to perform the evaluation, RAN4 is ready to start the work.

PA modeling

In NR, the PA model used in evaluation for below 6GHz is captured in [3]. For 6G, since the likely new spectrum is in the range of 7.125-8.4GHz, we can consider using the PA model as a starting point while considering if a more realistic PA model is available.

Signal spectral shaping method

Frequency domain or time domain shaping method should be considered. This can further refer to the RAN1 study.

RAN4 requirements including ACLR/SEM/IBE/EVM

As the 6G requirements are yet to be developed in R21, it is proposed to use 5G requirements as a starting point. If spectrum utilization (i.e., how many usable RBs in a channel bandwidth) needs to be assumed, 5G spectrum utilization can be used as starting point.

Net coverage gain

The net coverage gain can be defined as follows:

G = SNR + MPR,

Where SNR is the potential SNR degradation derived from link-level simulation given a BLER, and MPR is the power back-off gain due to reduced PAPR.

In addition, implementation complexity at both UE and BS, including BS receiver impact needs to be properly considered.

Need to coordinate with RAN1

Proposal 1: It is proposed to discuss and agree on waveform evaluation methodology and assumption in preparation for the low PAPA waveform study.

## 2.2 Modulation

We propose to consider the following as a starting point for 6G:

- UL: up to 256 QAM

- DL: up to 1024QAM

For other modulation schemes including higher-order QAM, RAN4 needs to investigate the required EVM and how implementation, such as PA linearity and phase noise, can meet such EVM requirements, from both BS and UE point of view.

Proposal 2: It is proposed to consider the following modulation as a starting point for 6G:

- UL: up to 256 QAM

- DL: up to 1024QAM

## 2.3 CBW (channel bandwidth) including irregular CBW

Maximum CBW

Looking at existing FR1 spectrum and the potential new 6G spectrum around 7GHz (i.e., up to 8.4GHz), the most likely contiguous spectrum one operator may have is 200MHz. Compared to 5G where the max. CBW of 100MHz is specified for FR1, this means the max. CBW is doubled, which is aligned with the expectation that a new G may increase CBW to meet the ever-increasing data rate. Therefore, it is reasonable to consider 200MHz as the max. CBW that both BS and UE should support.

Proposal 3: For 6G, maximum CBW of 200MHz can be considered, which both BS and UE should support.

There is some proposal to support 400MHz CBW as two operators may pool their spectrum together for RAN sharing. In this case, instead of specifying 400MHz CBW that both BS and UE should support, the following two alternative solutions can be considered:

- Alternative solution 1: 200 + 200 MHz CA. In our observation, there is no significant performance difference between 400MHz CBW vs. 200+200MHz intra-band contiguous CA.

  - Spectrum utilization (SU): Since SU of 100MHz in 5G is already very high at 98.28%, and it is likely the SU of 200MHz and 400MHz is not going to be much higher than 98.28%, the GB difference between 400MHz and 2x200MHz is going to be very small (<1%).

  - Assuming all CA enhancements such as SSB-less operation, cross-carrier scheduling, and Multi-cell PDSCH/PUSCH scheduling with a single DCI, etc. are deployed in the network and supported by UE, the overhead/complexity associated with CA is not much higher than that of single carrier

- Alternative solution 2: 6G introduces BS-only 400MHz CBW, i.e., 400MHz is only specified for BS, but not for UE. In real deployment, the chance of a UE getting scheduled with the entire 400MHz bandwidth is slim.

Proposal 4: In case where 400MHz contiguous spectrum is available, the following two solutions can be considered:

- Alternative solution 1: 200 + 200 MHz CA

- Alternative solution 2: 6G introduces BS-only 400MHz CBW, i.e., 400MHz is only specified for BS, but not for UE.

Minimum CBW

The minimum CBW mainly depends on operators’ spectrum holding. In addition, 6G needs to consider the support of IoT devices, which are expected to be low cost and low complexity. Therefore, it is proposed to consider 3MHz or 5MHz.

Proposal 5: It is proposed to consider 3MHz or 5MHz for minimum CBW.

CBW granularity

For CBW sizes under 100MHz, the current granularity in 5G FR1 is a good starting point, i.e., multiples of 5MHz/10MHz.

If the max. CBW is 200MHz, we need to decide the granularity between 100MHz and 200MHz. In this case, it is better to start from operator’s spectrum holdings, i.e., what is the size of spectrum operators may have. The aim is to reduce the number of CBWs being specified. Therefore, inputs from operators on their 6G spectrum holdings are encouraged.

It is also acknowledged that the discussion of supporting irregular CBW may impact how many regular CBWs, i.e., in multiples of 5 or 10MHz, may need to be defined in the end.

Proposal 6: In terms of CBW granularity, or how many CBWs need to be defined, the current granularity in 5G FR1 can be a starting point for up to 100MHz where the need for each channel bandwidth shall be further assessed based on operators’ deployment plan for 6G. For CBWs between 100MHz and 200MHz, it is recommended to look at operators’ spectrum holding first. Furthermore, the discussion on support of irregular CBW may also impact how many regular CBWs, i.e., in multiples of 5 or 10MHz, may need to be defined in the end.

Support of irregular CBW

Support of irregular CBW is part of the SI scope. That said, we propose to set a limit on the size of irregular CBW to be supported, say 10 or 20MHz, the reason being

- While operators may hold irregular spectrum, i.e., spectrum not multiples of 5 or 10MHz, the most common case is such spectrum is narrow, not wide.

- In case there is a wide irregular spectrum, CA of a regular CBW and irregular CBW may help utilize the full spectrum.

- While 6G SSB is to be designed, considering the minimum CBW of 3MHz or 5MHz, SSB bandwidth is not expected to be much different from 5G. Therefore, the 5G solution of supporting irregular CBW, e.g., the overlapping carrier from the network perspective assuming one SSB can be fit in the overlapped spectrum, can work from wide irregular CBW.

Having said the above, we are also open to studying new solutions to support irregular CBW because they may lead to ways to reduce the number of CBWs specified in 6G, which is in addition to the original goal of helping operators to fully utilize their irregular spectrum.

Proposal 7: To support irregular CBW, it is proposed to

- Set a limit on the size of irregular CBW to be supported, say 10 or 20MHz.

- Use 5G solutions as a starting point

- Study new solutions that may help reduce the number of CBWs to be specified in 6G.

## 2.4 FFT size and numerology

FFT size

To support larger CBW such as 200MHz in 6G, it is natural to consider a larger FFT size than 5G. At the same time, reasonable implementation complexity and cost needs to be considered. Therefore, it is proposed to increase FFT size from 4K in 5G to 8K in 6G.

Proposal 8: It is proposed to support 8K FFT in 6G.

Numerology

As stated in the SID, 6G is aimed to “avoiding multiple numerologies for the same band / sub-range”. Based on the 5G commercial deployments where only single numerology is used for a band, it is sensible to specify single numerology on a band.

Furthermore, although ECP is specified in 5G for 60kHz SCS, it has not been deployed. As a result, the support of ECP should be removed in 6G.

Specifically, we propose to have the following SCS and CP length for 6G in Table 1:

Table 1: Numerology for different frequency ranges

| Frequency range | Numerology |
| --- | --- |
| FR1 (up to 7.125GHz) and new spectrum in ~7GHz (FDD and TDD) | FDD bands: 15kHz SCS, NCP TDD bands: 30kHz SCS, NCP |
| 12 – 15GHz (TDD) | Deprioritized as the spectrum availability is unclear |
| FR2-1 (TDD) | 120kHz SCS, NCP |

Proposal 9: It is proposed to have the SCS and CP length for 6G as shown in Table 1.

As a result, 8K FFT and 30kHz SCS can be used to support 200MHz CBW for spectrum up to 8.4GHz.

## 2.5 Channel raster and synchronization raster

Channel raster

When channel raster was being discussed at the beginning of 5G, the need was questioned because a UE no longer needs it for initial access. Instead, the UE can use synchronization raster to identify the RF channel position in frequency domain. In the end, the main argument for the need is to identify a channel position in UE testing.

At the same time, there may be a need to indicate the absolute frequency in RRC signaling, i.e., through ARFCN. For example, the 5G ARFCN is defined as follows:

Therefore, we propose to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync. raster points.

Proposal 10: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync. raster points.

Synchronization raster

In 6G, there are proposals to increase SSB periodicity for energy saving by the network. Therefore, the need for reduced sync. raster points to reduce UE cell search complexity and enable fast cell search becomes even stronger. While any detailed sync. raster design would depend on SSB bandwidth, minimum CBW, and channel spectrum utilization, RAN4 can agree on the goal of reduced sync. raster points.

Proposal 11: It is proposed to study synchronization raster with a goal to reduce UE cell search complexity and enable fast cell search.

## 2.6 Device types

6G is expected to support several difference device types including smartphones, wearables, FWA/CPEs, and XR. Clearly, those device types have different form factors and different RF and BB capabilities.

Furthermore, it is worth noting that 5G NR did not specify different device types on day one and had to add RedCap device support in R17. This approach is not preferred as anything after day 1 may have less opportunity to be supported.

Therefore, it is necessary to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

We would also like to note that there is a need to further differentiate within each device type given the different form factors, implementation complexity and costs, etc.

As for the parameters used to define device types, we propose to consider the following:

- Number of TX/RX

- Min. and max. channel bandwidth

- FFS: Power classes

Proposal 12: It is proposed to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

By considering the following parameters:

- Number of TX/RX

- Min. and max. channel bandwidth

- FFS: Power classes

# 3	Conclusions

In this contribution, we discuss 6G system parameters and make the following proposals.

Proposal 1: It is proposed to discuss and agree on waveform evaluation methodology and assumption in preparation for the low PAPA waveform study.

Proposal 2: It is proposed to consider the following modulation as a starting point for 6G:

- UL: up to 256 QAM

- DL: up to 1024QAM

Proposal 3: For 6G, maximum CBW of 200MHz can be considered, which both BS and UE should support.

Proposal 4: In case where 400MHz contiguous spectrum is available, the following two solutions can be considered:

- Alternative solution 1: 200 + 200 MHz CA

- Alternative solution 2: 6G introduces BS-only 400MHz CBW, i.e., 400MHz is only specified for BS, but not for UE.

Proposal 5: It is proposed to consider 3MHz or 5MHz for minimum CBW.

Proposal 6: In terms of CBW granularity, or how many CBWs need to be defined, the current granularity in 5G FR1 can be a starting point for up to 100MHz where the need for each channel bandwidth shall be further assessed based on operators’ deployment plan for 6G. For CBWs between 100MHz and 200MHz, it is recommended to look at operators’ spectrum holding first. Furthermore, the discussion on support of irregular CBW may also impact how many regular CBWs, i.e., in multiples of 5 or 10MHz, may need to be defined in the end.

Proposal 7: To support irregular CBW, it is proposed to

- Set a limit on the size of irregular CBW to be supported, say 10 or 20MHz.

- Use 5G solutions as a starting point

- Study new solutions that may help reduce the number of CBWs to be specified in 6G.

Proposal 8: It is proposed to support 8K FFT in 6G.

Proposal 9: It is proposed to have the SCS and CP length for 6G as shown in Table 1.

Proposal 10: It is proposed to study the need of specifying channel raster in 6G. If a need is identified, we can consider specifying 5kHz raster points instead of 100kHz or SCS (15/30kHz) for FR1 to increase channel placement flexibility and to avoid too many sync. raster points.

Proposal 11: It is proposed to study synchronization raster with a goal to reduce UE cell search complexity and enable fast cell search.

Proposal 12: It is proposed to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

By considering the following parameters:

- Number of TX/RX

- Min. and max. channel bandwidth

- FFS: Power classes

# 4	References

RP‑251881, "New SID: Study on 6G Radio," NTT DOCOMO (Moderator)

R4-2511652, "Plan on RAN4 6G discussion and meeting arrangement in Rel-20," RAN4 Chair (Apple)

R4-164542, "Response LS on realistic power amplifier model for NR waveform evaluation," RAN4