3GPP TSG-RAN WG4 Meeting #116 bis		   					R4-2513039

Prague, Czech, 13th – 17th Oct, 2025

Agenda Item:	8.7

Source:	MediaTek Inc.

Title:	Views on 6G RRM

Document for:	Discussion

# Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G RRM requirements with the focus on the following aspects:

- MG framework

- Fast RF retuning/interruption-free

- Mobility and HO interruption reduction

- UE Tx timing

- Scell activation, and spectrum aggregation

- Measurement capability

- Inter-RAT sync

# MG framework

This section provides discussion on how measurement gaps are handled in NR (New Radio), especially on how the RRM requirements have evolved from Rel-15 to Rel-19 and how does that pave the way to 6G systems.

The measurement process usually begins when the serving cell’s RSRP measurement drops below a certain threshold—meaning the UE is no longer in the cell centre. At this point, the network configures the UE with measurement objects (MOs) so it can measure neighbouring cells. These MOs refer to reference signals in target bands, which may belong to intra-frequency, inter-frequency, or inter-RAT cells relative to the serving cell. Generally, a UE requires a measurement gap (MG) to measure reference signals (indicated by the MOs) on non-serving carriers, e.g., on a target cell operating in a band outside its serving cell’s band combination. If the UE informs the network in advance about which bands it can measure with or without a gap, based on this information the network can configure the UE with the MG configuration to enable these measurements. Throughout 5G NR development, multiple measurement gap related features where introduced, however, not all these features were successfully deployed in real field. In Rel-15/16 NR, two types of measurement gaps (Type 1 MG) were defined: per-UE and per-FR measurement gaps. In Rel-17, three enhancements were introduced: (i) concurrent MG (Type 2 MG); (ii) Pre-configured MG patterns (Pre-MG); and network-controlled small gap (NCSG). Rel-18 defined RRM requirements for the NeedForGap (NFG) feature, and Rel-19 introduced further work on measurement gap cancellation in the XR work item. A high-level summary of these features is given below:

Table 1 Summary of Gap features

| MG features | Details/pros/cons | Whether to consider this in 6G SI |
| --- | --- | --- |
| Type 1 MG | Pros: - gap location is known to both NW and UE - easy implementation from NW and UE Cons: - MGL is too long - Impacts all UL/DL data carriers | MG might be necessary for some scenarios, yet, the conventional MG impact the UL/DL scheduling and hence further study is needed on how to enhance this in 6G systems. Suggestion: include Type 1 MG in 6G SI. |
| Type 2 MG (concurrent-MG) | Pros: - gap location is known to both NW and UE - flexible configurations for NW Cons: - MGL is too long - Impacts all UL/DL data carriers - Complex MG collision handling | The con-current gap is important to improve NW configuration for MGs flexibility. Hence, RAN4 to further study whether this is needed from initial 6G releases. Suggestion: include Type 2 MG in 6G SI with simplifications, e.g., on the collision handling. |
| Pre-MG | Pros: - gap location is known to both NW and UE - un-needed MG occasions are cancelled Cons: - MGL is too long - Impacts all UL/DL data carriers - Limited benefits to intra-freq cells meas. - Does not work as long as one MO requires MG | This feature is associated with BWP switching and given it is not clear whether we have the same concept of BWP in 6G then there is no need to include this in 6G SI. Suggestion: not to discuss Pre-MG in 6G SI. |
| NCSG | Pros: - gap location is known to both NW and UE - UE can be scheduled during ML Cons: - Interruption (VIL) is still too long - Working assumption with an idle RF chain | This feature is beneficial compared to using gaps in all scenarios. Yet, further study on how to reduce the impact of interruption is needed. Consider this feature for 6G SI. Suggestion: include Gapless with/without interruption in 6G SI. |
| NFG | Pros: - UE might avoid interrupting scheduled slots - UE can be scheduled during meas. Cons: - Interruption is too long - Interruption location is un-known to NW - Working assumption with an idle RF chain | This feature is beneficial compared to using gaps in all scenarios. Yet, further study on how to reduce the impact of interruption is needed. Consider this feature for 6G SI. Suggestion: include Gapless with/without interruption in 6G SI. |
| MG cancellation | Pros: - Allowing more slots for data scheduling - Dynamic MG cancellation based on data traffic Cons: - Measurement delay becomes too long - Increase UE complexity | This feature is dependent on data traffic. Besides, the necessity of such feature is dependent also on how an MG occasion impact the data scheduling and given that the MG occasion structure is still to be discussed. A foundation of MG framework without cancellation is a prerequisite for this figure. Then, this feature can be left for future 6G releases. Suggestion: not to discuss MG cancellation in 6G SI. |

Based on the analysis provided in the table above, this document focuses on discussing: (i) Conventional MG (Type 1); (ii) Gapless with/without interruption; and (iii) general issues.

Conventional measurement gaps: In 5G NR spec, measurement gaps are defined in terms of measurement gap length (MGL) and measurement gap repetition period (MGRP), as in Table 9.1.2-1 [38.133]. According to the working assumptions in RRM group, the MGL consists of measurement length (ML) plus two parts of RF retuning time (RRT), as depicted in the figure below. Typical measurement length in FR1 can be either 5 ms or 2 ms, while the RF retuning is 0.5 ms at the sides of ML, resulting in 6 ms and 3 ms MGL. The mandatory gap patterns are configurations 0, and 1, which are MGL = 6ms, MGRP = 40ms/80ms, resulting in up to 15% interruption of data transmission and reception. Furthermore, the use of a measurement gap affects all uplink (UL) and downlink (DL) carriers, temporarily blocking data transmission and reception at the UE. As a result, the UE can only perform measurements on the specific target band or carrier defined by the measurement object. Therefore, the 15% of data interruption can impact system performance (e.g. lower throughput), also, a UE might not be able to achieve certain service requirements, such as latency.

Observation 1: Measurement gaps are typically responsible for a 15% interruption of data transmission and reception (e.g. MGL = 6ms, MGRP = 40ms), which impact user experience in terms of throughput and latency.

Proposal 1: Investigate methods to reduce the use of measurement gaps in 6G systems.

ML

Time

MGRP

MGL

RRT

MG

ML

RRT

Tx/Rx data only

Tx/Rx data only

No data 
Tx/Rx

Figure 1: representation of measurement gaps

In previous generations of mobile systems, such as LTE and NR, a User Equipment (UE) can indicate whether a MG is required to perform measurements on a target frequency band. However, the specifications do not explicitly state why a measurement gap is needed; as the spec only allows the UE to request one. It is generally understood within the industry that the UE requests an MG when it lacks sufficient RF chains to simultaneously measure frequencies outside its current operating carriers. In such cases, RF retuning is necessary to measure carriers on different frequency bands.

Additionally, the UE may require a measurement gap when the measurement object refers to reference signals in a target band that lies outside the operating band combination specified in 3GPP TS 38.101. In these scenarios, the need for a measurement gap arises due to desense (i.e., self-interference) between the serving cell bands and the target cell band, which contains the reference signals necessary for measurement. The desense, mentioned earlier, refers to interference caused by the UE's own transmissions or receptions, which can degrade measurement accuracy.

Observation 2: In LTE/NR, a UE might report a need for a measurement gap when RF retuning is necessary to measure frequency carriers outside its operating carriers.

Observation 3: In LTE/NR, a UE might report a need for a measurement gap due to a desense (MSD) issue between the target band, containing the SSB to be measured, and the serving bands.

Proposal 2: Identify and address the reasons behind measurement gaps in previous generations of mobile systems and how to handle these issues in 6G systems.

Gapless solutions: Gapless measurement solutions with/without interruption have been specified, allowing the UE to perform measurements without requiring conventional measurement gaps. These gapless solutions were first considered in 5G NR starting from Rel-16. On one hand, the NeedForGap solution was specified in terms of signalling since Rel-16, but the specifications did not fully address the RRM requirements until Rel-18. This created ambiguity regarding expected UE behaviour, particularly in terms of how the UE should avoid the impact of RF retuning during measurements. On the other hand, the NCSG solution was introduced in both signalling and RRM requirements in Rel-17. Although the specifications for NCSG were comprehensive, the late introduction of this feature made it challenging for companies to implement and deploy the solution in their products.

Observation 4: In 5G NR Rel-16, NeedForGap gapless solution, was specified only in terms of signalling, and RRM requirements were not included until Rel-18. This left ambiguity about the expectations for UE behaviour, such as how to avoid the impact of RF retuning.

Observation 5: In 5G NR Rel-17, NCSG gapless solution, was specified fully in terms of signalling and RRM requirements. However, given that the feature was introduced in the third release of 5G NR, this feature has not been implemented in real field.

Proposal 3: The gapless capability to be specified in the 6G Day-1 spec, including both signalling and RRM requirements.

General issues:

Issue 1: Available RF chain: As mentioned earlier in this document that the use of a measurement gap affects all UL and DL carriers, temporarily blocking data transmission and reception at the UE. As a result, during this measurement gap, the UE is only required to perform measurements on the target band or carrier specified by the measurement object. Nevertheless, from an implementation perspective, a single RF chain is typically required for the UE to perform measurements, with this RF chain operating as a downlink receiver for a certain carrier. Also, it should be noted that UEs that support carrier aggregation are equipped with multiple RF chains, allowing simultaneous transmission and reception on multiple carriers. Therefore, it is necessary to study whether it is truly required to interrupt all UL and DL carriers during the MGL. In other words, further investigation is needed to determine if the measurement gap can be limited to a single carrier, rather than affecting all carriers.

Observation 6: In 5G NR, during measurement gaps length, a UE interrupts uplink and downlink data transmission and reception across all serving cell carriers to allow measurement of a target SSB of a target cell.

Proposal 4: Further investigate how to limit the measurement gap impact to a single carrier or specific number of carriers.

Furthermore, as mentioned earlier that gapless solutions are based on the availability of un-used RF chain, hence the RRM discussion shall differentiate UE behaviour on whether the UE has available RF chain.

Observation 7: From a UE implementation perspective, whether all carriers should be interrupted depends on the availability of an idle RF chain to perform measurements on the target cell.

Observation 8: Measurement gap and gapless features are based on the RF chain availability at the UE side.

Proposal 5: When defining measurement gap/interruption requirements and reporting signalling, different UE behaviours should be specified based on the availability of an idle RF chain.

Issue 2: RF retuning time: In 5G NR, the working assumption values for RF retuning were 0.5 ms for FR1 and 0.25 ms for FR2. However, this number was based on initial NR implementation and have never been optimised throughout NR enhancements. Besides, from UE implementation perspective, it is expected that the RF retuning is much faster than that working assumption values. In NR, some Wis already introduced way faster RF retuning time for Rx or Tx switching (e.g., 35 us or 140 us). Therefore, further study shall be conducted to find new working assumption that are closer to real values based on latest UE implementation advancements.

Observation 9: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 10: With latest UE advancements, a UE can achieve faster RRT compared to that specified in the working assumption in 5G NR.

Proposal 6: RAN4 (RRM/RF) to further investigate the possibility of reducing the RF retuning time for measurement gaps/gapless.

Issue 3: Non-colliding multiple GAPs: In real field scenarios, different target cells are expected to require distinct measurement configurations, and each measurement is therefore associated with a specific MO. Associating different MO to different MG configurations has the potential of reducing unnecessary measurement gap occasions and increase network configuration flexibility. However, although the UE can be configured with different measurement settings via multiple MOs, these MOs are still constrained by the number of configured MG or NCSG in 5G NR. To address the issue, in 5G NR concurrent gap solutions have been introduced to address this limitation, such as concurrent gaps with NCSG. A key issue with concurrent gaps is that, while signalling allows for multiple gap configurations, the RRM requirements restrict the UE to a maximum of two configured gaps. This restriction is primarily due to the potential for collisions between the configured gaps, which limits the system to only two MG configurations. Furthermore, the concept of associating specific MOs with particular types of gap or gapless mechanisms has not been fully utilized, again mainly because of the two-gap limitation. Therefore, for 6G systems, it is necessary for RRM to further investigate how to associate different MOs with different gap or gapless mechanism. This would enable multiple gap configurations, allowing MOs for different carriers to be treated independently in terms of measurement requirements.

Observation 11: From a system operation perspective, applying different measurement gap configurations for different carriers can reduce unnecessary measurement gap occasions and increase network configuration flexibility.

Observation 12: In 5G NR, concurrent measurement gaps were introduced to improve network configuration flexibility and support a wider range of use cases. However, current RRM requirements limit the number of concurrent gaps to a maximum of two due to the collision among the configured gaps.

Proposal 7: Investigate the introduction of non-colliding multiple concurrent measurement gaps in 6G systems from the initial release.

# Fast RF retuning/interruption-free

The interruption defined in RRM requirements is associated with various procedures, such as Bandwidth Part (BWP) switching, and consists of both RF retuning and baseband (BB) processing time. This results in an overall interruption requirement of approximately 1 ms. Based on current working assumptions, RF retuning is expected to take 0.5 ms for FR1 and 0.25 ms for FR2, while BB processing time is assumed to be 0.5 ms. However, it is important to note that BB processing time may not impact all carriers in the same way as RF retuning does. Furthermore, recent advancements in UE implementation suggest that RF retuning can be performed much faster than the original assumption of 0.5 ms from NR Rel-15. Therefore, it is recommended that RAN4 further investigate ways to reduce interruption times and define requirements that more accurately reflect the capabilities of the latest UE technologies.

Observation 13: The RRM requirements of interruption including the RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Proposal 8: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

# Mobility and HO interruption reduction

Usually, RAN2 leads the discussion on mobility. RAN4 starts the discussion on mobility after RAN1/2 have sufficient progress. It may be not easy to carry out the discussion on enhancements with RAN1/2 dependency. But RAN4 can start from the solutions with less RAN1/2-dependency.

Proposal 9: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

In traditional HO solution, UE needs to do DL synchronization on target cell, which contribute to a large part of HO delay and interruption. The longer SSB period is, the longer time it takes for DL synchronization. So do the HO interruption. As far as we know, there are many proposals in RAN1 to extend 6G SSB periodicity, which will potentially lead to longer HO interruption. For LTM, the delay and interruption can be reduced compared to traditional HO through early DL synchronization. But the gain can only be obtained under some conditions, e.g., measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms. Further check the conditions, it can be observed that the conditions are even more difficult to meet with longer SSB periodicity. So in 6G SI, we need to study the solutions that can mitigate the problem of longer SSB periodicity.

Proposal 10: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Before discussing 6G HO solutions, we would like to recall the enhanced handover features in NR, i.e., CHO, DAPS, LTM and C-LTM. We summarized the pros and cons of DAPS, LTM and CHO as in Table 2.

Table 2 Enhanced handover features in 5G

|  | Pros | Cons |
| --- | --- | --- |
| DAPS | Interruption is the shortest. - Source cell will keep the link until UE sets up the connection with target cell | limited applicable scenarios or high UE complexity |
| LTM | Delay and interruption can be reduced through early RRC decoding, early DL/UL synchronization. | - Complex conditions to get the gain of early DL synchronization - measurement period is no larger than 160ms or the time gap between TCI state activated and cell switch command is less than 160ms - May be difficult to ensure measurement period within 160ms with even longer SSB periodicity in 6G. - Extra complexity to support early T/F tracking - Extra memory and conditions for early ASN.1 decoding - Additional interruption due to early RACH |
| CHO | Avoid the latency introduced by interaction between UE and NW (UE reports measurement results and then NW sends HO command) - HO initiated by UE under the guidance of NW | - Early preparation needed at Network - Potential resource waste due to network is not aware of when interruption starts. |

It can be observed that the interruption length of DAPS is the shortest at the cost of higher UE complexity, i.e., extra hardware or spare CA resource. Due to this, DAPS is applicable for limited scenarios. As mentioned above, the gain of early DL synchronization can only be obtained with some conditions in LTM. In 6G Day-1, the designed HO solution should be applicable to all the scenarios and work well without conditions. Therefore, we don’t think it is enough to reuse one of the enhanced HO features in 6G.

Proposal 11: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

To achieve the above goal as in Proposal 11, the 6G HO solution should get the advantages of individual enhanced HO features while resolving their problem as much as possible. Usually, NW stops the link “source cell ↔ UE” after sending HO/cell switch command, e.g., in legacy L3 HO and LTM. So HO interruption is almost the same as HO delay. And most of the time, reducing HO delay is similar as reducing HO interruption. There may be some other ways to counteract the impact of long delay, e.g., trigger HO earlier. But there is no way to counteract the impact of long interruption. So it is more important to reduce HO interruption.

Proposal 12: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Even in NR DAPS, the KPIs of HO delay and HO interruption have been decoupled. The main concept of DAPS is that source cell will keep the link until UE sets up the connection with target cell. In this way, the interruption length can be much less than the delay to achieve nearly “0” interruption. But to keep dual active links, i.e., source cell ↔ UE and target cell ↔ UE, it requires UE to be able to receive/transmit with both source and target cell simultaneously, and leads to high UE complexity.

Observation 14: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 15: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Checking the current HO procedure, upon receiving the HO command, UE will right away decode the RRC (TRRC) and apply all the new configurations of the target cell (Tprocessing). After Tprocessing, UE cannot be scheduled by the source cell. As a result, the interruption starts, as shown in Figure 2.

However, in our view, UE can choose when to apply the new configurations. The later UE applies the new configurations, the longer UE can maintain the link with the source cell. For example, UE can perform RRC decoding, DL synchronization as well as the time waiting for the RACH occasion, without interruption on serving cell(s) as shown in Figure 2. But after Tprocessing, UE usually breaks the link with serving cell except in DAPS. Inspired by DAPS and LTM, we propose a HO procedure as shown in Figure 3. In the new HO procedure, Tprocessing is performed just before RACH occasion. In this way, even for blind HO, HO interruption can be reduced to as short as “ Tprocessing”. To get the gain, it requires source cell keeps the link until UE starts to break the link with source cell, i.e., before Tprocessing.

Figure 2 Timeline of legacy handover procedure

Figure 3 Proposed HO procedure

Proposal 13: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 14: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

To further reduce HO delay, early preparation before HO/cell switch command, e.g., early RRC decoding, early DL/UL synchronization, is helpful. As mentioned in Table 2, supporting early T/F tracking requires some additional UE complexity. So the number of active TCI states that UE can support is limited. So for early RRC decoding. In 5G, early T/F tracking and early RRC decoding is totally under NW control. But the fact is that UE is always the first to know which cell is getting better. There will be always some latency introduced by reporting the measurement results. Inspired by CHO, we propose to support UE initiated early RRC decoding and early T/F tracking in 6G for better use of limited UE capability.

Proposal 15: Support UE initiated early RRC decoding and early T/F tracking in 6G.

# UE Tx timing

Initial Transmission Timing

UL performance depends on the UE’s TX timing accuracy, with requirements tied to the DL reference signal SCS (for DL timing detection) and the UL data SCS to avoid the interference beyond the CP length. However, high SCS can make compliance difficult, e.g. 60 kHz in NTN.

Given PRACH’s distinct CP length and role in initial access, defining PRACH-specific timing requirements could better enable higher SCS.

Proposal 16: Study PRACH (if introduced) specific timing accuracy requirement.

Timing Adjustment

In 5G, especially for advanced use cases such as Non-Terrestrial Networks (NTN) and high-speed trains (HST), the distance between the user equipment (UE) and the base station (BS) can change rapidly. This affects signal propagation time, and the UE transmit (Tx) timing needs to be adjusted so that uplink transmissions from different UEs arrive at the BS without collision and so that the BS can track each UE’s uplink (UL) timing. UE timing may change for the following scenarios:

  - UE movement. As shown in Figure 4 (a). The received timing changes due to UE motion or a frequency offset between the BS and UE, and the UE must maintain the timing difference between the receive and transmit timing with respect to the system timing.

  - Path change. As shown in Figure 4 (b). The dominant receive path may be attenuated or disappear due to channel variations, and the UE needs to re-synchronize its transmit timing to another receive path.

  - NTN Satellite movement. With the introduction of Non-Terrestrial Networks (NTN) in 3GPP Release 17, UEs implement timing/frequency pre-compensation, based on the provided Ephemeris and UE location, to project and adjust UE’s transmission timing.

  - Residual oscillator drift. Even without movement, small mismatches in clock frequencies between UE and BS or between UE TX and UE RX can accumulate, requiring periodic timing corrections.

  - UE-based TA for LTM cell switch. UE-based TA for LTM cell switch. When the UE switches cells, the distance to the new cell’s base station may be different, so the timing advance (TA) must be updated to maintain uplink synchronization.

(a) UE movement					(b) path change

Figure 4 Scenarios for UE timing adjustment.

In 5G, two opposite-direction approaches are used to manage UE timing adjustment:

- UE Gradual timing adjustment, a legacy mechanism present since 3G and retained through 4G and 5G (Rel‑15), where the UE monitors DL timing changes, which reflects the propagation delay changes, and shifts the UL transmit timing in the same direction; thus, if the DL timing is delayed, the UE further delays its UL, and the BS observes roughly double timing changes; to realign, Timing Advance Commands (TAC) must advance the UL timing (i.e., in the opposite direction of the DL change), which adds control overhead, and the adjustment rate is limited to 0.18–0.9 μs/s per TS 38.133 to prevent UL reception failures at the BS, making the mechanism potentially insufficient in fast timing‑change scenarios such as NTN or HST.

- UE pre-compensation, is a mechanism whereby the UE proactively adjusts its UL transmit timing in the opposite direction of DL timing change. For example, if the UE detects the DL timing is delayed, it advances its UL timing by the same amount, preventing misalignment before the BS detects it.

UE pre-compensation has been adopted in R17 NTN, R18 FR2 HST/POS and is typically applied when timing changes are large (e.g., > CP/4) or when TCI changes occur; in NTN, the adjustment is based on satellite/BS position to project the correct timing at satellite, though the UE can also derive it from the detected DL timing changes for TN network.

(1) UE movement			(2) path change

Figure 5 Methods for UE timing adjustment.

In 5G systems, gradual timing adjustment is established as the baseline, with mandatory test cases introduced since Release 15. This approach restricts the use of UE pre-compensation to certain scenarios. Consequently, a UE that adopts pre-compensation during normal operation may not pass the baseline tests designed for gradual timing adjustment.

In 6G, we propose studying the feasibility of adopting UE pre-compensation as the baseline approach, specifically replacing gradual timing adjustment for handling UE mobility and path changes, while retaining the Timing Advance command to correct the UE’s initial UL timing.

Observation 16: In 5G, two opposite direction approaches are used to adjust UE UL timing: gradual adjustment, which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

Observation 17: In 5G, a UE that adopts pre-compensation during normal operation may not pass the baseline tests designed for gradual timing adjustment.

Proposal 17: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

# SCell activation, and spectrum aggregation

Cell with multiple carrier

In 5G, the carrier-aggregation (CA) framework is critical for spectrum aggregation, yet it limits physical layer and protocol efficiencies because it enforces a one-to-one coupling of each physical carrier component and a logical cell, resulting in excess control overhead and slow carrier activation.

For 6G, we recommend decoupling this one-to-one mapping and aggregating multiple carriers with similar propagation channel properties to a single multi-carrier cell. As illustrated in Figure 6, intra-band and nearby inter-band carriers can be aggregated into a single cell when they meet specific conditions, e.g., time and frequency alignment, frequency span, and receive power difference, thereby enabling fast carrier activation and reduce the control overhead. Based on 5G experiences, for example, practical thresholds of conditions may include Maximum Receive Timing Difference (MRTD) < 3us (or CP), frequency error < 0.1ppm, and similar total received power for an intra-band-like UE receiver, where a common time/frequency synchronisation reference and AGC can be used at a UE. In addition, it is a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where these conditions are typically met.

Note that carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

While RAN1 will study multi-carrier cell support and procedures, RAN4 RF will need to study RF-switch time (to switch on a new carrier within a cell) requirements, acceptable Transmit timing alignment (TAE), BS frequency error, and total power differences at UE between carriers; RAN4 RRM can then assess aggregation delay, accounting for AGC resettling and fine time tracking.

Figure 6 Illustration of multi-carrier cell

Observation 18: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 19: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 18: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Due to the limited number of Tx chains at the UE, the number of bands that the UE can be configured for is constrained. For instance, a UE with two Tx chains can either be configured with one band using UL-MIMO or with two bands without UL-MIMO (one Tx chain per band). UL Tx switching allows the UE to dynamically switch the Tx chain(s) among bands, enabling configuration with more bands and supporting UL-MIMO transmissions without requiring additional Tx chains. UL Tx switching was introduced incrementally from R16 to R19. Each release targets a different scenarios, which adds unnecessary limitation to 5G networks. For example, if more than 2 UL bands are needed, it can only be supported from Release 18. For 3Tx UE, it can only be supported in Rel-19, but it is limited in 2 UL bands. Enabling flexible carrier switching in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity by ensuring the uplink resources for the UE can be allocated where resources are available. Therefore, improvements for UL Tx switching should be investigated in 6G to simplify and streamline its operation.

Beyond UL, enabling more flexible DL carrier switching without requiring carrier aggregation could deliver benefits comparable to those achieved with UL carrier switching; in 5G it was considered only in Rel-19 with limited scope, while carrier switching is especially valuable under hardware constraints (e.g., few RF chains or synthesizers) and in the presence of DL/UL interference and intermodulation distortion, so the corresponding switching-delay will need to be studied in RAN4 RF and the interruption requirements can be further studied in RAN4 RRM.

For 6G, we expect a more flexible mechanism for DL/UL switching with forward compatibility on aspects on number of bands, number of Tx/Rx, … etc. In this 6G SI, it would be good to take this as one design target and study its corresponding impact in 3GPP specs.

Observation 20: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 19: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Carrier aggregation (as in 5G) remains necessary when the synchronization and power conditions described above are not met across carriers, such as when aggregating two widely separated carriers.

Since Rel-15, 5G NR has continuously refined Scell activation to address a variety of conditions (with or without measurement reports, different synchronization states, power differences), diverse RS configurations (SSB-less, A-TRS, P/SP/AP CSI-RS), and different types of UE RF architecture (intra-/inter-band, co-/non-collocated), yet only limited scenario has seen widespread adoption in commercial 5G.

For 6G, the carrier aggregation activation should ensure timely CSI so the network can schedule sufficient radio resources for busty traffic, and the Scell activation should be specified only for critical, high-likely deployment cases to simply the requirement design.

Based on 5G experience, the SCell/carrier activation at least involves RF switch, AGC settling, cell search, fine sync tracking, and CSI report. RAN4 RF will need to study RF switch-time requirements, and RAN4 RRM will assess SCell activation interruption and the overall delay, taking other factors into account.

Observation 21: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 20: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

# Measurement capability

In 5G, 3GPP TS 38.133 (9.2.3, 9.3.3) specifies the UE measurement capability for number of cells. The requirement were introduced to support cell reselection and handover preparation.

- intra-frequency layer: 8 identified cells in FR1, 6 identified cells in FR2.

- inter-frequency layer: 4 identified cells in FR1 and FR2.

However, the maximum number of detectable cells (i.e. 8 cells) at the cell edge area is rarely observed from the 5G NR field data, especially under the -6 dB side condition. Table 3 provides an example of field log showing the number of detected cells in FR1. The data indicate that, with 90% probability, the number of detected cells in FR1 is <=4, and with 97% probability it is <=5. However, these values depend on the deployment scenario (e.g., cell size and operating frequency). This suggests that current requirements may overestimate practical needs, leading to unnecessary hardware complexity.

As 6G potentially utilizes more robust SS design and higher number of UE antennas. This increases the computational and memory burden on UE if the measurement requirements remain unchanged or are scaled up, RAN4 should reassess UE measurement requirements for 6G (i.e. cells and beams).

Table 3 Number of detected intra-frequency neighbor cells in FR1

| # of Detected neighbor cells (SINR>-6dB) | Probability |
| --- | --- |
| 0 | 2.7% |
| 1 | 15.1% |
| 2 | 27.8% |
| 3 | 28.4% |
| 4 | 16.4% |
| 5 | 7.1% |
| 6 | 2.0% |
| >=7 | 0.4% |

Observation 22: The UE capability of number of cells is underutilized in 5G.

Proposal 21: Study the appropriate measurement capability for number of cells in 6G. The number of detected cells in FR1 is <=4 with 90% probability and <=5 with 97% probability.

# Inter-RAT sync

In legacy systems, inter-RAT mobility usually refers to the process of measuring and managing the performance and quality of different radio access technologies within a mobile network. This is crucial for ensuring seamless connectivity and optimal performance as users move between areas covered by different technologies, such as 4G, 5G and later 6G. In addition, we learned in 4G and 5G that synchronization between different RATs is significantly important to ensure successful Inter-RAT measurements and mobility. If RATs are asynchronous, for example, if LTE NW does not know the timing information of the 5G NW, it not possible to configure an appropriate SMTC or measurement gap (MG) offsets for inter-RAT measurements. This means measurements in one RAT are likely to fail since SMTC or MG does not coincide with the transmission of the SSB from the other RAT, as shown in Figure 7.

Figure 7: Asynchronous Inter-RAT measurements

Observation 23: Without LTE/NR timing sync and without the LTE NW knowing NR NW timing, appropriate SMTC and MG offsets for inter-RAT measurement cannot be configured.

This issue was discussed in NR and for which a new solution called System Frame Time Difference (SFTD) was introduced in Rel-15, which allows requesting the UE to report the SFN and Frame Timing Difference between two RATs. However, such solution was never used or deployed in the real networks.

Observation 24: SFTD solution was introduced in Rel-15 but never used in real networks.

Therefore, to avoid repeating the legacy mistakes in 6G and spending time on designing complicated solutions which are not likely to be implemented in the real field, we propose that synchronization between RATs to be the baseline assumption in 6G, where timing information of the target cell’s RS can be provided to enable inter-RAT measurement and mobility. In addition, this synchronization does not need to be perfect as long as the RS in one RAT can occur within the measurement configuration by the other RAT.

Proposal 22: Synchronized Inter-RAT to be the baseline assumption for 6G to enable inter-RAT measurement and mobility.

# Summary

Our observations and proposals on the 6G RRM requirements are summarized as below:

// MG framework

Conventional measurement gaps:

Observation 1: Measurement gaps are typically responsible for a 15% interruption of data transmission and reception (e.g. MGL = 6ms, MGRP = 40ms), which impact user experience in terms of throughput and latency.

Proposal 1: Investigate methods to reduce the use of measurement gaps in 6G systems.

Observation 2: In LTE/NR, a UE might report a need for a measurement gap when RF retuning is necessary to measure frequency carriers outside its operating carriers.

Observation 3: In LTE/NR, a UE might report a need for a measurement gap due to a desense (MSD) issue between the target band, containing the SSB to be measured, and the serving bands.

Proposal 2: Identify and address the reasons behind measurement gaps in previous generations of mobile systems and how to handle these issues in 6G systems.

Gapless solutions

Observation 4: In 5G NR Rel-16, NeedForGap gapless solution, was specified only in terms of signalling, and RRM requirements were not included until Rel-18. This left ambiguity about the expectations for UE behaviour, such as how to avoid the impact of RF retuning.

Observation 5: In 5G NR Rel-17, NCSG gapless solution, was specified fully in terms of signalling and RRM requirements. However, given that the feature was introduced in the third release of 5G NR, this feature has not been implemented in real field.

Proposal 3: The gapless capability to be specified in the 6G Day-1 spec, including both signalling and RRM requirements.

Issue 1: Available RF chain:

Observation 6: In 5G NR, during measurement gaps length, a UE interrupts uplink and downlink data transmission and reception across all serving cell carriers to allow measurement of a target SSB of a target cell.

Proposal 4: Further investigate how to limit the measurement gap impact to a single carrier or specific number of carriers.

Observation 7: From a UE implementation perspective, whether all carriers should be interrupted depends on the availability of an idle RF chain to perform measurements on the target cell.

Observation 8: Measurement gap and gapless features are based on the RF chain availability at the UE side.

Proposal 5: When defining measurement gap/interruption requirements and reporting signalling, different UE behaviours should be specified based on the availability of an idle RF chain.

Issue 2: RF retuning time:

Observation 9: The working assumption of RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Observation 10: With latest UE advancements, a UE can achieve faster RRT compared to that specified in the working assumption in 5G NR.

Proposal 6: RAN4 (RRM/RF) to further investigate the possibility of reducing the RF retuning time for measurement gaps/gapless.

Issue 3: Non-colliding multiple GAPs:

Observation 11: From a system operation perspective, applying different measurement gap configurations for different carriers can reduce unnecessary measurement gap occasions and increase network configuration flexibility.

Observation 12: In 5G NR, concurrent measurement gaps were introduced to improve network configuration flexibility and support a wider range of use cases. However, current RRM requirements limit the number of concurrent gaps to a maximum of two due to the collision among the configured gaps.

Proposal 7: Investigate the introduction of non-colliding multiple concurrent measurement gaps in 6G systems from the initial release.

// Fast RF retuning/interruption-free

Observation 13: The RRM requirements of interruption including the RF retuning time in previous mobile generation has not been optimised since NR Rel-15.

Proposal 8: RAN4 (RRM/RF) to further study the possibility of reducing the interruption time including the RF retuning time for different UE procedures.

// Mobility and HO interruption reduction

Proposal 9: In 6G SI, RAN4 to start from mobility solutions with less RAN1/2-dependency.

Proposal 10: A solution that can mitigate the problem of longer SSB periodicity needs to be considered in 6G study.

Proposal 11: 6G Day-1 HO solution should be applicable to all the scenarios and work well without conditions.

Proposal 12: For 6G mobility, decouple the KPIs of HO delay and HO interruption with the priority on reducing interruption.

Observation 14: Based on the experience of DAPS, HO interruption can be much shorter than HO delay.

Observation 15: Based on the experience of DAPS, one method to reduce HO delay is to keep the link between source cell and UE when UE gets to set up the connection with target cell.

Proposal 13: HO interruption can be reduced by postponing Tprocessing to the last, e.g., just before RACH occasion for RACH-based HO.

Proposal 14: To further reduce HO interruption, source cell should keep scheduling until UE breaks the link with source cell.

Proposal 15: Support UE initiated early RRC decoding and early T/F tracking in 6G.

// UE Tx timing

Proposal 16: Study PRACH (if introduced) specific timing accuracy requirement.

Observation 16: In 5G, two opposite direction approaches are used to adjust UE UL timing: gradual adjustment, which doubles the timing changes observed at BS and requires frequent TAC, and UE pre-compensation, where the UE corrects timing before the BS detects misalignment.

Observation 17: In 5G, a UE that adopts pre-compensation during normal operation may not pass the baseline tests designed for gradual timing adjustment.

Proposal 17: Study the feasibility of replacing gradual timing adjustment with UE pre-compensation, while retaining the Timing Advance command.

// Scell activation, and spectrum aggregation

Cell with multiple carrier

Observation 18: Aggregate multiple carriers into a single cell can reduce the siganling overhead and UE processing complexity of PHY channels, enabling more efficient spectrum aggregation and scalable spectrum utilization.

Observation 19: Multi-carrier cell can be a simple approach to aggregate contiguous spectrum wider than a single carrier FFT size, where conditions (e.g. e.g. time and frequency alignment, frequency span, and received power differences) are typically met.

Proposal 18: RAN4 RF should study the conditions and requirements for aggregating multiple carriers to a single cell for 6G. For example, RF switch-time requirements, acceptable transmit timing alignment error (TAE), base-station (BS) frequency error, and total received power difference limits at the UE. And RAN4 RRM should study the corresponding interruption and delay requirements.

Carrier switch enhancements for UL and DL

Observation 20: Enabling flexible carrier switching between UL and between DL with forward compatibility to number of band/Tx/Rx in 6G enables improved spectral efficiency, coverage, latency, reliability and system capacity.

Proposal 19: RAN4 should study support of carrier switching and simplify configurations with forward compatibility; specifically, RAN4 RF should study applicable scenarios and RF switch-time requirements, and RAN4 RRM should study the corresponding interruption and delay requirements.

SCell activation, increase of spectrum aggregation

Observation 21: 5G NR Scell activation requirements are too complicated and only limited scenarios have seen widespread adoption in commercial.

Proposal 20: RAN4 should study critical, high-likely SCell activation scenarios and ensure timely CSI acquisition; RAN4 RF should study RF switch-time requirements, and RAN4 RRM should study SCell activation interruption and overall delay.

// Measurement capability

Observation 22: The UE capability of number of cells is underutilized in 5G.

Proposal 21: Study the appropriate measurement capability for number of cells in 6G. The number of detected cells in FR1 is <=4 with 90% probability and <=5 with 97% probability.

// Inter-RAT sync

Observation 23: Without LTE/NR timing sync and without the LTE NW knowing NR NW timing, appropriate SMTC and MG offsets for inter-RAT measurement cannot be configured.

Observation 24: SFTD solution was introduced in Rel-15 but never used in real networks.

Proposal 22: Synchronized Inter-RAT to be the baseline assumption for 6G to enable inter-RAT measurement and mobility.

# References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)