3GPP TSG-RAN WG4 #116bis	R4-2513042

Prague, Czech Republic, October 13-17, 2025

Agenda item:	8.9

Source:	Qualcomm Incorporated

Title:	On RAN4-driven AI/ML aspects of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework. Topic (8) of the study item covers AI/ML use cases and AI/ML framework to be identified and studied in all RAN working groups.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In this contribution, we discuss our views of the goals of the RAN4-driven AI/ML aspects of the 6G study item.

# Views for the 6G Study Item for RAN4-driven AI/ML

RAN1/2/3 will investigate AI/ML related use cases in Rel-20 and define requirements for those use cases in Rel-21. The RAN4 Rel-20 study item on AI/ML should not duplicate the efforts of RAN1/2/3. Hence, during Rel-20, RAN4 should identify the use cases of AI/ML that it can drive. RAN4 should ensure a clear split between RAN4 driven work and RAN1/2/3 driven work.

Proposal 1: The RAN4 Rel-20 study item on AI/ML should not duplicate the efforts of RAN1/2/3. RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

AI/ML based model, developed via prior training, can be applied in many different scenarios. However, all these different scenarios may not lead to gain. Hence, RAN4 should study the potential benefits of AI/ML in different uses and define requirements only if the performance of AI/ML is greater than or equal to legacy.

Proposal 2: RAN4 should study the potential benefits of AI/ML across different scenarios and consider defining RAN4 requirements only if the performance benefits of AI/ML are proven and justified.

In 5G, AI/ML related air interface topics got discussed in one session whereas AI/ML mobility got discussed in a different session. However, AI/ML related 5G air interface topics covered a wide range of sub-topics, e.g., 1) beam management, 2) OTA testing, 3) Positioning, 4) CSF prediction and compression. Traditionally, these sub-topics have been handled in different sessions, e.g. 1) beam management and positioning topics are typically handled in RRM session, 2) CSF prediction and compression are more suitable for demod sessions, 3) FR2 beam management OTA is more suitable for OTA sessions.

Unlike RRM, demod, OTA and RF, AI/ML based training and inference are tools that are used to analyze and solve different core issues. Hence, AI/ML should not have a separate session in 6G study item and work item. Rather, AI/ML should be part of each session (RRM, demod, OTA and RF) in 6G.

Observation 1: In 5G, AI/ML related air interface topics, covering wide range of topics like CSF, beam management, OTA and positioning, got discussed in one session.

Observation 2: Unlike RRM, demod and RF, AI/ML based training and inference are just tools that are used to analyze and solve different core issues.

Proposal 3: AI/ML should not be discussed in a dedicated standalone session within 6G. Instead, AI/ML topics should be integrated into individual sessions where relevant experts can participate and contribute.

For example, our proposals for the next section of this contribution, i.e., AI/ML based measurement adaptation and RX beam sweeping factor optimization, can be handled in RRM session.

In the following subsections we share our view of uses cases that could be driven by RAN4.

## Dynamic Adaptation of Measurement Procedure

Figure 1: Influence of gNB beam shape on beam failure experienced at UE

5G typically requires UE to select the periodicity of serving and neighbor cell measurement based on network configuration and channel condition. For example, most of the 5G serving cell measurement periods depend on network configuration. On the other hand, some 5G work items allow UE to dynamically update the measurement period of neighboring cells based on serving cell’s channel conditions.

Observation 3: 5G typically requires UE to select the periodicity of serving and neighbor cell measurement based on network configuration and channel condition.

Considering the growing demand for AI-native UE implementations, it is anticipated that mobile UEs will adopt AI-based measurement adaptation solutions, which may or may not be transparent to the network. While the UE may be able to prudently adjust its measurement behavior to enhance user experience without compromising performance, RRM requirements may limit the applicability of such intelligent implementations since these requirements are typically defined in a scenario- and context-agnostic manner. As AI-based solutions are expected to mature by the 6G timeline, RAN4 should investigate and explore how to ensure such implementations are not unfairly restricted by the requirement specifications.

Observation 4: Although AI-native UE implementations have gained significant traction and are expected to mature by the 6G timeframe, scenario-agnostic RRM requirements could unintentionally constrain such intelligent behaviors.

It would be prudent for 6G to allow Rel-21 UE to dynamically adapt the measurement period of both serving and neighbor cell based on channel condition, along with network configuration. AI/ML based model, developed via prior training, and subsequent inference may allow UE to judiciously adjust measurement procedure.

In scenario 1 of Figure 1, UE would experience beam failure quite quickly in its trajectory since gNB is using a narrow beam shape. On the other hand, in scenario 2 of Figure 2, UE would experience beam failure a bit late in its trajectory since gNB is using a wider beam shape. If this UE were pre-trained with this gNB’s beam configuration, it could have developed an AI/ML model to predict quality of serving cell soon. That AI/ML model, along with recent measurements, would have allowed UE to judiciously and dynamically adapt the measurement period of serving cell.

Observation 5: AI/ML based training and inference may allow 6G UE to judiciously adjust measurement periods.

Proposal 4: RAN4 should investigate if and how to accommodate AI-based UE dynamic measurement adaptation, which may or may not be transparent to the network, within 6G RRM specification to avoid limiting innovation.

## Support of Reduced RX Beam Sweeping Factor in FR2

Figure 2: Rx Beam Prediction via AI/ML

RAN4 has assumed the RX beam sweeping factor to be eight in many FR2 measurement periods of Rel-15. RAN4 assumed that UE needs to sweep eight RX beams in FR2 to cover angle of arrivals in relevant azimuth and elevation domain and detect or measure reference signals. This scaling factor of eight significantly increases many evaluation periods in FR2. This long evaluation period slows down the mobility procedures significantly.

Observation 6: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

The work item of 5G AI/ML beam management allowed UE to measure the properties of one set (setB) of reference signals and predict the properties of a different set (setA) of reference signals. 5G AI/ML assumed UE to sweep eight RX beams, too, to measure the reference signals of setB. Hence, the measurement period in 5G spatial only beam prediction scenario remains same as legacy FR2 L1-RSRP measurement period. On the other hand, the measurement period of 5G spatio-temporal beam prediction scenario gets additionally extended by the scaling factor of the minimum number of observation samples. RAN4 has not yet determined the minimum number of observation samples in spatio-temporal beam prediction. However, the presence of two scaling factors (FR2 RX beam sweeping factor and the minimum number of observation samples) may make the evaluation period of 5G spatio-temporal beam prediction excessively long.

Observation 7: 5G AI/ML beam management work item assumed FR2 UE to sweep eight RX beams to measure setB beams.

Observation 8: The measurement period of 5G spatio-temporal beam prediction gets scaled by both legacy RX beam sweeping factor (eight) and the minimum number of observation samples. The presence of these two factors may make the measurement period of 5G AI/ML based spatio-temporal beam prediction excessively long.

RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced beam sweeping factors for serving and neighbor cells. These UE capabilities do not assume any use of AI/ML. RAN4 should study if and how 6G UE can support reduced RX beam sweeping factor in FR2.

Observation 9: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells respectively.

An AI/ML based model, developed based on prior training, can help a UE to measure a reference signal with a reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. Figure 2 shows that UE can measure a set of gNB beams with a subset of RX beams and predict the properties of the same set of gNB beams with the full set of RX beams. RAN4 should discuss whether 6G UE’s support of reduced beam sweeping factor should depend on the availability of AI/ML model and prior training.

Observation 10: AI/ML based training and inference can help a UE to measure a reference signal with reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. This would significantly reduce the evaluation period in FR2.

Proposal 5: RAN4 should study if and how AI/ML based training and inference can help a 6G FR2 UE to support reduced RX beam sweeping factor.

## Generalization of AI/ML Model

AI/ML will play an important role in 6G. “Generalization” is an important aspect of AI/ML, i.e., one needs to see if the designed AI/ML model is “general” enough to handle different test environments. One could design multiple tests with the same network configuration and different channel conditions to check the generalization of AI/ML models. However, this does not prevent UE from employing different AI/ML models to handle different channel settings. An AI/ML model’s ability to generalize can be truly tested in transient scenarios, where channel parameters (e.g., Doppler in AI/ML based CSI prediction or temporal beam prediction) change during the tests. RAN4 should study if and how to change channel parameters during a performance test to check if an AI/ML model is “general” enough.

Observation 11: “Generalization” is an important aspect of AI/ML, i.e., one needs to see if the designed AI/ML model is “general” enough to handle different test environments.

Observation 12: An AI/ML model’s ability to generalize can be truly tested in transient scenarios, where channel parameters (e.g., Doppler in AI/ML based CSI prediction and temporal beam prediction) change during the tests.

Proposal 6: RAN4 should study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models.

# Conclusions

In this paper we provided our views on RAN-driven AI/ML in the 6GR study item. Our observations and proposals are summarized as follows:

Views for the 6G Study Item for RAN4-driven AI/ML

Proposal 1: The RAN4 Rel-20 study item on AI/ML should not duplicate the efforts of RAN1/2/3. RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

Proposal 2: RAN4 should study the potential benefits of AI/ML across different scenarios and consider defining RAN4 requirements only if the performance benefits of AI/ML are proven and justified.

Observation 1: In 5G, AI/ML related air interface topics, covering wide range of topics like CSF, beam management, OTA and positioning, got discussed in one session.

Observation 2: Unlike RRM, demod and RF, AI/ML based training and inference are just tools that are used to analyze and solve different core issues.

Proposal 3: AI/ML should not be discussed in a dedicated standalone session within 6G. Instead, AI/ML topics should be integrated into individual sessions where relevant experts can participate and contribute.

Dynamic Adaptation of Measurement Procedure

Observation 3: 5G typically requires UE to select the periodicity of serving and neighbor cell measurement based on network configuration and channel condition.

Observation 4: Although AI-native UE implementations have gained significant traction and are expected to mature by the 6G timeframe, scenario-agnostic RRM requirements could unintentionally constrain such intelligent behaviors.

Observation 5: AI/ML based training and inference may allow 6G UE to judiciously adjust measurement periods.

Proposal 4: RAN4 should investigate if and how to accommodate AI-based UE dynamic measurement adaptation, which may or may not be transparent to the network, within 6G RRM specification to avoid limiting innovation.

Support of Reduced RX Beam Sweeping Factor in FR2

Observation 6: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

Observation 7: 5G AI/ML beam management work item assumed FR2 UE to sweep eight RX beams to measure setB beams.

Observation 8: The measurement period of 5G spatio-temporal beam prediction gets scaled by both legacy RX beam sweeping factor (eight) and the minimum number of observation samples. The presence of these two factors may make the measurement period of 5G AI/ML based spatio-temporal beam prediction excessively long.

Observation 9: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells respectively.

Observation 10: AI/ML based training and inference can help a UE to measure a reference signal with reduced set of RX beams and infer the properties of full set of RX beams for that reference signal. This would significantly reduce the evaluation period in FR2.

Proposal 5: RAN4 should study if and how AI/ML based training and inference can help a 6G FR2 UE to support reduced RX beam sweeping factor.

Generalization of AI/ML Model

Observation 11: “Generalization” is an important aspect of AI/ML, i.e., one needs to see if the designed AI/ML model is “general” enough to handle different test environments.

Observation 12: An AI/ML model’s ability to generalize can be truly tested in transient scenarios, where channel parameters (e.g., Doppler in AI/ML based CSI prediction and temporal beam prediction) change during the tests.

Proposal 6: RAN4 should study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025