3GPP TSG-RAN WG4 Meeting #116bis                      R4-2513059                                                                                                       Prague, CZ, 13th – 17th October 2025

Source:	China Telecom

Title:	Discussion on demodulation aspects for 6G study

Agenda Item:	8.8

Document for:	Discussion

Introduction

In RAN #108 meeting, the new SI on 6G Radio was approved [1], with the following objectives for the demodulation aspect:

Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

In this contribution, our views on demodulation aspects for 6G study is provided.

Discussion

From our point of view, the demodulation part of the 6G study should mainly focus on receiver and channel modeling.

Receiver

In 5G, from Rel-15 to Rel-20, RAN4 has defined requirements for the following receivers/receiving features for UE and BS.

|  | UE | BS |
| --- | --- | --- |
| Rel-15 | - MMSE-IRC for 2/4Rx SU-MIMO - R-ML for 2/4Rx SU-MIMO |  |
| Rel-17 | - MMSE-IRC for 2/4Rx with inter-cell and intra-cell inter-user (MU-MIMO) interference - CRS-IM for inter-cell 4G CRS interference for dynamic spectrum sharing (DSS) assuming LLR-weighting processing - Requirements for 1Rx SU-MIMO (RedCap) |  |
| Rel-18 | - MMSE-IRC for 8Rx SU-MIMO (cover both baseline and simplified 8Rx structures) - R-ML for 2/4Rx with MU-MIMO interference |  |
| Rel-19 | - MMSE-IRC for 8Rx with inter-cell and MU-MIMO interference (cover both baseline and simplified 8Rx structures) - MMSE-IRC for 6Rx SU-MIMO | - MMSE-IRC for 2/4/8Rx with inter-cell interference |
| Rel-20 | - MMSE-IRC for 6Rx with inter-cell and MU-MIMO interference |  |

We have the following observations based on the above background:

- RAN4 has defined performance requirements for 1/2/4/6/8Rx UEs.

- MMSE-IRC receiver with DMRS-based interference covariance estimation is widely used for 5G UEs and BSs.

- Given that RAN4 has limited time to perform system-level simulations based on the 5G network structure in the WI phase, while defining the requirements for inter-cell interference scenarios for both DL and UL, RAN4 had to reuse the same interference power value as 4G LTE. This means some important 5G features (e.g., massive MIMO, HPUE) are ignored.

- For the receiving features (i.e., MU-MIMO R-ML) that requires additional information of the interference source, RAN4 had to introduce some default assumptions to balance the additional NW signaling overhead and the UE complexity.

Based on the above observations, we have the following proposals for the 6G demodulation study:

Proposal 1: The 6G SI should cover 1/2/4/6/8Rx UE and 2/4/8Rx BS. For 8Rx UE, RAN4 to discuss whether to consider different receiver structures (e.g., baseline and simplified)

Proposal 2: The MMSE-IRC should be the baseline receiver for both UE and BS.

Proposal 3: As a preparation for 6G requirement with interference definition, RAN4 should make use of this study phase, to perform system level simulation and derive inter-cell interference model for the state-of-the-art network.

We also propose to cover advanced receivers (R-ML, soft-IC, …) in the 6G study. The performance gain should be re-evaluated since UE processing capability is much improved since 5G Rel-15, which could enable higher algorithm complexity (e.g., larger radius for sphere decoding of R-ML). In addition, the required information for advanced receivers for MU-MIMO could also be re-visited to avoid many RAN4 default assumptions. While studying the performance of advanced receivers, the UE computation time should be considered.

Proposal 4: Cover advanced receivers (R-ML, soft-IC, …) in the 6G study, to re-evaluate the performance gain and to re-visit the required information for MU-MIMO scenario.

Proposal 5: The UE computation time should be considered while studying the performance of advanced receivers.

Moreover, we also notice that Multi-RAT Spectrum Sharing (MRSS) is a possible scenario for 6G in [1].

- Physical Layer structure for 6GR, - … - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation, to avoid the severe interference from the 4G neighbor cell CRS in 5G DSS.

Proposal 6: RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation.

Channel model

In 5G, currently, all UE and BS demodulation tests are using TDL channel modeling with some simplification compared with the original TDL captured in TR38.901, which is somehow sufficient for basic R15 5G performance testing. However, with the evolution of 5G, more features are introduced and many of them require to be tested under spatial channel model (SCM), e.g., rank 8 transmission with 2 CWs, MU-MIMO with e-Type II PMI feedback, PRACH repetition with different beams, etc.

In Rel-19, RAN4 has performed studies for SCM for SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected. As a result, simplified CDL channel model (namely rCDL-C1 in TR38.753) is derived and will be used for some example test requirement definition in Rel-20.

In addition, in Rel-20, RAN4 will do further studies for MU-MIMO based on simplified CDL channel model under SI FS_NR_demod_SCM_Ph2, and both DL and UL will be covered. According to the SID [2], this study will be applicable for both 5G-A and 6G.

To avoid duplicate studies, for this 6G SI thread, we propose to study the CDL performance for more DL SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting and so on.

Observation 1: In Rel-19, RAN4 has performed studies for SCM for SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected.

Observation 2: In Rel-20, RAN4 will do further studies based on simplified CDL channel model for MU-MIMO and both DL and UL will be covered under SI FS_NR_demod_SCM_Ph2, will be applicable for both 5G-A and 6G.

Proposal 7: For this 6G SI thread, RAN4 to study the simplified CDL performance for more DL SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting.

Conclusion

Proposal 1: The 6G SI should cover 1/2/4/6/8Rx UE and 2/4/8Rx BS. For 8Rx UE, RAN4 to discuss whether to consider different receiver structures (e.g., baseline and simplified)

Proposal 2: The MMSE-IRC should be the baseline receiver for both UE and BS.

Proposal 3: As a preparation for 6G requirement with interference definition, RAN4 should make use of this study phase, to perform system level simulation and derive inter-cell interference model for the state-of-the-art network.

Proposal 4: Cover advanced receivers (R-ML, soft-IC, …) in the 6G study, to re-evaluate the performance gain and to re-visit the required information for MU-MIMO scenario.

Proposal 5: The UE computation time should be considered while studying the performance of advanced receivers.

Proposal 6: RAN4 should also be prepared to deal with the possible interference caused by MRSS, by interference cancellation or mitigation.

Observation 1: In Rel-19, RAN4 has performed studies for SCM for SU-MIMO scenario, PDSCH 4T4R rank4 and 8T8R rank8 cases and PMI type I and eType II cases are selected.

Observation 2: In Rel-20, RAN4 will do further studies based on simplified CDL channel model for MU-MIMO and both DL and UL will be covered under SI FS_NR_demod_SCM_Ph2, will be applicable for both 5G-A and 6G.

Proposal 7: For this 6G SI thread, RAN4 to study the simplified CDL performance for more DL SU-MIMO cases such as inter-cell interference, advanced receiver, CSI reporting.

Reference

RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), RAN #108, June 2025

RP-252956, New SID: Study on spatial channel model for demodulation performance requirements for NR: Phase 2, Apple, RAN#109, Sep 2025