3GPP TSG RAN WG4 #116-bis                         			   		   R4-2513061

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.3

Source:	InterDigital Europe, Ltd.

Title:	Overview of 6GR System Parameters

Document for:	Discussion and Decision

- Introduction

RAN approved a new SID on 6G Radio at RAN#108 [1]. As indicated in the SID, the development of 6G Radio is needed to meet a range of future use cases requiring extreme performance, flexibility and scalability. RAN also initiated study on RAN requirements [2][3] to define specific quantitative targets with respect to KPI’s such as peak data rate, spectral efficiency, user experienced data rate, area capacity, user and control plane latency, reliability, mobility, and energy efficiency. Such targets are expected to surpass those of 5G in most or all cases.

The SID also emphasizes some key requirements and principles that should guide the design of the new system from the start. At a high-level, the requirements can be summarized as follows:

- Support new and expanded usage scenarios

- Enhance performance from user and system perspective

- CAPEX / OPEX reduction, including smooth migration from 5G

To achieve these requirements, 3GPP is to design 6GR with the following guiding principles:

- Focus on energy efficiency for NW and device

- Support enhanced spectral efficiency

- Enable wider bandwidth and improve support for diverse spectrum allocations

- Enable scalable and forward compatible design for diverse device types

- Common radio design to meet vertical needs (mobility broadband high priority)

- Enhanced overall coverage (UL), enabling 7 GHz operation with same site grid as 3.5 GHz

- Harmonized TN/NTN

- Avoid excessive configurations/options/capabilities

This contribution presents our views on the system parameters of the future 6GR system along with proposals on assumptions and aspects that may involve RAN4.

- General aspects of 6G Radio

This section provides an analysis of each main functionality of the system from the perspective of meeting the new or higher requirements of 6G radio.

  - Waveform

6G radio should target a scalable system design supporting various device types with mobile broadband service as a higher priority while meeting requirements for other services. As such, scalability, high spectral efficiency, energy efficiency, coverage, and MIMO integration should be considered as KPIs for waveform study for communication purposes in both uplink and downlink.

The potential new waveform for 6GR will be a part of fundamental PHY design study as it will impact other 6GR system designs including MIMO, duplex, spectrum management, and physical channel/signal design. Therefore, it is important that the waveform study should be efficient, and earlier decision will be helpful to progress other system design topics.

Enhancement of energy efficiency and coverage for 5G NR waveform was studied during Rel-18, such as reducing PAPR with tone reservation, and/or FDSS-SE. However, due to lack of time, PAPR reduction schemes were not supported in 5G-NR. A similar study can be conducted in 6GR study for the 5G NR waveform to compare with new candidate waveform. Additionally, the optimization/reduction of the MPR may also enhance the UL coverage.

However, if a new waveform is to be adopted, then from RAN4 perspective it must be at least as good as the 5G versions in terms of coexistence, Emissions Mask, OBE, Spurious Emissions, and nonetheless compatibility for CA and DC current band combinations. An early decision on the waveforms from RAN1 will allow RAN4 to assess as early as possible the implications for the RF and RRM requirements.

Proposal 1: Potential study of new 6G NR waveforms involving RAN4 shall include at least the following KPIs: better PAPR, CM, and Emission Mask, OBE, full compatibility with the current 5G CA and DC band combinations.

  - Modulation

A uniform constellation has been used in LTE and 5G-NR for both downlink and uplink mainly because of its simplicity, compatibility, and low implementation complexity. The uniform constellation with even higher modulation order can be considered for 6GR as well. From RAN4 perspective, the modulation has a direct impact on the MPR and directly impacts the UL coverage for example.

As an advanced feature, non-uniform constellation (e.g., probabilistic, geometric) can be studied as it has been widely studied in academia and shown more than 1dB gain in a high SNR region which seems to be beneficial to achieve higher spectral efficiency in 6GR. Therefore, the study of non-uniform constellation for 6GR is possible. RAN4 shall be involved in a such study since there is a direct impact on demodulation, and coexistence RF parameters (ex.: MPR, EVM).

Proposal 2: RAN4 shall be early involved in any potential study of non-uniform constellations for 6GR.

  - Frame structure - numerology

For frame structure, key considerations include allowing efficient multi-RAT spectrum sharing (MRSS) between 5G and 6G and minimizing options for the same functionality.

MRSS operation between 5G and 6G is most efficient if the time units of 5G are reused as much as possible in 6G and since no obvious benefit is identified for defining different values for the symbol-time units in 6G, it is suggested that the following can be assumed:

Proposal 3: 6GR symbol durations, slot definition, subframe and frame are the same as NR.

The same consideration leads to reusing same sub-carrier spacing (SCS) values as NR for a given frequency band. In addition, as identified in the SID objectives, configuring a UE with multiple SCS for the same band (or sub-range) is to be avoided for simplicity and efficiency of cell search/detection.

Proposal 4: 6GR supports at least the SCS values that are supported in NR.

Proposal 5: Support configuration of single SCS per band for a UE.

6GR is to support applications requiring very low latency and high reliability. This requires that, like NR, it supports symbol-level scheduling. In frequency domain, the basic scheduling unit of PRB of 12 subcarriers can be reused to facilitate MRSS operation. However, the symbol level scheduling shall be maintained.

Proposal 6: 6GR supports symbol level scheduling.

  - Sync raster and SSB structure

Sync raster

5G NR introduced global frequency raster (i.e., sync raster) for cell search. The frequency position of a SSB, the corresponding GSCN and size of sync raster are pre-defined for each specified frequency range. An example of GSCN parameters and sync raster for above-3 MHz channel bandwidth is as below [4].

Table 1. An example of GSCN parameters and sync raster for above 3MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} | 3N + (M-3)/2 | 22 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| 24250 – 100000 | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

5G NR has specified 26.6k entries of synchronization raster to enable flexible deployments for large number of bands supported by 5G NR. While many UE vendors have optimized the cell search process to reduce the overall initial access time, the large number of sync entries still represent high complexity and high energy consumption burden for the UEs. For 6G, there would be benefits to study further methods to reduce search complexity and time by enabling smaller sync raster entries. For example, methods could lean on leveraging network and/or UE specific information (such as supported bandwidth) for reduction of potential sync raster entries.

Proposal 7: Study sync raster enhancement to reduce cell search complexity and search time.

SSB structure

5G NR SSB is transmitted over four consecutive OFDM symbols and the SSB bandwidth is 20 RBs. One SSB includes PSS, SSS and PBCH with a time and frequency domain multiplexing structure of the SSB is as shown below. The PSS and SSS sequence are each mapped to 127 subcarriers and transmitted over a bandwidth of 12 RBs at the 1st and 3rd symbol. The PBCH transmission bandwidth is 20 RBs at the 2nd and 4th symbol and 48 subcarriers above and below SSS transmission at the 3rd symbol.

6GR is envisioned to support a wide range of UE device types with different UE capabilities in a single RAT. Certain UE type (e.g., low-cost RedCap UE) may only support a minimum channel bandwidth e.g., 3 to 5 MHz and thus may not be able to receive a 6G SSB based on same 5G SSB structure.

To address this, we think a unified framework with flexible SSB structure can be helpful. The framework can include a common SSB structure with a bandwidth based on the supported minimum channel bandwidth (e.g., 3 MHz) so it can be received by all device types supported in 6GR, i.e. common for all device types. The common SSB can include a basic PBCH and additional PBCH transmissions can be multiplexed in time domain with the common SSB within the same bandwidth. For advanced device types capable of operating in large bandwidth, the additional PBCH can be multiplexed with common SSB in frequency domain to reduce the access latency as shown in Figure 1.

Proposal 8: 6G radio shall support a common SSB in frequency domain for multiple device types.

The SSB periodicity

In 5G NES feature, significant energy saving gain (ESG) is observed in evaluation of time domain adaptation of always-on (AO)-SSB transmissions. The adaptation includes for example sparse AO-SSB transmission with a 160-ms periodicity instead of 20-ms periodicity. To standardize this sparse SSB transmission is however challenging in 5G NR, because all legacy UEs are designed and implemented with an assumption of 20-ms periodicity in cell search as specified in TS 38.213.

For example, for 6G, adaptive initial access signaling can be beneficial using OD-SSB transmissions adapted to the UE request or network configuration in combination of sparse AO-SSB transmission with a periodicity of at least 160ms. However, this may impact the RRM requirements and shall be part of RAN4 study, if a such approach is adopted for study in RAN1.

Proposal 9: RAN4 shall be involved in any potential study of the initial access performance that includes e.g. a combination of sparse SSB transmission with 160-ms or larger periodicity and adaptive on-demand SSB transmissions.

## Bandwidth management

Minimum and maximum bandwidth

NR supports a minimum channel bandwidth of 5 MHz in R15. In R18, 3GPP specified a new 3 MHz channel bandwidth to support operation in bands used for rail communications.

Since 6GR study is to target a scalable and forward compatible design for diverse device types, the decision on minimum bandwidth necessary for initial access needs to consider the needs of the most basic devices (e.g. LPWA) from the start. Setting a lower minimum bandwidth allows supporting lower complexity devices. However, the minimum bandwidth significantly impacts the design of basic signals (e.g. sync signals, PBCH) and associated functionality (e.g. cell search, idle mode operation) which also needs to be considered. A larger bandwidth results in higher frequency diversity and possibly faster acquisition. For 6GR, a minimum bandwidth of 3 MHz should satisfy future market needs for low-power devices while still enabling high performance for advanced devices.

Proposal 10: The minimum bandwidth for initial access is 3 MHz.

NR supports a maximum channel bandwidth of 100 MHz in FR1 (and 400 MHz in FR2 in R15). Such maximum bandwidth is feasible with implementation with FFT size of 4096 and subcarrier spacing of 30 kHz. For 6GR one can consider that implementations can support a larger FFT size of 8192. This leads to a maximum channel bandwidth of 200 MHz.

Proposal 11: The maximum channel bandwidth for 6GR is 200 MHz in FR1 and FR3.

Fast bandwidth adaptation

Dynamic BWP switching can be used as bandwidth adaptation mechanism in NR. Dynamic BWP switching was designed not only for this purpose but also for switching to a different part of the carrier possibly optimized for a different type of traffic. The multi-purpose nature of the BWP switching concept in NR introduced some limitations from the perspective of fast bandwidth adaptation as there is not necessarily overlap between BWPs and many parameters are BWP-specific. In addition, switching to a wider BWP proved difficult as the DCI size set based on source BWP is lower than what is required to describe initial transmission on the target BWP.

The design should aim at maximizing robustness and minimizing delay for the BWP switching. Another goal should be to provide channel information as fast as possible to the scheduler when the active bandwidth is expanded.

Proposal 12: 6GR supports robust, fast, bandwidth adaptation scheme focusing on power efficiency.

- Conclusion

This contribution discussed on the 6GR systems parameters from the perspective of RAN4. Based on the discussions, we propose the following:

Waveform:

Proposal 1: Potential study of new 6G NR waveforms involving RAN4 shall include at least the following KPIs: better PAPR, CM, and Emission Mask, OBE, full compatibility with the current 5G CA and DC band combinations.

Modulation

Proposal 2: RAN4 shall be early involved in any potential study of non-uniform constellations for 6GR.

Frame Structure and Numerology

Proposal 3: 6GR symbol durations, slot definition, subframe and frame are the same as NR.

Proposal 4: 6GR supports at least the SCS values that are supported in NR.

Proposal 5: Support configuration of single SCS per band for a UE.

Proposal 6: 6GR supports symbol level scheduling.

Sync Raster and SSB structure

Proposal 7: Study sync raster enhancement to reduce cell search complexity and search time.

Proposal 8: 6G radio shall support a common SSB in frequency domain for multiple device types.

Proposal 9: RAN4 shall be involved in any potential study of the initial access performance that includes e.g. a combination of sparse SSB transmission with 160-ms or larger periodicity and adaptive on-demand SSB transmissions.

Channel Bandwidth

Proposal 10: The minimum bandwidth for initial access is 3 MHz.

Proposal 11: The maximum channel bandwidth for 6GR is 200 MHz in FR1 and FR3.

Proposal 12: 6GR supports robust, fast, bandwidth adaptation scheme focusing on power efficiency.

- References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

[2] RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

[3] 3GPP TR 38.914: Study on 6G Scenarios and Requirements, v0.1.0.

[4] 3GPP TS38.101: NR; User Equipment (UE) radio transmission and reception, v19.2.0