3GPP TSG-RAN WG4 Meeting #116BIS							   R4-2513075

Prague, Czech, 13th – 17th Oct, 2025

Agenda Item: 	8.3

Source: 	MediaTek Inc.

Title: 	Views on 6G system parameters

Document for: 	Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In this paper, we provide our views on the 6G system parameters with the focus on the following aspects:

- Scalability for diverse device types

- FFT, numerology, min/max CBW, #Rx, #Tx, modulation

- Spectrum utilization

- Synchronization signal and raster

- Duplexing

# 2	Scalability for diverse device types

## 6G Candidate device types

RAN#109 Plenary endorsed the following [2].

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

Conclusion: proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) in # 109 Plenary Meeting report

Based on RAN1 and RAN plenary decision, RAN4 needs to wait the RAN plenary guidance which device types will be supported in 6G study and which parameters will be considered to categorize the supporting device types in 6G study.

If some early discussion is needed to evaluate 6G system performance and coexistence, RAN4 can consider the HHUE (e.g. smartphone) and CPE (e.g. FWA) in 6G day 1.

Proposal 1: If needed to evaluate 6G system performance and coexistence, RAN4 can consider HHUE (e.g. smartphone) and CPE (e.g. FWA) capabilities and 6G BS antenna configurations.

Even if we have a very comprehensive discussion on which device type to be introduced in 6G, it is impossible to perfectly predict the future. Therefore, a forward compatible framework is very important. We expect that some vertical industries may request to consider other device types in the future. 3GPP spec needs to be flexible to accommodate any new market trends.

Proposal 2: Introduce a forward compatible device type framework to accommodate the market need for potential new device types in the future.

## Initial capability for HHUE and CPE (FWA) in 6G day 1

We consider in the Table 1 below the need for at least the following kinds of device characteristics to be considered as representative examples to progress the 6G radio design performance.

Table 1. Examples of possible 6G device capabilities for around 7GHz

| Device types | Level of constraints | Example Capabilities Envelope |  |  |
| --- | --- | --- | --- | --- |
|  |  | Duplex | MIMO layers / Antennas | Default power class (dBm) |
| Reference 2, e.g. FWA form factor | Low | FDD, TDD | At least 4L/ At least #4T/#4R | 29 |
| Reference 1, e.g. handheld form factor | Medium | FDD, TDD | At least 4L/ At least #4T/#4R | 26 |

Table 2. Potential Min. Tx/Rx Capability per Handheld device and spectrum ranges

| FR1 low band (Sub-1GHz) | FR1 mid band (1.7~3GHz) | FR1 high band (3~6GHz) | U6G + around 7GHz (6~8.4GHz) | Around 15GHz (10~15GHz) |
| --- | --- | --- | --- | --- |
| 1T/2R | 2T/4R | 2T/4R | 4T/4R | 4T/4R |

The values in Table 2 are provided for information at this stage but may need more formal discussion once Device Types are further discussed by RAN plenary

Proposal 3: Consider the inputs in Table 1 and Table 2 for the 6GR coexistence evaluation and system performance analysis with different antenna elements in 6G BS and default power class of 6G UE.

# 3	FFT, numerology, Max/min Channel Bandwidth, #Rx, #Tx, modulation

<FFT size>

8192 should be considered, which can achieve 200MHz with 30KHz SCS. Further increasing the FFT size will demand a very high UE complexity for which the resulting benefit vs complexity trade-off does not seem well-justified.

Proposal 4: In 6G, a maximum 8192 FFT size for single carrier is supported.

<Numerology>

In NR, we introduced multiple numerologies for FR1 and FR2, e.g., Table 5.3.2-1 in both TS 38.101-1 and TS 38.101-2 as below.

In parallel, we also define the SCS and BW applicable in each band in Table 5.3.5-1 in both TS 38.101-1 and TS 38.101-2. Such framework can be kept for 6GR as well.

Proposal 5: Tables for band-specific SCS and CBW can be defined in 6GR spec. However, the detail number of RB and supported CBW are FFS.

Current NR spec allows multiple numerologies in one band – however, typical deployments only have a single numerology per band. Furthermore, in NR different numerologies for SSB and data are allowed which are not used in used in typical deployments. The 6G WG SID [1] states the following objective, therefore we suggest 6G to consider only single SCS for both SSB and data per band.

“Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]”

Observation 1: In NR, multiple SCS were introduced in one band, and different SCS is allowed for SSB and data. However, these are not used in typical deployments.

Proposal 6: In 6G, consider a single numerology for both SSB and data per band.

< Max. Channel bandwidth>

Based on the above-mentioned proposals for single numerology and a maximum FFT size of 8192 for a single carrier, the maximum CBW for different frequency ranges is proposed as shown in the following table 3.

Table 3. Suggested maximum CBW and SCS for 6G in different frequency ranges

| Spectrum Range | 410-1000MHz | 1000-5925MHz | 5.925- 10GHz | 10- 15.35GHz | 15.35-24.25GHz | 24.25-52.6GHz |
| --- | --- | --- | --- | --- | --- | --- |
| Max CBW (MHz) | 50 | 100(FDD), 200(TDD) | 200 | [200/400] | TBD | 800 |
| SCS (kHz) | 15 | 15(FDD), 30(TDD) | 30 | [30/60] | TBD | 120 |
| Max FFT size | 4096 | 8192 | 8192 | 8192 | 8192 | 8192 |

Proposal 7: Suggested maximum CBW and SCS for 6G in different frequency ranges are shown in Table 3.

<Min. Channel Bandwidth>

In 5G NR, a 3 MHz minimum channel bandwidth is specified for certain bands (3GPP TS 38.101-1, Table 5.3.5-1). It is expected such spectrum allocation should be catered for in 6G as well. It is thus proposed that the minimum 6GR channel bandwidth should follows the NR approach: 5 MHz nominal, with 3 MHz permitted in certain bands.

Proposal 8: Follow the NR approach: 5 MHz nominal, with 3 MHz permitted in certain bands.

<Modulation>

RAN1 currently is discussing some potential enhancement on the modulation, e.g., constellation shaping. The impact to RAN4 requirements is not very clear at this stage. We suggest waiting for RAN1 to achieve more conclusions before discussing the corresponding impact due to new modulation.

Proposal 9: Wait for RAN1 conclusions before discussing the requirement impact of potential new modulation in 6G.

# 4	Spectrum utilization

In NR, the basic rule is that carrier spectrum utilization is assumed to be higher than 90%, and the number of RBs for various CBWs with different SCS values is specified as shown below. For 6G spectrum utilization, we think it should be improved compared to 5G. Therefore, we could study the feasibility of enhancing spectrum utilization in 6G.

Table 4. Maximum transmission bandwidth configuration NRB and corresponding spectrum utilization

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| SU(%) | 90.00 | 90.00 | 93.60 | 94.80 | 95.40 | 95.76 | 96.00 | 96.69 | 97.20 | 96.80 | 97.20 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| SU(%) | N/A | 79.20 | 86.40 | 91.20 | 91.80 | 93.60 | 93.60 | 94.63 | 95.40 | 95.20 | 95.76 | 97.20 | 97.20 | 97.65 | 98.00 | 98.28 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| SU(%) | N/A | N/A | 79.20 | 86.40 | 86.40 | 89.28 | 91.20 | 90.51 | 91.80 | 92.80 | 93.60 | 94.80 | 95.66 | 96.30 | 96.80 | 97.20 |

Observation 2: In NR, the basic rule is that carrier spectrum utilization is assumed to be higher than 90%.

Proposal 10: Study the feasibility to enhance spectrum utilization in 6G.

# 5	Synchronization signal and raster

Initial access refers to the process by which UE discovers, synchronizes, and connects to a cell. This process relies on the transmission and detection of synchronization signals, which are typically mapped onto specific frequency locations referred to as sync raster points. In the legacy systems (e.g., NR), the design of sync raster relied on multiple factors including the minimum assumption of the channel bandwidth CBW (for different frequency ranges), the synchronization signals bandwidth (e.g., SSB BW in NR) and the channel raster. Such design resulted in a highly dense number of sync raster entries.

From the field readings and observations, it appears that the majority of sync raster entries defined by 3GPP for the existing operating bands (n1, n2, n3, etc.) are not being used in the real-world settings. In Table 5, we provided our field log results from multiple countries/regions. From the result, only about 10% of these sync raster entries can be seen to be utilized by the operators for allowing the UE to camp on them. However, from UE perspective, UE still need to search within all these defined sync raster entries by 3GPP, which results in longer and unnecessary delay for initial access and a higher power consumption. Therefore, the over-designed issue of sync raster in the legacy cellular systems needs to be addressed and RAN4 need to study more practical solutions to overcome inefficient high density sync raster design in 6G.

Table 5. Number of 5G sync raster points defined in spec and detected from real field for different bands

| band | # in spec | # in use | Ratio | band | # in spec | # in use | Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 141 | 22 | 16% | n38 | 9 | 1 | 11% |
| n2 | 141 | 3 | 2% | n40 | 228 | 3 | 1% |
| n3 | 177 | 1 | 1% | n41 | 158 | 23 | 15% |
| n5 | 54 | 6 | 11% | n38 | 9 | 1 | 11% |
| n7 | 165 | 3 | 2% | n70 | 52 | 1 | 2% |
| n8 | 78 | 1 | 1% | n71 | 78 | 3 | 4% |
| n14 | 15 | 2 | 13% | n77 | 619 | 10 | 2% |
| n25 | 153 | 1 | 1% | n78 | 341 | 34 | 10% |
| n28 | 102 | 15 | 15% | n79 | 26 | 4 | 15% |
| n34 | 3 | 1 | 33% |  |  |  |  |
| Note: Field log from Australia, Brazil, Canada, China, Estonia, Finland, France, Germany, Guam, India, Japan, Kuwait, Malaysia, Pakistan, Qatar, Saudi Arabia , Singapore, Spain, South Africa, Sweden, Thailand, United Arab Emirates, UK, USA, Vietnam, and other EU countries |  |  |  |  |  |  |  |

Observation 3: NR sync raster is over designed and only about 10% of sync raster entries are utilized in real-field environment.

Proposal 11: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access.

In addition to the high-density issue, the delay of initial access is associated with the structure of the synchronization signals SSB (e.g., periodicity and BW). In 6G, there is a common intention to increase SSB periodicity > 20ms (e.g., 40ms, 80ms, or 160ms) for network energy saving purposes. RAN1 is evaluating the energy efficiency benefits of increasing the periodicity and in which scenarios this may be applied. Increasing the periodicity also introduces the risk of proportionally increasing the delay associated with sync raster and initial access, potentially degrading user experience. For example, if SSB periodicity becomes 160ms in 6G, the delay of initial access becomes eight times longer than the delay in 5G (assuming the same sync raster design is reused). Therefore, in 6G, we need to study the consequences of extending SSB periodicity on initial access delay and sync raster and how to mitigate this issue while ensuring initial access delay in 6G is not worse than 5G.

Observation 4: Increasing SSB periodicity to be larger than 20ms would potentially increase the delay associated with sync raster in 6G, e.g. a periodicity of 160ms could lead to 8x initial access delay.

Proposal 12: 6G sync raster design needs to ensure its initial access delay is not worse than 5G.

Given these technical issues, there is a need to consider new sync raster solutions in 6G that can achieve a more balanced design, compatible with the evolving structure of SSB in 6G in terms of periodicity and bandwidth and at the same time can maintain or minimize the delay associated with sync raster with respect to 5G. In addition, such solutions should accommodate diverse operator requirements across a wide range of deployment scenarios and frequency bands.

In NR, sync raster with the corresponding GSCN is defined as shown in Table 5.4.3.1-1 in TS38.101-1. The basic principle is to always guarantee one SSB for arbitrary CBW and channel raster. It considers three main parameters: minimum assumption of channel bandwidth CBW (transmission bandwidth, TBW), SSB BW and channel raster, as given by the following formula.

Sync raster step = TBW – SSB BW + channel raster step

In addition, the corresponding GSCN for each operating band is defined in Table 5.4.3.3-1 in TS38.101-1.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1 | 3N + (M-3)/2 | 22 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200. |  |  |  |

Table 5.4.3.3-1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| … | … | … | … |

One approach to address the high-density issue of sync raster and to accommodate a longer SSB periodicity in 6G is to study a larger granularity for channel bandwidth assumption for 6G (i.e., optimizing TBW in the above formula). In NR, a minimum CBW of 5MHz is assumed for sub 3GHz frequency range as a baseline. While such assumption is useful to accommodate operators with small CBW 5MHz, this, however, leads to a very dense sync raster and significant delay, even though this is unnecessary for most of the CBWs which are larger than 5MHz in the field. Therefore, we propose to study the design of a coarse sync raster in 6G, in which a larger CBW is assumed as the baseline (i.e., unlike the legacy where the finer CBW is the baseline). Other sync raster with smaller or intermediate CBW assumptions can also exist and used by the UE when initial access based on the coarse sync raster cannot be achieved. This coarse sync raster also tries to address and compensate for any additional delay that might be introduced by applying a larger SSB periodicity than 20ms.

Observation 5: A minimum CBW of 5MHz for sub 3GHz frequency range is assumed as a baseline for sync raster design in NR.

Proposal 13: Study a coarse sync raster(s) based on a larger channel bandwidth(s) as a baseline assumption for 6G.

Another approach that can also be studied in 6G is the consideration of a coarse sync raster based on a larger step size. It can be noticed from Table 5.4.3.3-1, a step size <1> is used in the spec as a baseline for the sync raster entries GSCN for most of the operating bands. Although this step size is beneficial for accommodating the minimum assumption of a 5MHz channel bandwidth (CBW), it also results in a very dense sync raster and causes significant delays. Therefore, we also propose to study multiple sync raster with different step sizes, in which the baseline (coarse) sync raster is the one associated with the largest step size (unlike the legacy where the finer step size is the baseline).

Observation 6: In NR, a fine step size <1> is assumed as a baseline for GSCN sync raster entries.

Proposal 14: Study coarse sync raster(s) based on larger step size(s) as baseline assumption for 6G.

In the above discussion, we highlighted the importance of revisiting the legacy baseline assumptions in 6G (e.g., step size and CBW) to achieve more efficient and practical solutions based on a coarse sync raster design. On the other hand, we also learned from NR sync raster design that SSB BW is a major factor contributing to the high density of sync raster entries. For instance, in the sub-3GHz frequency range, the assumed minimum CBW of 5MHz and SCS 15kHz yield TBW 4.5MHz and SSB BW 3.6MHz, thus the subtraction of TBW – SSB BW results in a small value. In other words, given the NR principle of the sync raster as in the formula below, the larger the SSB bandwidth, the denser the sync raster becomes.

- Sync raster = TBW – SSB BW + channel raster

With this observation, we would like also to study the feasibility of associating sync rater design in 6G with PSS bandwidth instead of SSB BW since PSS BW is considerably smaller than the total bandwidth of the SSB (in NR, PSS BW is 12 PRBs compared to 20 PRBs for SSB BW). The consideration of PSS BW can help to reduce the sync raster density by at least 50% for sub 3GHz frequency range and even higher reduction for above 3GHz frequency ranges (while keeping the legacy assumptions of minimum CBW, SCS etc.).

- Sync raster = TBW – PSS BW + channel raster

Observation 7: SSB bandwidth is one of the main contributors for having a highly dense sync raster in NR.

Proposal 15: Study the feasibility of associating sync rater design in 6G with PSS bandwidth instead of SSB bandwidth.

<Harmonized design for TN and NTN>

For the agreements [2] in RAN plenary, RAN also had agreements on RAN4’s involvement to discuss the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective as following.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

On this minimum BW (which impacts the sync raster design), one very important aspect to pay attention to is the harmonization of TN and NTN. From NR experience, we prefer to have disjoint sync raster for TN and NTN, because NTN SSB may suffer from larger uncertainty of the Doppler. RAN4 should consider this point when studying the minimum channel bandwidth and sync raster.

Proposal 16: When discussing the minimum channel bandwidth and sync raster, RAN4 needs to consider both TN and NTN.

# 6	Duplexing

6G will need to operate on paired spectrum and unpaired spectrum. For paired spectrum, FD-FDD operation is expected to be catered for at both network and UE side. In addition, Half-Duplex operation at the UE side should be considered to accommodate for device complexity reduction, potentially depending on the frequency band.

Proposal 17: Target both FD-FDD and HD-FDD operation at UE side for paired bands.

For unpaired spectrum in 6G, two duplexing schemes should be studied: TDD and SBFD. In TDD operation, a semi-static indication of the TDD pattern serves as a baseline. This pattern can be aligned among base stations - whether they belong to the same operator or different operators - resulting in synchronous TDD. Alternatively, the pattern may not be aligned among base stations, leading to asynchronous TDD. Asynchronous TDD poses a risk of inter-gNB and inter-UE Cross-Link Interference (CLI), for which RAN1 should investigate effective CLI mitigation and handling procedures.

NR specifications support dynamic TDD from Rel-15 by defining “Flexible” symbols, where the transmission direction is not predetermined. Additionally, Slot Format Indication (SFI) was introduced to enable the network to dynamically change the TDD pattern for “Flexible” symbols. Enabling flexible symbols and dynamic SFI resulted in significant impact on UEs, as they had to manage collisions between UL and DL scheduling/configurations and monitor SFI. However, due to a lack of use cases and the presence of high co-channel CLI, dynamic TDD was not deployed in 5G networks. CLI in dynamic TDD is typically very high because the DL/UL transmission on specific PRBs of aggressor gNB/UE interferes with the same PRBs of the UL/DL reception at a victim gNB/UE, with no frequency isolation unlike with SBFD.

Observation 8: For unpaired spectrum, TDD will be required. However, TDD flexible symbols caused lots of complexity without any clear need or benefit in practical deployment.

Proposal 18: Do not consider dynamic TDD or flexible symbols for 6GR.

gNB-SBFD has been specified for NR in R19 and can provide significant benefits in 6G. Compared to TDD, gNB-SBFD offers advantages in terms of UL coverage, throughput, and latency. With the introduction of new “greenfield” bands in 6G, network side-SBFD can be deployed with coordination among gNBs to avoid the issue of co-channel CLI with neighbouring non-SBFD legacy gNBs in the same carrier. However, to achieve this and to ensure optimal SBFD awareness, it is important that SBFD is catered in the initial 6G basic design. Prior to that, SBFD operation will need to be fully catered for in the 6G study phase.

Observation 9: SBFD improves the system performance in unpaired spectrum in terms of latency and UL coverage/throughput, and performance can be maximised in new bands with SBFD aware equipment.

Proposal 19: Study TDD enhanced with SBFD as a fundamental 6G design component for unpaired bands.

UE-SBFD can further enhance performance compared to network-side SBFD by eliminating the half-duplex limitation at the UE, allowing simultaneous transport of both downlink and uplink traffic for the same UE.

It is important to highlight (especially for RAN4) that UE-SBFD can be “opportunistically” enabled in favourable conditions to avoid high self-interference, such as with narrow uplink allocation and low transmission power. Additionally, more dynamic “frequency separation” can be ensured between the uplink and downlink allocations at the UE to reduce self-interference (and therefore minimise the UE design complexity to cope with self-interference), without impacting system spectral efficiency.

Implementing SBFD at both the gNB and UE can significantly improve uplink latency performance and system capacity compared to TDD operation, which is essential for latency-sensitive applications like XR (see Figure 1).

Figure 1. UL performance gains with SBFD: Dense Urban, XR Traffic (10Mbps data-rate, 30ms PDB), FR1/4GHz.

Observation 10: UE-SBFD further improves the capacity for latency-bound services and coverage by eliminating the trade-off between DL and UL transmissions.

Proposal 20: Study UE-side SBFD on top of network-side SBFD for 6G.

# 7	Summary

In this paper, we provide our views on 6G system parameters with the following observations and proposals for individual sub-topics:

<Device type>

Proposal 1: If needed to evaluate 6G system performance and coexistence, RAN4 can consider HHUE (e.g. smartphone) and CPE (e.g. FWA) capabilities and 6G BS antenna configurations.

Proposal 2: Introduce a forward compatible device type framework to accommodate the market need for potential new device types in the future.

Proposal 3: Consider the inputs in Table 1 and Table 2 for the 6GR coexistence evaluation and system performance analysis with different antenna elements in 6G BS and default power class of 6G UE.

<CBW, FFT, numerology, #Rx, #Tx>

Proposal 4: In 6G, a maximum 8192 FFT size for single carrier is supported.

Proposal 5: Tables for band-specific SCS and CBW can be defined in 6GR spec. However, the detail number of RB and supported CBW are FFS.

Observation 1: In NR, multiple SCS were introduced in one band, and different SCS is allowed for SSB and data. However, these are not used in typical deployments.

Proposal 6: In 6G, consider a single numerology for both SSB and data per band.

Proposal 7: Suggested maximum CBW and SCS for 6G in different frequency ranges are shown in Table 3.

Proposal 8: Follow the NR approach: 5 MHz nominal, with 3 MHz permitted in certain bands.

Proposal 9: Wait for RAN1 conclusions before discussing the requirement impact of potential new modulation in 6G.

<Spectrum utilization>

Observation 2: In NR, the basic rule is that carrier spectrum utilization is assumed to be higher than 90%.

Proposal 10: Study the feasibility to enhance spectrum utilization in 6G.

<Synchronization signal and raster>

Observation 3: NR sync raster is over designed and only about 10% of sync raster entries are utilized in real-field environment.

Proposal 11: Study how to overcome the high-density design of sync raster in 6G to achieve more efficient delay and power consumption for initial access.

Observation 4: Increasing SSB periodicity to be larger than 20ms would potentially increase the delay associated with sync raster in 6G, e.g.

Proposal 12: 6G sync raster design needs to ensure its initial access delay is not worse than 5G.

Observation 5: A minimum CBW of 5MHz for sub 3GHz frequency range is assumed as a baseline for sync raster design in NR.

Proposal 13: Study a coarse sync raster(s) based on a larger channel bandwidth(s) as a baseline assumption for 6G.

Observation 6: In NR, a fine step size <1> is assumed as a baseline for GSCN sync raster entries.

Proposal 14: Study coarse sync raster(s) based on larger step size(s) as baseline assumption for 6G.

Observation 7: SSB bandwidth is one of the main contributors for having a highly dense sync raster in NR.

Proposal 15: Study the feasibility of associating sync rater design in 6G with PSS bandwidth instead of SSB bandwidth.

Proposal 16: When discussing the minimum channel bandwidth and sync raster, RAN4 needs to consider both TN and NTN.

<Duplexing>

Proposal 17: Target both FD-FDD and HD-FDD operation at UE side for paired bands.

Observation 8: For unpaired spectrum, TDD will be required. However, TDD flexible symbols caused lots of complexity without any clear need or benefit in practical deployment.

Proposal 18: Do not consider dynamic TDD or flexible symbols for 6GR.

Observation 9: SBFD improves the system performance in unpaired spectrum in terms of latency and UL coverage/throughput, and performance can be maximised in new bands with SBFD aware equipment.

Proposal 19: Study TDD enhanced with SBFD as a fundamental 6G design component for unpaired bands.

Observation 10: UE-SBFD further improves the capacity for latency-bound services and coverage by eliminating the trade-off between DL and UL transmissions.

Proposal 20: Study UE-side SBFD on top of network-side SBFD for 6G.

Proposal 21: RAN4 to identify whether any issue has potential cross-WG impact and arrange the required action, if any.

# 8	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)