3GPP TSG-RAN WG4 Meeting #116bis				                                                             R4-2513097

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	7.12.3

Source: 	OPPO

Title: 	Discussion on RAN4 requirements for RRM measurement prediction

Document for:	Approval

- Introduction

The WID for Rel-20 AI mobility was agreed in [1] and revised in [2]. The objectives of core part are as following:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

According to the objectives, RAN4 need to define core requirements for the supported use cases and LCM related requirements for LCM functionality management for UE sided model. In this contribution, we will discuss the identified requirements for measurement prediction and give our proposals.

- Discussion

2.1 Scenarios and sub-use cases

According to the WID, the following cases are supported for L3 cell level measurement prediction for which RAN4 needs to define the requirements:

- Temporal domain prediction case A and case B

- Frequency domain prediction for co-located cases

Based on the studies in Rel-19 SI, the above cases are described in TR 38.744 [3]. As cited below, scenarios 1~5 are included in the work item while scenario 6 is precluded.

Table 1: Evaluation scenarios for measurement prediction

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Proposal 1: RAN4 to define requirements for measurement prediction for scenarios 1~5.

| Scenario number | Evaluation scenario | Methodology |
| --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency temporal domain case A | intra-cell |
| 2 | FR1 to FR1 intra-frequency temporal domain case B | Intra-cell |
| 3 | FR1 to FR1 inter-frequency (frequency domain) | Inter-cell |
| 4 | FR2 to FR2 intra-frequency temporal domain case A | Intra-cell |
| 5 | FR2 to FR2 intra-frequency temporal domain case B | Intra-cell |

For each scenario, RAN2 has defined 3 sub-use cases as below:

3 sub-use cases are considered for cell-level RRM measurement prediction: - RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - RRM sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

And for inter-frequency prediction, both cluster-based and single-cell-based approaches are considered. But based on the conclusions in TR 38.744, it is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length. We think there is no need to differentiate requirements for different sub-use cases or different implementation. i.e., single set of requirements can be defined for all the sub-use cases. To simplify the discussion, the requirements can be discussed based on sub-use case 2 and applied to other sub-use cases unless specific issues are identified. And for inter-frequency prediction, the requirements can be defined based on single-cell-based approach.

Proposal 2: RAN4 to define single set of requirements applied to all sub-use cases.

Proposal 3: RAN4 to define single set of requirements applied to single-cell-based and cluster-based approach for frequency domain prediction.

2.2 Measurement and prediction related requirements

In Rel-19 NR_AIML_air, the requirements for AI/ML based beam management are discussed and the following delay requirements are defined:

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

For AI mobility, before we reusing the framework of delay requirements in beam management, we need to clarify the concepts firstly. Figure 1 as below illustrates the procedures and aspects to be considered in delay requirements based on our understanding including time for measurement, for inference and for the first available reporting resource.

Figure 1: illustration of prediction reporting delay

Based on the above understanding, we think measurement delay and prediction delay for AI mobility can be defined with the similar concepts with beam management. I.e., the measurement delay is the observation period to make one prediction report (i.e., the time to get prediction input), and the prediction delay is the time to get the report results (i.e., prediction output) including measurement delay and inference delay.

But there is one thing different for AI mobility. RAN2 agreed in study phase that it is up to UE’s implementation to decide on model related choices including OW length, i.e., the observation period for UE to make one prediction report depends on UE. So we think the measurement delay can be defined as minimum observation period which can make one prediction report.

Proposal 4: RAN4 to define measurement period and prediction delay requirements for AI mobility with the following definition:

- Measurement period is the minimum observation period during which UE samples the reference signals to make 1 prediction report.

- Prediction delay is the time to get prediction report which includes measurement period and inference delay.

But for prediction reporting delay, we think further discussion is needed. In legacy measurements, the reporting requirements are defined for event triggered reporting and the first report in event triggered periodic measurement reporting. The reporting delay excludes the delay which caused by no UL resources being available for UE to send the measurement report on and excludes the delay for L3 filtering, i.e., only includes the delay for cell identification.

### 9.2.4 Measurement Reporting Requirements #### 9.2.4.1 Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). #### 9.2.4.2 Event-triggered Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2.4.3. #### 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_with_index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. A cell is detectable only if at least one SSBs measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

For AI mobility, we think the same definition of reporting delay as legacy measurement can be reused, i.e., the prediction reporting delay is defined as the time between an event that will trigger a prediction report and the point when the UE starts to transmit the report over the air interface. According to this definition, the time for the first available reporting resource is not needed and the delay is same as the prediction delay.

Proposal 5: RAN4 to define prediction reporting delay for event triggered reporting and the first report in event triggered periodic reporting.

Proposal 6: For the prediction reporting delay requirements to be defined, the time for the first available reporting resource and time for L3 filtering are not included.

For the inference delay used in above requirements which depends on the AI model implementation, we think it is similar for at least measurement related use cases. Since it is still under discussion in Rel-19 AI based beam management, to avoid parallel discussion on the same issue, we can postpone the discussion and wait for the progress there.

Proposal 7: For details of inference delay, wait for the progress of beam management in Rel-19 NR_AIML_air.

For the measurement delay used in above requirements, it is related to the different scenarios including temporal domain prediction case A and case B, and frequency domain prediction.

For temporal domain measurement prediction case A illustrated in Figure 1 as below, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. And the samples with predicted results will be still actually measured.

Figure 1: Example of intra-frequency temporal domain case A

Since the measurements are not reduced and the measurement behaviour is same as legacy for case A, we understand the legacy measurement requirements can be reused.

Proposal 8: For temporal domain prediction case A, the legacy measurement period requirements can be reused.

For temporal domain prediction case B illustrated in Figure 2 and Figure 3 as below, measurement results in PW are predicted by historical measurement result(s) in OW. The samples with predicted results will be skipped when performing measurement. And the measurement reduction rate is defined as MRRT assuming same length of measurement time instances, where,

- MRRT = skipped measurement time instances / total measurement time instances

Figure 2: Skipping pattern example 1 of intra-frequency temporal domain case B

Figure 3: Skipping pattern example 2 of intra-frequency temporal domain case B

Based on the above examples, it is noted that UE may have different skipping patterns for different MRRT or even same MRRT for case B. And for the measurement configuration of case B, RAN2 has the following agreements:

Following inference parameters can be configured to UE for inference and assessing applicability: - PW length for intra-frequency temporal domain case A; - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction; - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement); - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available); - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction; - Optional associated ID.

It means that NW will only configure the available SSB resources and how to measure and skip the resources is based on UE implementation. In our understanding, the different MRRT and different skipping patterns (e.g., whether the measurements in observation window are continuous or not) will impact the measurement requirements. For example, for the skipping pattern in figure 2, the measurements are continuous and the behavior is same as legacy which means the legacy measurement can be reused. But for the skipping pattern in figure 3, the measurement period needs to be extended if the measurement configuration includes the resources to be predicted.

Proposal 9: For temporal domain prediction case B, define the measurement period requirements considering the assumptions on MRRT, skipping patterns and measurement configurations.

For frequency domain measurement prediction, only co-located scenario is included, i.e., the cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighbouring site. And for the prediction method, both signal cell prediction and cluster-based prediction are considered. Based on the above RAN2 agreements on inference parameters, network will configure the measured frequency carrier and predicted frequency carrier information for inter-frequency prediction. With this information, UE behavior on the configured measured frequency carrier can be same as legacy and the existing requirements can be reused.

Proposal 10: For frequency domain prediction, the legacy measurement requirements can be reused by clarifying in CSSF that the frequency layers configured for prediction are not counted.

2.3 LCM related requirements

According to the WID, for the LCM related discussion in AI mobility, the LCM framework for Rel-19 AI/ML for NR air interface WI can be reused as much as possible. Based on the discussion in Rel-19 AI/ML for NR air interface, the following agreements are reached and no other delay requirements except AI/ML functionality activation delay.

Issue 1-1-3: Whether to create delay requirements for functionality activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. General LCM principles: Agreement: - No need to create deactivation requirements for AI functionality LCM for beam management: Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. No agreements on performance monitoring accuracy.

We think the same conclusions can be reused for AI mobility except activation delay. For AI/ML functionality activation, before we reuse the conclusion, the scenario and procedure of activation need to be clarified. For performance monitoring, since the metrics can be different from beam management, we can postpone the discussion and wait for RAN2 progress.

Proposal 11: Do not define deactivation/switching/fallback delay requirements for AI functionality.

Proposal 12: For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework.

2.4 Simulation assumption

According to the discussions in study phase, simulation is needed to define requirements and the measurement error on AI model input should be taken into account. Referring to the simulation in beam management [4], we think the following procedures can be used for AI mobility simulation:

- Companies to generate ideal L3-RSRP dataset from the SLS assumption. - Use the LLS assumptions to generate L3-RSRP difference as the baseband errors. - Use the Simulation assumptions for RF error to generate RF errors. - Add the measurement error into the ideal SLS dataset in 1st step to derive the dataset with measurement errors. - Both baseband errors and RF errors are included. - Measurement error assumptions follow 2nd step and 3rd step. - Add L3 filtering to the dataset with added measurement error generated from the 4th step. - Use subset samples of the dataset generated from the 5th step for training models. - Filter UEs in the other subset samples of the dataset with added measurement error (non-overlapped samples with the subset dataset in 6th step) based on SINR value (e.g., SINR>= -6dB as the starting point) to derive the dataset for inference. - Use the dataset with measurement error for inference in 6th step to evaluate the performance metrics (predicted L3-RSRP) assuming that the ground truth is with the measurement error.

For the measurement error, the following assumptions are agreed as baseline:

- For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

- For RF error model, use a truncated Gaussian distribution.

  - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

Regarding RF error model, we can understand that the values for different frequency layers or different time may be different, but we think we do not need to model such details on the RF error. One single value is enough for Gaussian distribution and the legacy assumption on the RF margin (4dB for FR1 and 2.5dB for FR1) can be reused.

Proposal 13: Use the following assumption for RF error model in simulation:

- Truncated Gaussian distribution under [±4] dB RF error (μ=0,  σ2=[4]) for FR2

- Truncated Gaussian distribution under [±2.5] dB RF error (μ=0,  σ2=[2.5]) for FR1

The next issue is how to add the measurement error. For RF error, we can derive a set of values based on the above distribution and add one value to each sample of the dataset. For baseband error, according to the experience in legacy link level simulation, we think the simulated error is the measurement error after L1 filtering. On the other hand, the dataset derived based on SLS simulation includes the samples under different SINR, we need to discuss whether and how to add different errors based on SINR values on each sample. We think the following options can be considered:

- Option 1: Single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range.

For both option 1 and option 2, the following approaches can be considered to add the measurement error on each sample in each group:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

Proposal 14: Discuss whether to add different baseband errors on different group of datasets.

- Option 1: Add one set of measurement errors on single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range and add different sets of measurement error on each group.

Proposal 15: For each group of datasets, consider the following approaches to add the measurement error on each sample:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

For the definition of ground truth, there are two alternatives left based on the study item discussion, where alt1 is to use the expected measurement results calculated based on transmission power and channel modelling and alt2 is to use the reported measurement results under certain conditions.

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

For alt1, the advantage is that it is easily calculated and TE side can clearly know the expected results at each timing point, but the issue is that TE does not know when the measurement is performed or predicted. Therefore, TE doesn’t know which one to be compared which may request additional report from UE on the time stamp. The other issue is that the performance requirements may be used in model monitoring in LCM procedure, we understand it may cause problem since UE cannot acquire the expected measurement results in real LCM implementation.

For alt2, according to the existing measurement model, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. Based on the legacy procedure, when the SINR is high enough, the UE measurement results are approximate to the ideal values. The advantage is that this can accommodate different UE implementation e.g., L1 filtering, and can be aligned with model training and inference procedure. To define the prediction accuracy and testing, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions. The issue for alt2 is that UE may not perform actual measurement on certain occasions in some of the prediction cases, but we think this can be resolved by setting additional procedure in the testing. The other concern is that both ground truth and report are from UE which makes UE possible to cheat in testing. But we understand the actual measurement used as ground truth still needs to meet the legacy requirements, and cheating in the testing is not benefit for UE since it may lead bad performance in real practice. So we do not need to assume that UE will cheat in the testing.

From the analysis above, we prefer to use alt2 as baseline to define the ground truth.

Proposal 16: Use Alt2 (i.e., reported measurement value under certain conditions) to define the ground truth for prediction accuracy in FR1.

Based on the above discussion, the simulation assumption for measurement prediction is provided in [5] as starting point.

- Conclusions

In this contribution, we discussed the RAN4 requirements for RRM measurement prediction and got the following observations and proposals:

Proposal 1: RAN4 to define requirements for measurement prediction for scenarios 1~5.

| Scenario number | Evaluation scenario | Methodology |
| --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency temporal domain case A | intra-cell |
| 2 | FR1 to FR1 intra-frequency temporal domain case B | Intra-cell |
| 3 | FR1 to FR1 inter-frequency (frequency domain) | Inter-cell |
| 4 | FR2 to FR2 intra-frequency temporal domain case A | Intra-cell |
| 5 | FR2 to FR2 intra-frequency temporal domain case B | Intra-cell |

Proposal 2: RAN4 to define single set of requirements applied to all sub-use cases.

Proposal 3: RAN4 to define single set of requirements applied to single-cell-based and cluster-based approach for frequency domain prediction.

Proposal 4: RAN4 to define measurement period and prediction delay requirements for AI mobility with the following definition:

- Measurement period is the minimum observation period during which UE samples the reference signals to make 1 prediction report.

- Prediction delay is the time to get prediction report which includes measurement period and inference delay.

Proposal 5: RAN4 to define prediction reporting delay for event triggered reporting and the first report in event triggered periodic reporting.

Proposal 6: For the prediction reporting delay requirements to be defined, the time for the first available reporting resource and time for L3 filtering are not included.

Proposal 7: For details of inference delay, wait for the progress of beam management in Rel-19 NR_AIML_air.

Proposal 8: For temporal domain prediction case A, the legacy measurement period requirements can be reused.

Proposal 9: For temporal domain prediction case B, define the measurement period requirements considering the assumptions on MRRT, skipping patterns and measurement configurations.

Proposal 10: For frequency domain prediction, the legacy measurement requirements can be reused by clarifying in CSSF that the frequency layers configured for prediction are not counted.

Proposal 11: Do not define deactivation/switching/fallback delay requirements for AI functionality.

Proposal 12: For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework.

Proposal 13: Use the following assumption for RF error model in simulation:

- Truncated Gaussian distribution under [±4] dB RF error (μ=0,  σ2=[4]) for FR2

- Truncated Gaussian distribution under [±2.5] dB RF error (μ=0,  σ2=[2.5]) for FR1

Proposal 14: Discuss whether to add different baseband errors on different group of datasets.

- Option 1: Add one set of measurement errors on single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range and add different sets of measurement error on each group.

Proposal 15: For each group of datasets, consider the following approaches to add the measurement error on each sample:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

Proposal 16: Use Alt2 (i.e., reported measurement value under certain conditions) to define the ground truth for prediction accuracy in FR1.

- References

RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#108

RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#109

3GPP TR 38.744: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR”

R4-2511797 WF on updated simulation assumption for AI based beam management, RAN4#116

R4-2513096 Simulation assumption for measurement prediction, RAN4#116bis