# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513100

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.3.2

Source:	OPPO

Title:	Discussion on RRM performance requirements for R19 LB CA

Document for:	Approval

Introduction

Based on the timeline of R19 low band CA work item [1], the corresponding performance requirements need to be discussed from this meeting. In this contribution, we will provide our considerations on the test cases based on the requirements defined in core part [2][3].

Discussion

Based on the conclusions of core part discussion and the CR split, the following requirements are defined for low band CA via switching:

Agreement in AH: Table 1. Necessity of requirement on SDL CC Requirement; Whether or not RAN4 will have requirement; Note. interruption requirement for CC switching; No (can be revisited if not aligned with RAN1 further agreement.); Issue 1-2-1, Agreed on Tuesday online session. Activated SDL SCell L3 measurement; Yes; Issue 1-3-1, Agreed on Tuesday online session. Deactivated SDL SCell measurement [and corresponding interruption]; Yes; Issue 1-3-3, Agreed on Tuesday online session. SDL SCell activation/deactivation and corresponding interruption; Yes. [BFD/CBD]; Yes; Related with issue 1-3-10. TCI state switch delay; Yes; Related with issue 1-3-11. PL-RS switch delay; No; Related with issue 1-3-12. L1-RSRP/SINR; Yes; Related with issue 1-3-13.

Last meeting reached a big CR [3] to capture the related requirements. From the perspective of PCell and SDL SCell, we summarize the updated requirements below.

Observation 1: The following requirements are updated for R19 LB CA via switching:

- For FDD PCell:

  - Intra-frequency L3 measurement period (outside gap only)

  - RLM measurement period

  - BFD/CBD measurement period

  - L1-RSRP/SINR measurement period

  - TCI state switch delay

  - PL-RS switch delay

- For SDL SCell:

  - activated SDL SCell intra-frequency L3 measurement period (outside gap only)

  - deactivated SDL SCell measurement and corresponding interruption

  - SCell (de)activation delay and corresponding interruption

  - BFD/CBD measurement period

  - L1-RSRP/SINR measurement period

  - TCI state switch delay

To verify the above requirements, the corresponding test cases need to be defined. Additional UE processing time TLBCA is introduced for SDL SCell (de)activation delay and should be verified with high priority. Besides, for deactivated SDL SCell measurement, the legacy delay requirements are used but the interruption to PCell is new requirement and should be verified.

However, a common scaling factor method to extend measurement period are used for several requirements, such as RLM, BFD and CBD. We prefer to select one or a subset of them to define test case considering the test overhead, and give the following proposal.

Proposal 1: Define the following test cases for LB CA via switching:

- For FDD PCell:

  - Intra-frequency L3 measurement period (outside gap only)

  - Select one TC among RLM/BFD/CBD/L1-RSRP/SINR measurement

  - TCI state switch delay measurement period

  - PL-RS switch delay requirement

- For SDL SCell:

  - Activated SDL SCell intra-frequency L3 measurement period (outside gap only)

  - Interruption due to deactivated SDL SCell measurement

  - SDL SCell (de)activation delay and corresponding interruption

  - Select one TC among SCell BFD/CBD/L1-RSRP/SINR measurement

According to the requirements design, except interruption, the measurement requirements is based on the legacy requirements, and with the restriction that only the SMTC occasions overlapped with serving cell (PCell or SDL SCell) reception duration indicated by switching pattern are considered. So for each test case, the corresponding legacy test configurations in FR1 can be used as baseline and the switching pattern is configured.

To verify the different overlapping ratio between SMTC occasions and Cell reception duration, different switching patterns can be configured in different test cases.

Proposal 2: For each test case, the corresponding legacy test configurations can be used as baseline.

Proposal 3: Different switching patterns can be configured in different test cases to cover fully overlapping and partially overlapping cases between SMTC occasions and corresponding cell reception duration.

Conclusions

In this paper, we provided our views on the performance requirements for low band carrier aggregation via switching. And the following observation and proposals are provided.

Observation 1: The following requirements are updated for R19 LB CA via switching:

- For FDD PCell:

  - Intra-frequency L3 measurement period (outside gap only)

  - RLM measurement period

  - BFD/CBD measurement period

  - L1-RSRP/SINR measurement period

  - TCI state switch delay

  - PL-RS switch delay

- For SDL SCell:

  - activated SDL SCell intra-frequency L3 measurement period (outside gap only)

  - deactivated SDL SCell measurement and corresponding interruption

  - SCell (de)activation delay and corresponding interruption

  - BFD/CBD measurement period

  - L1-RSRP/SINR measurement period

  - TCI state switch delay

Proposal 1: Define the following test cases for LB CA via switching:

- For FDD PCell:

  - Intra-frequency L3 measurement period (outside gap only)

  - Select one TC among RLM/BFD/CBD/L1-RSRP/SINR measurement

  - TCI state switch delay measurement period

  - PL-RS switch delay requirement

- For SDL SCell:

  - Activated SDL SCell intra-frequency L3 measurement period (outside gap only)

  - Interruption due to deactivated SDL SCell measurement

  - SDL SCell (de)activation delay and corresponding interruption

  - Select one TC among SCell BFD/CBD/L1-RSRP/SINR measurement

Proposal 2: For each test case, the corresponding legacy test configurations can be used as baseline.

Proposal 3: Different switching patterns can be configured in different test cases to cover fully overlapping and partially overlapping cases between SMTC occasions and corresponding cell reception duration.

Reference

- RP-243317 New WID on low band carrier aggregation via switching

- R4-2512119 WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2509499 Big CR for R19 NR_LBCA_Sw RRM, Apple