3GPP TSG-RAN WG4 Meeting #116bis		 										R4-2513106

Prague, Czech Republic, October 13th – 17th, 2025

Source: 	OPPO

Title: 	On RRM performance requirements of SSB adaptation

Agenda Item:	6.16.2.2

Document for:	Discussion

Introduction

In this document, we provide our views on the RRM performance and test case requirements of SSB adaptation.

Discussion

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

For SSB adaptation, L1 and L3 impact are identified in core requirements. But compared to legacy UE behaviour, the main different parts due to SSB adaptation are about DCI triggering and new SMTC mapping to SSB periodicity.

Observation 1: For testing SSB adaptation, RAN4 to focus on the new aspects like DCI triggering and new SMTC mapping to SSB periodicity。

From procedure wise, we think the impact of SSB adaptation for either L1 or L3 measurements could be similar. To reduce the amount of test cases, we prefer to define one TC for L3 measurement.

Proposal 1: To reduce the amount of test cases,

- RAN4 to test SSB adaptation for either Scell activation or for L3 measurement, and

- not to define new test case for L1 measurement, and

- not to define TC for SSB adaptation for L1/L3 transition requirement.

For example, RAN4 tests SSB adaptation with normal Scell activation or for L3 measurement, assuming adapted SSB periodicity and updated SMTC periodicity applies after DCI 2_9 indication.

Proposal 2：For SSB adaptation for L3 measurement, RAN4 to define the test cases for SMTC and SSB periodicity adaptation based on DCI 2-9.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1: For testing SSB adaptation, RAN4 to focus on the new aspects like DCI triggering and new SMTC mapping to SSB periodicity。

Proposal 1: To reduce the amount of test cases,

- RAN4 to test SSB adaptation for either Scell activation or for L3 measurement, and

- not to define new test case for L1 measurement, and

- not to define TC for SSB adaptation for L1/L3 transition requirement.

Proposal 2：For SSB adaptation for L3 measurement, RAN4 to define the test cases for SMTC and SSB periodicity adaptation based on DCI 2-9.

- Reference

[1]. R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1.